3GPP TSG SA WG2 Meeting #147E	S2-2107230r03

# Elbonia, October 18 – 22, 2021	Revision of S2-21xxxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3030 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE location verification handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The description of how AMF can verify UE location for satellite access has been added in multiple procedures in 23.502. At the same time, a common description for how AMF can verify UE location for satellite access has subsequently been added in 23.501. To ensure alignment and avoid duplicated descriptions it is preferrable to refer to the common descrioption in 23.501. The description in 23.501 is slightly enhanced to provide additional clarity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance description in 23.501 to allow it to be used as common reference for UE location verification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Overlapping description and inconsistencies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First change ***************

5.4.11.4	Verification of UE location

In order to ensure that the regulatory requirements are met, the network may be configured to enforce that the selected PLMN is allowed to operate in the country of the UE location by verifying the UE location during Mobility Management and Session Management procedures. In this case, when the AMF receives a NGAP message containing User Location Information for a UE using NR satellite access, the AMF may decide to verify the UE location. If the AMF determines based on the Selected PLMN ID and ULI (including Cell ID) received from the gNB that it is not allowed to operate at the present UE location the AMF should reject any NAS request with a suitable Cause value and, if known in AMF, inform the UE of the country of the UE location. If the UE is already registered to the network and has to be rejected, the AMF may initiate deregistration of the UE. The AMF should not reject the request or deregister the UE unless it has sufficiently accurate UE location information to determine that the UE is located in a country in which the PLMN is not allowed to operate.

If the AMF, based on the ULI, is not able to determine the UE’s location with sufficient accuracy to make a final decision, the AMF proceeds with the Mobility Management or Session Management procedure and may initiate UE location procedure, as specified in clause 6.10.1 of TS 23.273 [87], to determine the UE location. The AMF shall be prepared to deregister the UE if the information received from LMFindicates that the UE is registered to a PLMN that is not allowed to operate in the country of the UE location. In case of a NAS procedure, the AMF should either reject any NAS request targeted towards a PLMN that is not allowed to operate in the country ofthe known UE location and indicate a suitable Cause value and, if known in AMF, the country of the UE location, or accept the NAS procedure and initiate deregistration procedure once the UE location is known.In the deregistration message to the UE, the AMF may include a suitable Cause value and,if known in AMF, inform the UE of the country of the UE location.

In the case of a handover procedure, if the (target) AMF determines that it is not allowed to operate at the current UE location, the AMF either rejects the handover, or accepts the handover and later deregisters the UE.

NOTE 2:	Some countries use multiple MCCs; and some MCCs, such as 901, can be allowed in multiple countries. Therefore, the UE can attempt to register in a PLMN with MCC different from the MCC provided by the AMF in the reject or deregistration message.

*************** End of changes ***************