3GPP TSG-SA WG2 Meeting #147E (e-meeting)	S2-2107252r05

# Elbonia, 18 – 22 October, 2021										   		 (revision of S2-xxxxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3264 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MINT Updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT |  |  |  |  |  | Date: |  |  | 2021-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - “List of PLMN(s) to be used in Disaster Condition” is defined in CT1 #131e meeting with priority order. Synchronising with latest agreed CT1 CRs and terminologies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Synchronising the list name ‘list of one or more PLMN(s) with Disaster Condition for which Disaster Roaming service is offered by the available PLMN’ with CT1 in 5.40.3. - Synchronizing the congenstion handling with CT1 CRs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In complete MINT functions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.40.2, 5.40.3, 5.40,4, 5.40.6 |  |  |  |  |  |  |  |  |
| s |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.40.2	UE configuration and provisioning for Disaster Roaming

A UE supporting Disaster Roaming is configured with the following information:

-	List of PLMN(s) to be used in Disaster Condition.

The 'list of PLMN(s) to be used in Disaster Condition' may be pre-configured in USIM or provided by HPLMN after a successful registration procedure over 3GPP access or non-3GPP access. The 'list of PLMN(s) to be used in Disaster Condition' may be configured over non-3GPP access before disaster condition has occurred.

While roaming (i.e. not in the country of the HPLMN), the Registered PLMN may provide the 'list of PLMN(s) to be used in Disaster Condition in the roamed to country' after a successful registration procedure to the UE. This list shall not alter any list provided by the HPLMN.

The details of the UE behaviour regarding the usage of this list are described in TS 23.122 [17] and TS 24.501 [47].

Editor's note:	The details of provisioning of the list (e.g. Registration Accept or UCU after the registration procedure is completed) are FFS to be determined by CT WG1.

* * * * Start of 2nd Change * * * *

5.40.3	Disaster Condition Notification and Determination

The NG-RAN in the PLMN that provides Disaster Roaming service, broadcasts an indication of accessibility for Disaster Roaming service, and optionally, a 'list of one or more PLMN(s) with Disaster Condition for which Disaster Roaming service is offered by the available PLMN' in the impacted area.

Editor's note:	The details of broadcasting information of PLMN(s) with Disaster Condition by NG-RAN in a PLMN providing Disaster Roaming service is for further discussion in RAN groups.

A UE determines the Disaster Condition based on the information broadcasted from the NG-RAN providing Disaster Roaming service, and performs the network selection and the access control for the Disaster Roaming as described in TS 23.122 [17] and TS 24.501 [47].

NOTE 1:	How a PLMN is notified that another PLMN is a PLMN with Disaster Condition and how a PLMN is notified of the area where the associated Disaster Condition applies is managed by the government agencies or the authorities, and is out of scope of 3GPP.

NOTE 2:	The broadcast for Disaster Roaming service from the NG-RAN occurs only during the Disaster Condition.

* * * * Start of 3rd Change * * * *

5.40.4	Registration for Disaster Roaming service

For a UE to receive Disaster Roaming service from a PLMN providing Disaster Roaming service, the UE sends a NAS Registration Request message with Registration Type value "Disaster Roaming":

-	When the AMF in the PLMN providing Disaster Roaming service receives a NAS Registration Request with Registration Type set to "Disaster Roaming", the AMF controls if the UE is allowed to access Disaster Roaming service in the area with Disaster Condition as specified in TS 23.502 [3] clause 4.2.2.2.2.

To support the Disaster Roaming service, the PLMN providing Disaster Roaming service is configured to support communication with the network entities in the HPLMN of the UE, i.e. configurations related to roaming interfaces for communication between serving PLMN and HPLMN shall be deployed in the affected entities. This communication between the PLMNs need only be enabled during the Disaster Condition.

Editor's note:	It is FFS if Disaster Roaming indication needs to be provided from AMF to SMF, and further in charging related information. This will be evaluated based on the feedback from SA3 and SA5.

The Disaster Roaming service is limited to the impacted geographic area with Disaster Condition. The NG-RAN nodes and AMF in the PLMN providing Disaster Roaming service are configured with the area information, i.e. a list of TAIs which can be formulated by the PLMN providing the Disaster Roaming service based on the geographic area with Disaster Condition in the other PLMN(s).

The AMF in the PLMN providing Disaster Roaming service provides the mobility restriction list to the NG-RAN as specified in clause 5.3.4.1.1 considering the area with Disaster Condition, and also indicating that EPC is not an allowed core network.

* * * * Start of 4th Change * * * *

5.40.6	Prevention of signalling overload related to Disaster Condition and Disaster Roaming service

The load control, congestion and overload control mechanism specified in clause 5.19 can be used to mitigate the load caused by UE requesting the Disaster Roaming service in the PLMN providing Disaster Roaming service and returning of UE to allowable PLMN when Disaster Condition is no longer applicable.

To prevent signalling overload in PLMN providing Disaster Roaming, the HPLMN or registered PLMN:

-	may provide the UE in a prioritized manner with the list of PLMNs described in clause 5.40.2 for Disaster Roaming;

-	may provide disaster roaming wait range information to control  when the UE can initiate the registration for Disaster Roaming service upon arriving in the PLMN providing Disaster Roaming service as specified in TS 23.122  [17] and TS 24.501 [47]; and

-	applies Access Identity 3 for Disaster Roaming service request as specified in TS 24.501 [47].

Editor's note:	Whether these restrictions are signalled, pre-configured, or computed at the UE (possibly based on signalled or pre-configured parameters) is FFS to be determined by CT WG1.

NOTE:	The mechanisms available at the AMF and the SMF for mitigation of overload and congestion are used for 5GSM congestion mitigation during the Disaster Roaming.

To prevent signalling overload by returning UEs in PLMN previously with Disaster Condition which is no long applicable, the HPLMN or registered PLMN:

-	may provide disaster roaming wait range information to control  when the UE can initiate the registration upon returning to the PLMN previously with Disaster Condition.

* * * * End of Changes * * * *