3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107266r01

# Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0443 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the OSE analytics per RAT type and/or per Frequency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Correction. In clause 6.2.1 and 6.2.2.1, the reference about subscribing to AMF or SMF via UDM to clause 4.15.4.3 of TS23.502 is incorrect. It should be clause 4.15.4.4 of TS23.502. - Clarification on OSE analytics. As indicated in clause 6.4.3, the OSE analytics could be provided per RAT Type and/or Frequency. However, the corresponding description similar to Service Experience for a Network Slice or an Application or an Edge Application over a UP path is not provided in clause 6.4.1. For example, the “Service Experience for a RAT Type or Frequency” is missing in the following options: The Observed Service Experience analytics may provide one or both of the following: - Service Experience for a Network Slice: Service Experience for a UE or a group of UEs or any UE in a Network Slice; - Service Experience for an Application: Service Experience for a UE or a group of UEs or any UE in an Application or a set of Applications. - Service Experience for an Edge Application over a UP path: Service experience for a UE or a group UEs or any UE in an Application or a set of Applications over a specific UP path (UPF, DNAI and EC server). It is reasonable that, a consumer, e.g., a PCF may request the service experience for more than one RAT type and/or one frequency for specific application(s) in order to find out what is the best RAT type and or frequency for best experience. So in the filters/outputs tables where mention “RAT type” / “frequency” should use “RAT Types” / “Frequency(ies)” Wha’s more, the Reporting Thresholds parameter is missing when a consumer NF subscribes the OSE analytics. This parameter could reduce the analytics notifications when the Reporting Threshold is not met. For example, the PCF may expect to receive the OSE analytics when the service experience for a RAT Type is less than the Reporting Threshold then it determines to update the RFSP index. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Correct reference 4.15.4.3 to 4.15.4.4 in clause 6.2.1 and 6.2.2.1; 2. Add the general discriptions for OSE analytics per RAT Type and/or per Frequency into the clause 6.4.1; 3. Modify RAT Type and Frequency to RAT Type(s) and Frequency(ies) in Table 6.4.1-1, Table 6.4.3-1, Table 6.4.3-2; 4. Provide the optional Reporting Thresholds parameter when a consumer NF subscribes the OSE analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Incorrect reference. 2. There is no clue that the NWDAF could provide OSE analytics per RAT Type and/or per Frequency in the clause 6.4.1; 3. Cause confusion on consumer indicating filter information; 4. The number of OSE analytics notofications will be very large when the Reporting Thresholds are not provided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2.1, 6.4.1, 6.4.3, 6.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2.1	General

The Data Collection feature permits NWDAF to retrieve data from various sources (e.g. NF such as AMF, SMF, PCF, and AF; OAM), as a basis of the computation of network analytics.

All available data encompass:

-	OAM global NF data,

-	Data available in NFs, e.g. behaviour data related to individual UEs or UE groups (e.g. UE reachability), and pre-computed metrics covering UE populations (e.g. number of UEs present in a geographical area), per spatial and temporal dimensions (e.g. per region for a period of time),

-	NF data available in the 5GC (e.g. NRF),

-	Data available in AF.

When DCCF, ADRF, MFAF or NWDAF hosting DCCF or ADRF are present in the network, the data collection also follows the principles described in clause 6.2.6.

The NWDAF shall use at least one of the following services:

-	the Generic management services as defined in TS 28.532 [6], the Performance Management services as defined in TS 28.550 [7] or the Fault Supervision services as defined in TS 28.545 [9], offered by OAM in order to collect OAM global NF data.

-	the Exposure services offered by NFs in order to retrieve data and other non-OAM pre-computed metrics available in the NFs.

-	Other NF services in order to collect NF data (e.g. NRF)

-	DCCF data management service to retrieve data using DCCF.

The NWDAF shall obtain the proper information to perform data collection for a UE, a group of UEs or any UE:

-	For an Analytics ID, NWDAF is configured with the corresponding NF Type(s) and/or event ID(s) and/or OAM measurement types.

-	NWDAF shall determine which NF instance(s) of the relevant NF type(s) are serving the UE, the group of UEs or any UE, taking into account the S-NSSAI(s) and area of interest as defined in clause 7.1.3 of TS 23.501 [2].

-	NWDAF invokes Nnf_EventExposure_Subscribe services to collect data from the determined NF instance(s), and/or triggers the procedure in clause 6.2.3.2 to subscribe to OAM services to collect the OAM measurement.

The NWDAF performs data collection from an AF directly as defined in clause 6.2.2.2 or via NEF as defined in clause 6.2.2.3. According to the data collection request, the AF may further perform data collection from UE (see clause 6.4.2 and clauses 6.5.2-6.5.4) as defined in clause 6.2.8.

The NWDAF shall be able to discover the events supported by a NF.

Data collection procedures enables the NWDAF to efficiently obtain the appropriate data with the appropriate granularity.

When a request or subscription for statistics or predictions is received, the NWDAF may not possess the necessary data to perform the service, including:

-	Data on the monitoring period in the past, which is necessary for the provision of statistics and predictions matching the Analytics target period.

-	Data on longer monitoring periods in the past, which is necessary for model training.

Therefore, in order to optimize the service quality, the NWDAF may undertake the following actions:

-	The NWDAF may return a probability assertion as stated in clause 6.1.3 expressing the confidence in the prediction produced. Prediction may be returned with zero confidence as described below. This confidence is likely to grow in the case of subscriptions.

-	The value of the confidence depends on the level or urgency expressed by the parameter "preferred level of accuracy of the analytics" as listed in clause 6.1.3, the parameter "time when analytics information is needed" as listed in clause 6.1.3, and the availability of data. If no sufficient data is collected to provide an estimation for the requested level of accuracy before the time deadline, the service shall return a zero confidence. Otherwise, the NWDAF may wait until enough data is collected before providing a response or a first notification.

-	In order to be prepared for future requests on analytics from NFs/OAM, the NWDAF, upon operator configuration, may collect data on its own initiative, e.g. on samples of UEs, and retain the data collected in the data storage.

NOTE 1:	The NWDAF can send an error response to the analytics consumer to indicate that statistics are unavailable if the NWDAF was not prepared for future requests and did not collect data on its own initiative.

The volume and maximum duration of data storage is also subject to operator configuration.

The NWDAF may decide to reduce the amount of data collected to reduce signalling load, by either prioritizing requests received from analytics consumers, or reducing the extent (e.g. duration, scope) of data collection, or modifying the sampling ratios. When using sampling ratio, the NWDAF may, depending on the analytics required and based on local configuration, provide additional partitioning criteria to the NFs to allow for a better UEs representation and to request that the NFs first partition the UEs before applying sampling ratio (see Event Reporting Information as specified in TS 23.502 [3]). The NWDAF may provide one or multiple partitioning criteria in its request for data collection from NFs.

The NWDAF may skip data collection phase when the NWDAF already has enough information to provide requested analytics.

The data which NWDAF may collect is listed for each analytics in input data clause and is decided by the NWDAF.

NOTE 2:	NWDAF can skip data collection phase for some specific input data per the requested analytics e.g. when some of the data is already available at NWDAF for the requested analytics, or when NWDAF considers that some of the data is not needed at all to provide the requested analytics as per the analytics consumer request (e.g. based on preferred level of accuracy or based on the time when analytics are needed).

Event exposure subscriptions for data collection from the AMF and the SMF may need to survive after the removal of UE context in the AMF including event exposure subscriptions, or upon the creation of new UE context in AMF or SMF. In order for event exposure subscriptions in AMF and SMF to be (re)created in these cases, the NWDAF may subscribe to the events in AMF and/or SMF via UDM for a UE or group of UEs, as specified in TS 23.502 [3], clause 4.15.4.4.

In hierarchical interactions among NWDAFs, without standalone DCCF, or co-located DCCF, the efficiency of data collection can be achieved by inter-NWDAF instance cooperation among NWDAF instances on different levels of the hierarchy. An efficient data collection means that the same data required for the same Analytics ID or different Analytics IDs should not be collected multiple times by the different NWDAFs of the hierarchy.

# * * * * Next change * * * *

6.2.2.1	General

The Data Collection from NFs is used by NWDAF to subscribe/unsubscribe at any 5GC NF to be notified for data on a set of events.

The Data Collection from NFs is based on the services of AMF, SMF, UDM, PCF, NRF and AF (possibly via NEF):

-	Event Exposure Service offered by each NF as defined in TS 23.502 [3] clause 4.15 and clause 5.2.

-	other NF services (e.g. Nnrf_NFDiscovery and Nnrf_NFManagement in NRF as defined in TS 23.502 [3] clause 4.17)

This data collection service is used directly in order to retrieve behaviour data for individual UEs or groups of UEs (e.g. UE reachability), and also to retrieve global UE information (e.g. Number of UEs present in a geographical area).

Table 6.2.2.1-1: NF Services consumed by NWDAF for data collection

| Service producer | Service | Reference in TS 23.502 [3] |
| --- | --- | --- |
| AMF | Namf_EventExposure (NOTE 3) | 5.2.2.3 5.2.3.5 |
| SMF | Nsmf_EventExposure (NOTE 3) | 5.2.8.3 5.2.3.5 |
| PCF | Npcf_EventExposure (for a group of UEs or any UE) Npcf_PolicyAuthorization_Subscribe (for a specific UE) | 5.2.5.7 |
| UDM | Nudm_EventExposure | 5.2.3.5 |
| NEF | Nnef_EventExposure | 5.2.6.2 |
| AF | Naf_EventExposure | 5.2.19.2 |
| NRF | Nnrf_NFDiscovery | 5.2.7.3 |
|  | Nnrf_NFManagement | 5.2.7.2 |

NOTE 1:	The present document specifies that NWDAF can collect some UPF input data for deriving analytics, but how NWDAF collects these UPF input data is not defined in this Release of the specification.

NOTE 2:	There is no data collected from the PCF by the NWDAF defined in this Release of the specification.

NOTE 3:	The Nudm_EventExposure can be used when NWDAF uses the procedures specified in TS 23.502 [3] clause 4.15.4.4 to subscribe to AMF or SMF via UDM.

To retrieve data related to a specific UE, there are two cases:

-	If no Area of interest is indicated by the consumer, the NWDAF shall first determine which NF instances are serving this UE as stated in table 6.2.2.1-2 unless the NWDAF has already obtained this information due to recent operations related to this UE.

-	If an Area of interest is indicated, the NWDAF can:

-	First determine the AMF serving the UE and subscribe UE location from the AMF. Once the UE is in or moves into the Area of interest, the NWDAF determines which NF instances are serving this UE as stated in table 6.2.2.1-2 unless the NWDAF has already obtained this information due to recent operations related to this UE; or

-	Determine the NF instances of a given type of network function serving the Area of interest by querying NRF unless the NWDAF has already obtained this information due to recent operations related to this UE.

Table 6.2.2.1-2: NF Services consumed by NWDAF to determine which NF instances are serving a UE

| Type of NF instance (serving the UE) to determine | NF to be contacted by NWDAF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- | --- |
| UDM | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| AMF | UDM | Nudm_UECM | 5.2.3.2 |
| SMF | UDM | Nudm_UECM | 5.2.3.2 |
| BSF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| PCF | BSF | Nbsf_Management | 5.2.13.2 |
| NEF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| NWDAF | NRF UDM | Nnrf_NFDiscovery Nudm_UECM | 5.2.7.3 5.2.3.2 |

The UDM instance should be determined using NRF as described in clause 4.17.4 of TS 23.502 [3] and factors to determine as described in clause 6.3.8 of TS 23.501 [2].

The AMF, SMF instances should be determined using a request to UDM providing the SUPI. To determine the SMF serving a PDU session, the NWDAF should in addition provide the DNN and S-NSSAI of this PDU Session; otherwise the NWDAF will obtain a list of possibly multiple SMFs (e.g. one per PDU session).

The BSF instance should be discovered using NRF thanks to optional request parameters (e.g. DNN list, IP domain list, IPv4 address range, IPv6 prefix range) as stated in clause 4.17.4 of TS 23.502 [3] or based on local configuration at the NWDAF.

The PCF instance serving UE PDU Session(s) should be determined using a request to BSF with the allocated UE address, DNN and S-NSSAI.

When NWDAF receives a request addressed to an Internal Group ID from a consumer, NWDAF may need to initiate data collection from several 5GC NFs, such as AMF, SMF, UDM, PCF, AF (e.g. via NEF), etc. If an Area of interest is indicated by the consumer, NWDAF may first discover the instances of the required 5GC NFs deployed in the network, e.g. by querying NRF, otherwise:

-	For discovering the UDM, NWDAF can query the NRF with the Internal Group ID as the target of the query.

-	For discovering AMF, SMF, PCF, NEF, and AF, NWDAF may need to discover all instances in the network by using the Nnrf_NFDiscovery service.

NOTE 4:	It is assumed that all members of an Internal Group ID belong to the same UDM Group ID. NWDAF can select a UDM instance supporting the UDM Group ID of the Internal Group ID.

Then, if data needs to be collected from AMF, SMF, UDM, and PCF, NWDAF may initiate the data collection with the Internal Group ID as the target, e.g. subscribing to the event exposure in all the instances of a given type of network function. This subscription to all the instances of required source of event exposure handles, e.g. mobility of UEs across AMFs, or initiation of new PDU sessions with different allocated SMFs.

For collecting data from AMF and SMF, NWDAF may collect the data directly from AMF and/or SMF, or indirectly via UDM, according to TS 23.502 [3] clause 4.15.4.4. The indirect method may be required if the event exposure subscription from NWDAF, for a UE or group of UEs, needs to survive the removal of UE context in the AMF including event exposure subscriptions, or upon the creation of new UE context in AMF or SMF serving the UE or group of UEs. In this case the UDM is responsible for (re)creating event exposure subscriptions in AMF and SMF, as specified in TS 23.502 [3], clause 4.15.4.4.

The NWDAF determines to collect data from a trusted AF supporting specific Event ID(s) and serving specific application(s) based on internal configuration.

The NEF instance that is serving a specific network slices and/or applications of a UE should be determined using NRF using optional request parameters as defined in clause 6.3.14 of TS 23.501 [2]

If NWDAF needs to collect data from an AF deployed outside the operator's domain, the NWDAF shall contact NEF with a SUPI or Internal Group ID as the target of the data collection. NEF is responsible for translation of SUPI to GPSI, or internal to external group identifiers, by querying UDM, prior to contacting the AF.

NOTE 5:	It is assumed that an AF is provisioned with the list of UE IDs (GPSIs or SUPIs) belonging to an External or Internal Group ID.

To retrieve required data for any UE, the NWDAF may subscribe to events from the AMF and/or SMF instances it has determined, setting the target of event reporting to "any UE" and the event filter(s) according to the Analytics Filter Information. Alternatively, if the required data is communication related and for any UE within an Area of interest, the NWDAF can obtain from the AMF instances it has determined a list of UEs located within the Area of Interest. Based on the obtained UE list, for each UE in the list, the NWDAF retrieves the SMF serving the UE and the NWDAF subscribes to data from the relevant SMF per each specific UE. The indirect event exposure subscription to AMF or SMF via UDM is not available for "any UE" or "any UE within an Area of interest". If the required data is collected from UE via AF as described in clause 6.2.8 and the target of analytics reporting received from consumer is "any UE", the NWDAF may either set the target of event reporting to "any UE" in the data collection request to the AF, or may determine a list of UEs from AMF and/or SMF based on the analytics filter information and send the data collection request to the AF for the determined list of UEs.

NOTE 6:	If NWDAF requires collecting data from either AMF or SMF for "any UE" or "any UE within an Area of Interest", NWDAF can use the direct Event Exposure subscription to AMF or SMF, since subscriptions to "any UE" or "any UE within an Area of Interest" are persistent by nature in AMF or SMF, due to not being linked to a UE context.

To retrieve data related to "any UE" based on analytics filter information, the NWDAF shall first determine which NF instances are matching the analytics filter information (see clause 6.7.5.1) as stated in table 6.2.2.1-3 unless the NWDAF has already obtained this information due to recent operations related to this analytics filter information.

Table 6.2.2.1-3: NF Services consumed by NWDAF to determine which NF instances are matching analytics filters

| Type of NF instance (matching analytics filters) to determine | NF to be contacted by NWDAF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- | --- |
| AMF, SMF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |

To retrieve data related to analytics IDs for "any UE" with analytics filter information defining an area of interest in terms of TA or Cells and/or with specific S-NSSAIs, NWDAF requires the network slice association information to properly determine the AMFs to collect data from as well as the proper queries to OAM for data collection.

NOTE 7:	Examples of analytics ID requiring NWDAF to use network slice association information for data retrieval are: network performance clause 6.6.1; user data congestion clause 6.8.1; QoS Sustainability clause 6.9; Dispersion Analytics clause 6.10.1; observed service experience for a Network Slice clause 6.4.1; and slice load analytics clause 6.3.

The network slice association information comprises the TAs associated with each AMF, and for each TAI its associated access type, cells and list of supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF). Additionally, the mapping of cells per TAI and supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF) for TAI for each AMF can change and NWDAF shall obtain this accurate information in order to properly retrieve data for analytics generation.

In order to derive the network slice association information, NWDAF may be configured with the mapping of cells per TAI, and the S-NSSAIs per TAI. NWDAF may subscribe to the "S-NSSAIs per TAI mapping" event exposed by AMF. The NWDAF may use the configured information (when the analytics subscription or request is at cell granularity) and the area of interest in the analytics subscription or request to retrieve from AMF the list of supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF) per TAI, and access types per TAI for each AMF in the required area of interest. NWDAF consumes the "S-NSSAIs per TAI mapping" event exposed by AMF using, as target of event reporting, the list of TAIs based on the area of interest received in the analytics filter information or identified by the mapping of the Cells per TAI matching to the Cell granularity included in the analytics filter information. The AMF "S-NSSAIs per TAI mapping" event output contains, for each of the TAIs requested by NWDAF, its associated access type and the list of supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF).

To retrieve data from SMFs for analytics IDs subscription or requests for "any UE" including analytics filter information with specific Applications, DNNs, DNAIs and area of interest per TA granularity, NWDAF shall first discover the SMF serving the area of interest via NRF.

NOTE 8:	Examples of analytics ID requiring NWDAF to collect data related to PDU sessions associated with an AoI with TA granularity are: network performance clause 6.6.1; user data congestion clause 6.8.1, QoS Sustainability clause 6.9.

NWDAF may directly consume events from the discovered SMF using the event target set to "any PDU session", and event filters with the same parameters of the analytics filter information, i.e. list of Application IDs, and/or DNNs, and/or DNAI, and the area of interest related to the requested analytics ID.

1.	When SMF supports the exchange of UE Location parameter when SMF interacts with AMF via Nsmf_PDUSession_Create/Update/CreateSMContext/UpdateSMContext due to session establishment, modification, or release, service request, or handover procedures (as defined in the TS 23.502 [3] clause 5.2.8.2), SMF can directly map the PDU sessions to an AoI with TA granularity.

If there are any changes in PDU sessions in the area of interest, for the Application ID, and/or DNN, and/or DNAI subscribed by NWDAF, SMF notifies the detected changes to NWDAF via Nsmf_EventExposure_Notify service operation, enabling NWDAF to keep an updated map of SMF and PDU sessions associated with the analytics filter information in an area of interest.

2.	When SMF does not support the exchange of UE Location parameter when SMF interacts with AMF but supports the mapping of PDU sessions per TA (as defined in TS 23.501 [2] clause 5.6.11), SMF may subscribe to UE mobility event notifications of AMF as described in clause 5.3.4.4 of TS 23.501 [2] using event ID "UE moving in or out of Area of Interest" and Event Filters as described in Table 5.2.2.3.1-1 from TS 23.502 [3] to retrieve the list of SUPIs (and GPSIs if available) in the area of interest. Based on the retrieved list of SUPIs in the area of interest, SMF identifies the PDU sessions in the area of interest.

3.	When SMF does not support the exchange of UE Location parameter when SMF interacts with SMF nor supports the mapping of PDU sessions per TA (as defined in TS 23.501 [2] clause 5.6.11), SMF rejects the request from NWDAF. Upon the reject, NWDAF identifies the need to create the mapping of PDU sessions per TA. NWDAF subscribes to UE mobility event notifications of AMF as described in clause 5.3.4.4 of TS 23.501 [2] using event ID "UE moving in or out of Area of Interest" and Event Filters as described in Table 5.2.2.3.1-1 from TS 23.502 [3] to retrieve the list of SUPIs (and GPSIs if available) in the area of interest. Based on the retrieved list of SUPIs in the area of interest, NWDAF subscribes to the SMFs serving the UEs in the area of interest and derives the mapping of PDU sessions per TA.

An NWDAF may require to discover and select other NWDAFs for UE related analytics. In this case, the NWDAF may discover from UDM if an NWDAF is already collecting data related to the UE, as specified in clauses 5.2 and 6.1C.

# * * * * Next change * * * *

6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average of observed Service MoS and/or variance of observed Service MoS indicating service MOS distribution for services such as audio-visual streaming as well as services that are not audio-visual streaming such as V2X and Web Browsing services) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or  more of the following:

-	Service Experience for a Network Slice: Service Experience for a UE or a group of UEs or any UE in a Network Slice;

-	Service Experience for an Application: Service Experience for a UE or a group of UEs or any UE in an Application or a set of Applications;

-	Service Experience for an Edge Application over a UP path: Service experience for a UE or a group UEs or any UE in an Application or a set of Applications over a specific UP path (UPF, DNAI and EC server);

-	Service Experience for a RAT Type or Frequency: Service experience for a UE or a group UEs in an Application or a set of Applications over a specific RAT Type and/or Frequency.

Therefore, Observed Service experience may be provided individually per UE or group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

The service consumer may be an NF (e.g. PCF, NSSF, AMF), AF, or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1;

-	optionally, maximum number of objects and maximum number of SUPIs;

-	optionally, preferred level of accuracy of the analytics;

-	optionally, accuracy level per analytics subset (Network Slice instance service experience, Application service Experience);

-	optionally, preferred order of results for the list of Application Service Experiences and/or Slice instance service experiences: "ascending" or "descending"; and

-	optionally, preferred granularity of location information: TA level or cell level.

Table 6.4.1-1: Analytics Filter Information related to the observed service experience

| Information | Description | Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Application | Network Slice | Edge Applications over a UP path | RAT Type | Frequency |
| Application ID (0...max) | The identification of the application(s) for which the analytics information is subscribed or requested. | Y | N | Y | Y | Y |
| S-NSSAI | When requesting Service Experience for a Network Slice: identifies the Network Slice for which analytics information is subscribed or requested. When requesting Service Experience for an Application: identifies the S-NSSAI used to access the application together with the DNN listed below. | N | Y | N | N | N |
| NSI ID(s) | Identifies the Network Slice instance(s) for which analytics information is subscribed or requested. | N | N | N | N | N |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] for which the analytics information is subscribed or requested. | N | N | N | N | N |
| DNN | When requesting Service Experience for an Application, this is the DNN to access the application. | N | N | N | N | N |
| DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. | N | N | Y | N | N |
| RAT Type(s) (NOTE X) | Identifies the RAT type(s). | N | N | N | Y | N |
| Frequency(ies) (NOTE X) | Identifies the frequency(ies). | N | N | N | N | Y |
| Application Server Address(es) (NOTE 1) | List of IP address(s)/FQDN(s) of the Application Server(s) that a UE, group of UEs, or 'any UE' has a communication session for which Service Experience Analytic information is requested. | N | N | Y | N | N |
| UPF anchor ID (NOTE 1) | Identifies the UPF where a UE has an associated PDU session | N | N | N | N | N |
| NOTE 1: These parameters may be provided when a consumer requires analytics for an edge application over a UP path. NOTE X: These parameters may be provided when a consumer requires analytics for a UE or a group UEs in an Application or a set of Applications over specific RAT Type(s) and/or Frequency(ies). |  |  |  |  |  |  |

NOTE:	A service consumer may use the Area of Interest in order to reduce the amount of signalling that the analytics subscription or request generates.

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address;

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for the Service Experience (see clause 6.4.3) in order to be notified by the NWDAF.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provide statistics and predictions on the observed service experience to a consumer NF or to OAM.

Based on the Analytics Filter information in Table 6.4.1-1 and the Target of Analytics reporting provided by the service consumer in the analytics subscription or request, NWDAF determines whether service experience analytics should be delivered for:

i)	Application(s);

ii)	Network Slice;

iii)	both Application(s) and Network Slice;

iv)	Edge Applications over a UP path;

v) RAT Type(s) and/or Frequency(ies).

If NWDAF is unable to differentiate based on the analytics subscription or request, it provides service experience analytics for both Application(s) and Network Slice.

If service experience for both Application(s) and Network Slice is desired but the Target of Analytics reporting or Analytics Filter information values (e.g. Area of Interest) need to be different, separate subscriptions/requests may be provided by the service consumer.

# * * * * Next change * * * *

6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) (NOTE 1) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI | Identifies the Network Slice |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for which the slice instance service experience applies. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| Application service experiences (0..max) (NOTE 1) | List of observed service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered (see NOTE 4). |
| > UPF Info | Indicating UPF serving the UE. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) with the same application service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > RAT Type(s) | Indicating the RAT type(s) for which the application service experience analytics applies. |
| > Frequency(ies) | Indicating the carrier frequency(ies) of UE's serving cell(s) where the application service experience analytics applies. |
| NOTE 1: Whether analytics subset is included or not is determined by the analytics filter information. |  |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| Slice instance service experiences (0..max) (NOTE 1) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI | Identifies the Network Slice |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for which the slice instance service experience applies. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| > Probability assertion | Confidence of this prediction. |
| Application service experiences (0..max) (NOTE 1) | List of predicted service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > UE location | Indicating the UE location information (e.g. TAI list, gNB ID, etc) when the UE service is delivered (see NOTE 4). |
| > UPF Info | Indicating UPF serving the UE. |
| > DNAI | Indicating which DNAI the UE service uses/camps on. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) with the same application service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > Probability assertion | Confidence of this prediction. |
| > RAT Type(s) | Indicating the RAT type(s) for which the application service experience analytics applies. |
| > Frequency(ies) | Indicating the carrier frequency(ies) of UE's serving cell(s) where the application service experience analytics applies. |
| NOTE 1: Whether analytics subset is included or not is determined by the analytics filter information. |  |

NOTE 1:	The NSI ID is an optional parameter. If not provided the Slice instance service experience indicates the service experience for the S-NSSAI.

NOTE 2:	The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and are assigned with the same value(s) in the service experience information for the slice instance which the application belongs to. Otherwise, the SUPI list and Ratio are mandatory to be provided for an application service experience.

NOTE 3:	If the consumer NF is an AF, the item "DNN" and "UPF info" shall not be included, and the NEF is responsible for translation of SUPI to GPSI, internal group identifiers to external ones, and UE location to geographical area, by querying UDM, prior to contacting the AF.

NOTE 4:	When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information.

NOTE 5:	The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1.

The number of Service Experiences and SUPIs are limited respectively by the maximum number of objects and the Maximum number of SUPIs provided as part of Analytics Reporting Information by the NWDAF Service Consumer.

# * * * * Next change * * * *

### 6.4.6	Procedures to request Service Experience for a  RAT Type and/or frequency

Figure 6.4.6-1: Procedure for NWDAF providing Service Experience for a  RAT Type and/or Frequency

This procedure is similar to the procedure in clause 6.4.4, with the following differences. The consumer needs to request the Analytics ID "Service Experience" for a UE or a group of UEs. The consumer includes both the Application ID for which their Service Experience is requested and indicates that the Target of Analytics Reporting is a UE or a group of UEs. The  Analytic  Filter can be set according to clause 6.4.1.

The Observed Service Experience per RAT Type and/or per frequency for a UE when consumed by PCF could be used as described in the TS 23.503 [4], clause 6.1.1.3.

# * * * * End of changes * * * *