SA WG2 Meeting #S2-147E	S2-2107297

18 - 22 October, 2021, Electronic meeting	(revision of S2-2106798)

# Source:	Nokia, Nokia Shanghai Bell, Verizon, AT&T, Sennheiser, Huawei,  HiSilicon, Matrixx, ZTE, China Unicom, Vivo, NTT Docomo,  ETRI, Xiaomi, Orange, China Mobile, KDDI, Tencent, T-Mobile USA, Interdigital, Samsung, CATT, Deutsche Telekom, Ericsson, Denso, BMWi, SK Telecom, Oracle, CBN, Broadcom, LG Electronics, NICT

# Title:	New SID on Study on 5G Timing Resiliency and TSC enhancements

# Document for:	Approval

# Agenda Item:	9.1.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on 5G Timing Resiliency and TSC&URLLC enhancements

Acronym: FS_5GTTUe

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | x | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 910039 | 5G Timing Resiliency System | Stage 1 work item |
|  | (Stage 1 work item for the above study to be added) |  |

# 3	Justification

Following are the justifications for the study objectives:

1) SA1 is also specifying requirements for 5G System to remain resilient if there is GNSS failure and for 5G System to act as a backup and offer wireless and indoor-capable time synchronization service for other applications (e.g. financial, power grid systems).

2) Generic TSC and exposure enhancements to 5GS for IP and ETH applications are needed for the following reasons:

- Current Exposure framework enables AF to request QoS parameters, provide traffic characteristics but not redundancy aspect which is important for some IP and ETH applications

3) Redundancy is most efficient when applied end to end.

4) It may be possible to apply TSN networks in the transport network. In such deployments, it may be beneficial to be able to interwork so that the transport network can also provide delay guarantees.

It has been raised whether RAN feedback on when the burst should be sent may be helpful. The performance benefit of such proposals can be studied and the gain can be compared with the complexity of such solutions.

# 4	Objective

Following are the objectives for this study:

WT#1. Support for 5G Timing Resiliency requirements defined by SA1.

WT#1.1. Study how to report time source status (failure condition or deviation from requested parameter such as accuracy) divergence from UTC and time sync degradation) to UEs and 3rd party applications (AFs):

- Study how RAN and 5GC learn about network time synchronization deviations from requested service parameters to be able to inform UEs and AFs.

- Study if additional information needs to be provided to UEs and AFs to inform about time synchronization degradation.

WT#1.2. Study how to enable AFs to request time synchronization service along with requested service parameters in a specific coverage area and how to enforce the requested service parameters.

WT#1.3. Study how to control 5G timing service based on subscription

NOTE: 	Configuration and operation of the synchronization network and mitigation actions when time source fails or degrades is assumed to be outside the scope of 3GPP, since there are protocols, techniques and practices already specified by other standardization bodies.

WT2. Study if any exposure enhancements to 5GS are needed for AF to request redundancy within the 3GPP network:

WT2.1 Ability for AF to request a certain reliability criteria and influence PER, use of redundancy mechanisms within the 3GPP network, such as PDCP duplication or redundant transmission on N3/N9 (i.e. in addition to QoS) for a given application. Identify if any gaps exist in the 3GPP mechanisms which need to be addressed to address AF request.

Identify if any gaps exist in the 3GPP mechanisms which need to be addressed.

WT4. Support for low latency and low jitter.

WT4.1 Study mechanisms for interworking with TSN transport networks. Study interworking mechanisms with TSN networks deployed in the transport network in order to support of E2E determinism and low latency communication and efficient N3 transmission;

Editor’s note: whether other low-latency, low-jitter transport systems is within scope of WT4.1 is FFS.

WT4.2 Study if it is beneficial to set the applications burst sending time based on feedback from RAN.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 3 | 2 | Yes | WT#1 is self-contained |
| WT#2 | 2 | 1.5 | No | WT#2 is self-contained |
| WT#3 | 2 | 1.5 | Yes | WT#3 is self-contained |
| WT#4 | 2 | 2 | Yes | WT#4 is self-contained |

Total TU estimates for the study phase: 9

Total TU estimates for the normative phase: 5.5

Total TU estimates: 9 + 5.5 = 14.5

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.abc | Study on timing resiliency and TSC & URLLC enhancements | SA#96 June 2022(TBD) | SA#97 Sep 2022(TBD) | Devaki Chandramouli, Devaki.chandramouli@nokia.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Devaki Chandramouli, Devaki.chandramouli@nokia.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Potential RAN impact to be covered by RAN WGs.

Potential security impact to be covered by SA3.

Potential charging and OAM impact to be covered by SA5.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

Verizon

AT&T

Sennheiser

Huawei

HiSilicon

Matrixx

ZTE

China Unicom

Vivo

NTT Docomo

ETRI

Xiaomi

Orange

China Mobile

Tencent

T-Mobile USA

Interdigital

Samsung

CATT

Deutsche Telekom

KDDI

Ericsson

Denso

BMWi

SK Telecom

Oracle

CBN

Broadcom

LG Electronics

NICT

Bell Canada