SA WG2 Meeting #147E (e-meeting)	S2-2107298r02

# 18-22 October, 2021, Electronic, Elbonia							         (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3274 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNPN corrections to clarify use of DNN and S-NSSAI for onboarding, AUSF discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Current wording in specifications “DNN and S-NSSAI used for onboarding” is unclear as it does not exclude using this DNN and S-NSSAI also for other purposes or use an existing DNN and S-NSSAI also for onboarding. - Current spec states that the Routing ID can used along with HNI to discover and select AUSF. It needs to be clear that the section 5.30.2.3 is referenced for completeness – otherwise, there is a risk one can use Routing ID even when the assumptions in section 5.30.2.3 are not true |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that DNN and S-NSSAI used for onboarding are only used in the context of UE onboarding. - Refer to 5.30.2.3 for UE configuration assumption |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear stage 2 specification as specification text is misleading for implementation and deployment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.9.2, 5.30.2.10.2.6, 5.30.2.10.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

##### 5.30.2.9.2	Credentials Holder using AAA Server for primary authentication and authorization

The AUSF in SNPN may support primary authentication and authorization of UEs that use credentials from an AAA Server in a Credentials Holder (CH).

-	If the UDM decides that the primary authentication is performed by AAA Server in CH based on UE subscription data and UE's SUPI, which is de-concealed by UDM from the SUCI received from AUSF, then the UDM instructs AUSF that primary authentication by a AAA server in a CH is required, the AUSF shall discover and select the NSSAAF, and then forward EAP messages to the NSSAAF. The NSSAAF selects AAA Server based on the domain name corresponds to the realm part of the SUPI, relays EAP messages between AUSF and AAA Server (or AAA proxy) and performs related protocol conversion. The AAA server acts as the EAP Server for the purpose of primary authentication.

-	The SUPI is used to identify the UE during primary authentication and authorization towards the AAA sever. SUPI privacy is achieved according to existing methods in Annex I.5 of TS 33.501 [29].

-	The AMF discovers and selects the AUSF as described in clause 6.3.4 using the Home Network Identifier (realm part) and Routing Indicator present in the SUCI provided by a UE configured as described in clause 5.30.2.3.

-	The AMF and SMF shall retrieve the UE subscription data from UDM using SUPI.

Figure 5.30.2.9.2-1 depicts the 5G System architecture for SNPN with Credentials Holder using AAA Server for primary authentication and authorization.

Figure 5.30.2.9.2-1: 5G System architecture with access to SNPN using credentials from Credentials Holder using AAA Server

* * * * Next change * * * *

5.30.2.10.2.6	Registration for UE onboarding

When the user or UE has selected an ON-SNPN according to clause 5.30.2.10.2.5, the UE establishes an RRC connection towards the NG-RAN node of the ON-SNPN. The UE provides an indication in RRC Connection Establishment that the RRC connection is for onboarding as defined in TS 38.331 [28]. This indication allows the NG-RAN node to select an appropriate AMF that supports the UE onboarding procedures. The UE indicates the ON-SNPN as the selected network, and the NG-RAN node shall indicate the selected PLMN ID and NID of the ON-SNPN to the AMF.

NOTE X:	As the configuration information in the UE does not include any S-NSSAI and DNN used for onboarding, the UE does not include S-NSSAI and DNN in RRC when it registers for UE onboarding purposes to the ONN.

The UE shall initiate the NAS registration procedure by sending a NAS Registration Request message with the following characteristics:

-	The UE shall set the 5GS Registration Type to the value "SNPN Onboarding" indicating that the registration request is for onboarding.

-	The UE shall provide a SUCI derived from a SUPI as specified in TS 23.003 [19] and TS 33.501 [29]. The SUPI shall uniquely identify the UE and shall be derived from the Default UE Credentials. The ON-SNPN may determine the corresponding DCS identity or address/domain, based on the SUCI (i.e. based on the Home Network Identifier of the SUCI).

The UE does not include a Requested NSSAI in NAS signalling when it registers for UE onboarding purposes to the ONN.

The AMF supporting UE onboarding is configured with AMF Onboarding Configuration Data that may include e.g.:

-	S-NSSAI and DNN to be used for UE onboarding or a configured SMF for the DNN and S-NSSAI used for UE onboarding.

-	Information to enable User Plane Remote Provisioning of UEs in SNPNs, see clause 5.30.2.10.4.

-	Information to use a local AUSF(s) within the ON-SNPN for onboarding of UEs with a SUCI for a DCS with AAA server.

NOTE Y:	 It is assumed that S-NSSAI and DNN to be used for onboarding are only used in the context of UE onboarding.

When the AMF receives a NAS Registration Request with a 5GS Registration Type set to "SNPN Onboarding", the AMF:

-	starts an authentication procedure towards the AUSF, the authentication procedure is specified in TS 33.501 [29]. The AMF selects an appropriate AUSF as described in clause 6.3.4 based on the Home Network Identifier of the SUCI used during onboarding.

-	applies the AMF Onboarding Configuration Data e.g. used to restrict UE network usage to only onboarding for user plane remote provisioning of UE as described in clause 5.30.2.10.4.3.

-	stores in the UE context in AMF an indication that the UE is registered for SNPN onboarding.

Upon successful authentication from AUSF, the AMF informs the UE about the result of the registration. If the UE is not successfully authenticated, the AMF shall reject the registration procedure for onboarding, and the UE may select a different ON-SNPN to attempt to register.

* * * * Next change * * * *

5.30.2.10.4.4	User Plane Remote Provisioning of UEs when Onboarding Network is a PLMN

Onboarding Services for a UE may be provided via PLMN using PDU Session for DNN(s) and S-NSSAI(s) used for onboarding. Subscription data of such a UE shall contain the DNN and S-NSSAI used for onboarding.

NOTE Z:	 It is assumed that DNN and S-NSSAI used for onboarding are only used in the context of UE onboarding.

The AMF selects an SMF used for remote provisioning using the SMF discovery and selection functionality as described in clause 6.3.2, considering the DNN and S-NSSAI used for onboarding provided by the UE or the default DNN and S-NSSAI provided by UDM.

The UPF selection function described in clause 6.3.3 is applied, considering the DNN and S-NSSAI used for onboarding.

The SMF may be configured with one or more PVS FQDN and/or PVS IP address(es) per DNN and S-NSSAI used for onboarding. The SMF may send the PVS FQDN and/or PVS IP address(es) associated to the DNN and S-NSSAI of the PDU Session to the UE as part of Protocol Configuration Options (PCO) in the PDU Session Establishment Response.

* * * * End of changes * * * *