3GPP TSG-SA2 Meeting # 147E (e-meeting)	S2-2107314r04

# Oct 18 – 22, 2021											 	 	                         (revision of S2-2105912)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3008 | rev | 1 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the NRF services between SNPN and DCS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The NF/NF service discovery services between SNPN and credential holder is introduced but the NF/NF service discovery between SNPN and DCS is still missing. In SA3#104 Adhoc, it has concluded the following: “Conclusions on KI #4: Securing initial access for UE onboarding between UE and SNPN For initial access for UE onboarding, the following authentication methods are concluded to be specified in normative work: - Primary authentication between UE and AUSF in the O-SNPN. No interaction with DCS during primary authentication. As currently specified in TS 33.501 [2]. - Primary authentication with mutual authentication between UE and DCS. AUSF is involved. DCS can be AAA server, in that case NSSAAF is involved. DCS can also be external entity using AUSF/UDM. ” Based on SA3 conclusion, it can deduce the DCS may host AUSF/UDM or not. For the two scenarios, the corresponding NF/NF service discovery procedures are different. When DCS hosts AUSF/UDM, the target AUSF/UDM are located in DCS, otherwise, when DCS doesn't host AUSF/UDM, the target AUSF/UDM are located in SNPN. In 23.501. it has agreed to use the realm part of NSI based SUCI/SUPI to discover DCS related NFs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the NF/NF service discovery between SNPN and DCS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NF/NF service discovery between SNPN and DCS is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.5a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

### 4.17.5a	NF/NF service discovery between SNPN and Credentials Holder hosting AUSF/UDM or between SNPN and DCS hosting AUSF/UDM

Figure 4.17.5a-1: NF/NF service discovery cross SNPN and Credentials Holder

In the case of a UE accessing SNPN using credentials from a Credentials Holder hosting AUSF/UDM, similar procedure can be used for service discovery across PLMNs as specified in clause 4.17.5 with the difference as below:

-	The serving PLMN is replaced by SNPN and home PLMN is replaced by CH;

-	In step 1:

-	the home PLMN ID in Nnrf_NFDiscovery_Request is replaced by identification for the Credentials Holder, i.e.:

-	the realm in the case of Network Specific Identifier based SUCI/SUPI; or

-	the MCC and MNC in the case of an IMSI based SUCI/SUPI;

NOTE:	When IMSI based SUPI is used for a UE of a CH, the IMSI is assumed to be globally unique and assigned by the owner of a PLMN ID containing MCC and MNC of the IMSI as defined in TS 23.501 [2].

-	the serving PLMN ID is replaced by SNPN ID (i.e. PLMN ID and NID);

-	In step 2, the NRF in SNPN identifies NRF in CH based on the identification for the Credentials Holder.

In the case of a UE accessing ON-SNPN using Default UE credentials from a DCS hosting AUSF/UDM, a similar procedure can be used than for service discovery across PLMNs as specified in clause 4.17.5, with the difference as below:

-	The serving PLMN is replaced by SNPN and home PLMN is replaced by DCS;

-	In step 1:

-	the home PLMN ID in Nnrf_NFDiscovery_Request is replaced by identification for the DCS, i.e.:

-	the realm in the case of Network Specific Identifier based SUCI/SUPI;

-	the serving PLMN ID is replaced by SNPN ID (i.e. PLMN ID and NID);

-	In step 2, the NRF in SNPN identifies NRF in DCS based on the identification for the DCS.

****************** END CHANGES ***************