3GPP TSG-SA WG2 Meeting #147E e-meeting	S2-2107322

# Elbonia, Oct 18 – 22, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0009 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UUAA-MM Procedure Updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) Currently in clause 5.2.2.1, step 4, only AMF behaviour of stroing the UUAA-MM failure in UE context is mentioned but failure indication needs to be informed to UE as well which has been missed. If UUAA is configured in the AMF to be performed during 5GS registration, the UE did not provide a CAA-Level UAV ID in the registration request in step 1, but UE has aerial subscription in the UE subscription data retrieved from UDM in step 2, then the AMF accepts the registration and ensures that the UE is not allowed to access any aerial service by storing in the UE context that 'UUAA-MM has FAILED', and further rejecting PDU session establishment requests for aerial services (identified by DNN/S-NSSAI). Proposal 1: AMF shall update the UUAA-MM failure indication while accepting the registration request. Without this information UE will not be knowing that UAS service is not supposed to be initiated. 3) Currently in clause 5.2.2.1, step 4, network behaviour has been addressed which is rejecting the registration with indication that UAS service is not allowed. If UUAA is configured in the AMF to be performed during 5GS registration and the UE has provided a CAA-Level UAV ID in the registration request in step 1, but the UE does not have an aerial subscription in the UE subscription data retrieved from the UDM in step 2, then the AMF rejects the registration with an indication that UAS services are is not allowed which triggers the UAV to not re-register for aerial services and ensures that the UE is not allowed to access any aerial service. Here UE may do normal registration to get services from network. During this time period if operator is enabling the aerial subscription for the UE, it is not specified how this information will be updated to UE. Proposal 3: It is proposed that AMF shall trigger a UE configuration update procedure to inform the UE about the enabling of the aerial subscription. Without this information UE will be never able to get aerial service. 4) Currently in clause 5.2.2.1, step 4, specification mentioned that UE shall do deregistration followed by initial registration to get aerial services. If UUAA is configured in the AMF to be performed during 5GS registration, the UE did not provide a CAA-Level UAV ID in the registration request in step 1, but UE has aerial subscription in the UE subscription data retrieved from UDM in step 2, then the AMF accepts the registration and ensures that the UE is not allowed to access any aerial service by storing in the UE context that 'UUAA-MM has FAILED', and further rejecting PDU session establishment requests for aerial services (identified by DNN/S-NSSAI). At a later point in time, if the UE wants to use the aerial services by providing the CAA Level UAV ID later on via UUAA-MM procedure, then the UE shall first perform UE-initiated Deregistration procedure as explained in clause 4.2.2.3.2 of TS 23.502 [3] followed by an Initial Registration to the 5GS including the CAA Level UAV ID in the registration request. We feel this will lead to full wastage of radio as well as network resources as it involves one complete deregistration followed by one complete registration call flow. This can be hanlded by UE using Mobility registration update procedure. Proposal 4: It is proposed that UE shall initiate Mobility registration update procedure to get aerial service |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - AMF shall update the UUAA-MM failure indication while accepting the registration request. Without this information UE will not be knowing that UAS service is not supposed to be initiated. - It is proposed that AMF shall trigger a UE configuration update procedure to inform the UE about the enabling of the aerial subscription. Without this information UE will be never able to get aerial service. - It is proposed that UE shall initiate Mobility registration update procedure to get aerial service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviour will be undefined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

### 5.2.2	UUAA at Registration in 5GS (UUAA-MM)

#### 5.2.2.1	General

The UUAA-MM procedure is optional and triggered for a UE that requires UAV authentication and authorization by a USS when registering with 5GS. The UUAA-MM procedure is triggered by the AMF. UUAA-MM is triggered during the UE Registration based on the local network policy, if the UE has an Aerial UE subscription with the 5GS and if the UE has provided the CAA-Level UAV ID of the UAV in the Registration Request, or when the USS that authenticated the UAV triggers a re-authentication.

The UE is authenticated and authorized by USS using a CAA-Level UAV ID and credentials associated to the CAA-Level UAV ID, different from the 3GPP subscription credentials (e.g. SUPI and credentials used for PLMN access). During UUAA-MM procedure, the AMF communicates with the USS via a UAS NF and forwards authentication messages transparently between the UE and UAS NF.

UAS NF stores the UAV UEs UUAA context after successful UUAA procedure. The UUAA context may be stored in the UDSF or may be stored locally in the UAS NF depending on deployments. The UAS NF shall also create an implicit subscription for notification towards the AMF after the successful UUAA procedure. This notification is used by the UAS NF to trigger re-authentication, update authorization data or revoke authorization of the UAV, upon receipt of such request from the USS.

Figure 5.2.2.1-1: UUAA in the context of the Registration procedure (UUAA-MM)

1.	The UE sends a Registration request message and, if configured with one, it shall provide a CAA-level UAV ID of the UAV and optionally a USS address when registering for UAS services.

2.	If primary authentication is required (e.g. if this is an initial Registration), AMF invokes it as described in step 9 in Figure 4.2.2.2.2-1 of TS 23.502 [3]. Subsequently AMF retrieves UE subscription data from UDM as described in step 14 in Figure 4.2.2.2.2-1 of TS 23.502 [3] - (not shown in the figure).

3.	AMF shall determine whether UUAA-MM is required for the UAV. The AMF decides that UUAA is required if:

a)	the UE has a valid Aerial UE subscription information;

b)	UUAA is to be performed during Registration according to local operator policy;

c)	there is no successful UUAA result from a previous UUAA-MM procedure;

d)	the UE has provided a CAA-Level UAV ID.

AMF shall not perform UUAA-MM for non-3GPP access and shall ensure that the UE is not allowed to access any aerial services in non-3GPP access by rejecting PDU session establishment requests for aerial services (identified by DNN/S-NSSAI).

4.	If AMF determines in step 3 that a UUAA-MM is to be performed, AMF shall include a pending UUAA-MM indication in the Registration Accept message. The AMF stores in the UE context that a UUAA is pending. The UE shall wait for completion of the UUAA-MM procedure without attempting to register for UAS services or to establish user plane connectivity to USS or UAV-C.

If AMF determines that UUAA is not to be performed during this Registration procedure, UUAA may be triggered during PDU Session Establishment later on.

If UUAA is configured in the AMF to be performed during 5GS registration and the UE has provided a CAA-Level UAV ID in the registration request in step 1, but the UE does not have an aerial subscription in the UE subscription data retrieved from the UDM in step 2, then the AMF rejects the registration with an indication that UAS services are not allowed which triggers the UAV to not re-register for aerial services and ensures that the UE is not allowed to access any aerial service.

After the UE is registered successfully as a normal UE and if there has been an update in aerial subscription from disabled to enabled, then AMF shall trigger a UE Configuration Update procedure as specified in clause 4.2.4.2 of TS 23.502 [3] to deliver this information. If the UE wants to use aerial service, it can perofrm Mobility Registration Update by sending CAA-Level-ID.

If UUAA is configured in the AMF to be performed during 5GS registration, the UE did not provide a CAA-Level UAV ID in the registration request in step 1, but UE has aerial subscription in the UE subscription data retrieved from UDM in step 2, then the AMF shall include a failure UUAA-MM indication while accepting the registration request and ensures that the UE is not allowed to access any aerial service by storing in the UE context that 'UUAA-MM has FAILED', and further rejecting PDU session establishment requests for aerial services (identified by DNN/S-NSSAI). At a later point in time, if the UE wants to use the aerial services by providing the CAA Level UAV ID later on via UUAA-MM procedure, then the UE shall  perform Mobility Registration Update  as explained in clause 4.2.2.2.2  of TS 23.502 [3].

5.	If UE indicates its support for Network Slice-Specific Authentication and Authorization (NSSAA) procedure in the UE MM Core Network Capability, and if the UE includes Requested S-NSSAI in Registration Request which is subject to NSSAA, however, the Requested S-NSSAI has not been successfully authenticated, the NSSAA procedure is executed as described in clause 4.2.2.2.2 of TS 23.502 [3].

6.	If required based on step 3 determination, and if the S-NSSAI that is associated with the UAS services is part of the Allowed NSSAI, UUAA-MM procedure (see clause 5.2.2.2) is executed at this step. Once the UUAA-MM procedure is successfully completed for the UAV, the AMF stores a successful UUAA result and updates the UE context indicating that UUAA is no longer pending and with an optional authorized CAA-Level UAV ID received from the USS, and shall trigger a UE Configuration Update procedure (see TS 23.502 [3], clause 4.2.4.2) to deliver the UUAA result and the UUAA Authorization Payload containing UAV configuration to the UE. The AMF may also deliver the authorized CAA-Level UAV ID that it may have received from the USS.

NOTE 1:	The UAV configuration is application layer information outside the scope of 3GPP.

If UUAA fails, based on local network policy, the AMF may decide to de-register the UE with an appropriate cause value in the De-Registration Request message, or keep the UE-registered with a failure UUAA result in UE context as described in step 7 of clause 5.2.2.2 ensures that the UE is not allowed to access any aerial service based on the DNN/S-NSSAI value. If the UE is de-registered, the UE may re-attempt to re-register without including the CAA-level UAV ID.

NOTE 2:	Security details will be determined by SA WG3.

Editor's note:	It is FFS, how to coordinate UUAA status of UE(UAV) with UAS NF, when UE registration status changes due to 5GS initiated events (e.g. NSSAA failure).

* * * * end of changes * * * *