SA WG2 Meeting #147E (e-meeting)	S2-2107368

# 18-22 October, 2021, Electronic, Elbonia									  (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0661 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for PVS address formats |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clarify for SNPN UE Remote Provisioning specified in clause 6.1.3.25 that Provisioning Server address(es) may include IP address(es) and FQDN(s) to align with agreements in clause 5.30.2.10.4.2 of TS 23.501 and with step 7b in clause 4.3.2.2.1 of TS 23.502. Also, clarify that DNS address(es) are locally configured at the PCF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that Provisioning Server address(es) may include IP address(es) and FQDN(s). Also, clarify that DNS address(es) are locally configured at the PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 23.503 not aligned with TS 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.10.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

#### 6.1.3.25	SNPN UE Remote Provisioning support via User Plane

SNPN UE remote provisioning support via user plane is specified in clause 5.30 of TS 23.501 [2].

SNPN UE remote provisioning is a network service provided through a DNN and S-NSSAI used for UE onboarding.

For a PDU Session with a DNN and S-NSSAI used for UE onboarding, the PCF may make authorization and policy decisions that restrict the use of the PDU Session, e.g. by restricting the traffic to/from Provisioning Server address(es) and DNS server address(es) only.

For a PDU Session established to the DNN and S-NSSAI used for onboarding, the PCF does not perform a subscription check. Instead, the PCF uses the locally stored Onboarding Configuration Data for this DNN and S-NSSAI combination to make authorization and policy decisions.

The PCF sets the SDF template of the PCC rule(s) according to the Onboarding Configuration Data for this DNN and S-NSSAI combination that may include the Provisioning Server address(es) and DNS server address(es). If the PCF receives Provisioning Server address(es) or DNS server address(es) from the SMF, then the PCF creates the SDF template in the PCC rule(s) using the received Provisioning Server address(es) or DNS server address(es) instead of using the Provisioning Server address(es) or DNS server address(es) stored locally as part of the Onboarding Configuration Data.

The PCC Rule Authorization function selects QoS parameters applicable to the SNPN UE remote provisioning service. Then, the PCF installs these PCC Rules at the SMF to enable traffic to/from the dedicated IP addresses (i.e. Provisioning Server address(es) and DNS server address(es)) with the associated QoS.

The PCC Rules provided by the PCF take precedence over the locally stored policy used for the restricted PDU Session at the SMF.

* * * * End of changes * * * *