3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107387

# Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0017 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on EAS Discovery Procedure with EASDF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Lenovo, Motorola Mobility, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Node Level DNS Handling Information is defined in SA2-146e, the introduction of Node Level DNS Handing Information could reduce the size of each session level DNS context, and avoid session modification related signalling overhead between SMF and EASDF in case the application is dynamically deployed or local DNS server is changed. However, there are still leftover issues about whether the Node Level DNS Handling Information on EASDF should be per SMF or should be shared by different SMF sets. The following is an illustration of these two options: Figure 1 Two options for Node Level DNS Handing Information For option 1, each SMF set creates Node Level DNS Handling Information for its own use, not sharing with other SMFs, there will be duplicated Information stored in EASDF from different SMF. For option 2, there is only one copy of Node Level DNS Handing Information, which is shared and maintained by different SMFs in the SMF sets, This contribution specifies that The Node Level DNS message detection template ID and the Node level DNS handling actions ID are unique within an EASDF. This contribution also clarifies the use of Node Level DNS message detection template ID. SA2#146E meeting agreed to introduce Node Level DNS message detection template ID in DNS message handling rules, in order to refer to Node Level DNS message detection template. However, such utilization is not specified in TS 23.548. This contribution clarifies the use of Node Level DNS message detection template ID. According to the action part of DNS message handling rules, at least one of the following may be included in the DNS message handling rules: - (option A) The information to build ECS option, or reference to the Node Level DNS handling actions information. - (option B) Local DNS server IP address, or reference to the Node Level DNS handling actions information. . In clause 6.2.5.0 of TS23.501, there is a reference to TS23.548 for EAS deployment functionality of NEF as follows: - Support of EAS deployment functionality: Details are defined in TS 23.548 [130]. This contribution complements the description of EAS deployment functionality of NEF. This contribution also clarify how SMF send control information to EASDF for report once control. Also, In Figure 6.2.3.2.2-1 the EASDF shall be configured to UE as DNS server after the creation of the DNS context in the EASDF. Hence, the description in step 3 is moved to step 4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some updates on EAS Discovery Procedure with EASDF as described above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Edge computing procedures incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.2, 7.1.3, 6.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.2.3.2.2	EAS Discovery Procedure with EASDF

For the case that the UE DNS Query is to be handled by EASDF, the following applies.

-	The AF may provide EAS Deployment Information to NEF which may store it in UDR, , as defined in clause  6.2.3.4. SMF may retrieve EAS Deployment Information from NEF as described in clause 6.2.3.4 or has locally preconfigured information. EAS Deployment Information is not dedicated to specific UE session(s).

-	The SMF may provide Node Level DNS Handling Information to EASDF, the Node Level DNS Handling Information are derived from EAS Deployment Information provided by AF and are not dedicated to specific PDU session; SMF configures EASDF with Node Level DNS Handling Information according to the procedures defined in clause 6.2.3.4.

Node Level DNS Handling Information  may apply to all PDU sessions with a certain (DNN, S-NSSAI).

The Node Level DNS message detection template ID may be used by the EASDF to refer to Node Level DNS message detection template, and derive array of FQDN ranges and/or array of EAS IP address ranges. The Node level DNS handling actions ID may be used by the EASDF to refer to Node Level DNS handling actions information, and derive actions related parameters.

The Node Level DNS message detection template ID and the Node level DNS handling actions ID are unique within an EASDF

.

NOTE 3:   The definition of structure of Node Level DNS handling actions ID and Detection template ID is left to stage 3.

Node Level DNS Handling Information may contain either a Node Level DNS message detection template or Node level DNS handling actions information. It may be updated or deleted using Node Level DNS message detection template ID or Node Level DNS handling actions ID to identify the updated or deleted information

-	Node Level DNS message detection template

-	Node Level DNS message detection template ID

-	DNS message type = DNS Query or DNS Response:

-	If DNS message type = DNS Query:

-	Array of (FQDN ranges).

-	If DNS message type = DNS Response:

-	Array of FQDN ranges and/or array of EAS IP address ranges.

-	Node level DNS handling actions information

-	Node Level DNS handling actions ID

-	ECS option

-	Local DNS server IP address.

NOTE 1:	The FQDN can be set to wildcard to indicate the default DNS Server (e.g. the C-DNS), for the case in which the DNS message should be forwarded to the default DNS Server.

NOTE 2:	The node level DNS Handling Information can be configured for a specific application with the related FQDN set as the detection template.

NOTE x: The Node Level DNS Handling Information is maintained per SMF set.

-	During the PDU Session establishment procedure, the SMF gets the EAS Deployment Information via the PDU Session related policy information from PCF or the SMF is preconfigure with the EAS Deployment Information based on UE subscription, and the SMF selects an EASDF and provides its address to the UE as the DNS Server to be used for the PDU Session.

The SMF configures the EASDF with DNS message handling rules to handle DNS messages related to the UE(s). The DNS message handling rule has a unique identifier and includes information used for DNS message detection and associated action(s). The DNS handling rules is defined as following:

-	Precedence of the DNS message handling rule

-	DNS Handling Rule Identity

-	A Node Level DNS message detection template ID and/or a DNS message detection template (which includes at least one of the following):

-	DNS message type = DNS Query or DNS Response:

-	If DNS message type = DNS Query:

-	Source IP address (i.e. UE IP address).

-	Array of (FQDN ranges) (optional).

-	If DNS message type = DNS Response:

-	Array of FQDN ranges and/or array of EAS IP address ranges (optional).

NOTE 4:	For DNS message type = Query, the UE IP address provided at DNS context creation (Neasdf_DNSContext_Create Request) is considered if not provided explicitly as part of the DNS message detection template.

-	Action(s) (includes at least one action); the possible actions include:

-	Report DNS message content to SMF (i.e. target FQDN and if available: IP address information provided back by the authoritative DNS server) and buffer the DNS message, including reporting-once indication. If this indication is included, the EASDF reports the DNS message content to the SMF once if the DNS message detection template matches the first incoming DNS Query or DNS Response message.

NOTE 5:	With reporting-once indication, the DNS message detection template should contain EAS IP address ranges corresponding to the same DNAI. Resetting the Reporting-once indication can be used by the SMF to allow reporting associated with a DNS handling rule when the SMF has removed the UL-CL/BP e.g. when the UE has moved out of the area associated with the current DNAI and thus insertion of a new UPF offloading capability can be considered.

-	Send the DNS message(s) to a DNS server/resolver(s) as follows:

-	(possibly) Including the information to build optional EDNS Client Subnet option in the DNS message (The information for the EASDF to build the EDNS Client Subnet option is included in the DNS handling rule, or set to Node Level DNS handling actions ID  as a reference to the Node level DNS handling actions Information . This corresponds to the option A defined below.

-	Replacement of the DNS message target address with either DNS Server Address indicated in the rule, or from the Node level DNS handling actions Information referred to by Node Level DNS handling actions ID included in the DNS handling rules; if no DNS Server Address is provided by the SMF in the rule, then the EASDF is to forward the DNS message to a locally preconfigured DNS server/resolver. This corresponds to the option B defined below.

-	Send the buffered DNS response(s) message to UE.

-	Discard cached DNS response message(s).

When the EASDF forwards a DNS message (to the UE or towards a DNS server over N6), it uses its own address as the source address of the DNS message.

The SMF may use following information to create DNS message handling rules associated with a PDU session:

-	Local configuration associated with the (DNN, S-NSSAI) of the PDU Session; and/or

-	EAS Deployment Information provided by the AF or preconfigured in the SMF; and/or

-	Information derived from the UE location such as candidate L-PSA(s);

-	PDU Session information, like PDU Session L-PSA(s) and ULCL/BP;

NOTE 6:	For example, the SMF can derive the IP address for ECS based on the N6 IP address(es) associated with serving L-PSA(s) locally configured or in the NRF.

NOTE 7:	Providing in DNS EDNS Client Subnet option an IP address associated with the L-PSA UPF protects the privacy of the (IP address of the) UE.

-	If the FQDN in a DNS Query matches the FQDN(s) provided by the SMF in a DNS message detection template, based on instructions by SMF, one of the following options is executed by the EASDF based on a corresponding DNS message handling rule:

-	Option A: The EASDF includes an EDNS Client Subnet (ECS) option into the DNS Query message as defined in RFC 7871[6] and sends the DNS Query message to the DNS server for resolving the FQDN. The DNS server may resolve the EAS IP address considering the EDNS Client Subnet option and sends the DNS Response to the EASDF;

-	Option B: The EASDF sends the DNS Query message to a Local DNS server which is responsible for resolving the FQDN within the corresponding L-DN. The EASDF receives the DNS Response message from the Local DNS server.

NOTE 8:	Option B does not support the scenario where the PSA UPF for transferring DNS query between EASDF and DNS server, or the EASDF has no direct connectivity with the Local DNS servers.

The SMF instructions for a matching FQDN may as well indicate EASDF to contact SMF. SMF then provides the EASDF with a DNS message handling rule;

-	If the DNS Query from the UE does not match a DNS message handling rules set by the SMF, then the EASDF may simply forward the DNS Query towards a preconfigured DNS server/resolver for DNS resolution;

-	When the EASDF receives a DNS Response message, the EASDF notifies the EAS information (i.e. EAS IP address(es), the EAS FQDN and if available the corresponding IP address within the ECS DNS option) to the SMF if the DNS message reporting condition provided by the SMF is met (i.e. the EAS IP address or FQDN is within the IP/FQDN range). The SMF may then select the target DNAI based on the EAS information and trigger UL CL/BP and L-PSA insertion as specified in clause 6.3.3 in TS 23.501 [2] based on the Notification.

NOTE 9:	To avoid SMF overloading caused by massive reporting, the overload control mechanisms defined in clause 6.4 of TS 29.500 [9] can be used.

The information to build the EDNS Client Subnet option or the Local DNS server address provided by the SMF to the EASDF are part of the DNS message handling rules to handle DNS queries from the UE. This information is related to DNAI(s) for that FQDN(s) for the UE location. The SMF may provide DNS message handling rules to handle DNS queries from the UE to the EASDF when the SMF establishes the association with the EASDF for the UE and may update the rules at any time when the association exists. For the selection of the candidate DNAI for an FQDN for the UE, the SMF may consider the UE location, network topology, EAS Deployment Information and related policy information for the PDU Session provided/modified/deleted as defined in TS 23.503 [4] clause 6.4 or be preconfigured into the SMF. After the UE mobility, if the provided Information for EDNS Client Subnet option or the Local DNS server address needs be updated, the SMF may send an update to DNS message handling rules to the EASDF.

NOTE 10:	If multiple candidate DNAIs are available after considering the UE location, network topology and EAS deployment, the SMF selects one DNAI from the multiple ones based on operator's policy. For examples, the SMF can select the DNAI randomly, or based on selection weight factor if provided by AF, or select the DNAI closest to the UE location.

NOTE 11:	To protect the SMF (e.g. to block DOS from the EASDF), the EASDF IP address for DNS Query Request is only accessible from the UE IP address via UPF.

Once the UL CL/BP and L-PSA have been inserted, the SMF may decide that the DNS messages for the FQDN are to be handled by Local DNS resolver/server from now on. This option is further described in clause 6.2.3.2.3.

To avoid EASDF sending redundant DNS message reports triggering UL CL/BP insertion corresponding to the same DNAI, the SMF may send report-once control information (i.e. DNS message handling rule with DNS message detection template containing EAS IP address ranges corresponding to the DNAI, and with reporting-once indication set) to EASDF to instruct the EASDF to report only once for the DNS messages matching with the DNS message detection template of the report-once control information for the DNAI. In addition, the SMF may instruct the EASDF not to report DNS responses to SMF corresponding to some FQDN ranges and/or EAS IP address ranges e.g. once the UL CL/BP and L-PSA have been inserted for the corresponding EAS IP address ranges for Pre-established session breakout while there is configuration for the related EASDF reporting DNS responses. After the removal of the L-PSA, the SMF may instruct the EASDF to restart the reports of the DNS messages.

If the SMF, based on local configuration, decides that the interaction between EASDF and DNS Server in the DN shall go via an UPF, the SMF sends corresponding N4 rules to this UPF to instruct this UPF to forward DNS message between EASDF and the external DNS server. In this case, DNS messages between EASDF and DNS Server described in this clause are transferred via this UPF transparently.

NOTE 12:	Based network configuration, one UPF is used to transmit DNS signalling between EASDF and DNS servers. The N4 session between the SMF and this UPF is not related to a specific PDU Session but provides rules targeting Downlink traffic from DNS servers to the EASDF and associated with the traffic of multiple UE(s); the traffic forwarding between EASDF and this UPF is realized by IP in IP tunnelling .The EASDF provides the SMF with the source address it uses to contact DNS servers and with the destination address where it expects to receive the tunnelled traffic.

Figure 6.2.3.2.2-1: EAS discovery procedure with EASDF

1.	UE sends PDU Session Establishment Request to the SMF as shown in step 1 of clause 4.3.2.2.1 of TS 23.502 [3].

2.	During the PDU Session Establishment procedure, the SMF selects EASDF as described clause 6.3 of TS 23.501 [2]. The SMF may consider the UE subscription information to select an EASDF as the DNS server of the PDU Session.

If the SMF, based on local configuration, decides that the interaction between EASDF and DNS Server in the DN shall go via the PSA UPF, the SMF configures PSA UPF within N4 rules to forward the DNS message between EASDF and DN.

3.	The SMF invokes Neasdf_DNSContext_Create Request (UE IP address, SUPI, DNN, notification endpoint, (DNS message handling rules)) to the selected EASDF.

This step is performed before step 11 of PDU Session Establishment procedure in clause 4.3.2.2.1 of TS 23.502 [3].

After this step, the SMF includes the IP address of the EASDF as DNS server/resolver for the UE in the PDU Session Establishment Accept message as defined in step 11 of clause 4.3.2.2.1 of TS 23.502 [3]. The UE configures the EASDF as DNS server for that PDU Session.

The EASDF creates a DNS context for the PDU Session and stores the UE IP address, SUPI, the notification endpoint and potentially provided DNS message handling rule(s) into the context.

The EASDF is provisioned with the DNS message handling rule(s), before the DNS Query message is received at the EASDF or as a consequence of the DNS Query reporting.

4.	The EASDF invokes the service operation Neasdf_DNSContext_Create Response.

After this step, the SMF includes the IP address of the EASDF as DNS server/resolver for the UE in the PDU Session Establishment Accept message as defined in step 11 of clause 4.3.2.2.1 of TS 23.502 [3]. The UE configures the EASDF as DNS server for that PDU Session.

Editor's note:	How to guarantee that the UE uses the EASDF's IP address for the subsequent DSN Query in step 8 is FFS.

5.	The SMF may invoke Neasdf_DNSContext_Update Request (EASDF Context ID, (DNS message handling rules)) to EASDF. The update may be triggered by UE mobility, e.g. when UE moves to a new location, or by a reporting by EASDF of a DNS Query with certain FQDN, or, the update may be triggered by insertion/removal of Local PSA, e.g. to update rules to handle DNS messages from the UE or by new PCC rule information.

6.	The EASDF responds with Neasdf_DNSContext_Update Response.

7.	The UE sends a DNS Query message to the EASDF.

8.	If the DNS Query message matches the DNS message handling rule for reporting, the EASDF sends the DNS message report to SMF by invoking Neasdf_DNSContext_Notify Request (information from the DNS Query e.g. target FQDN of the DNS Query).

9.	The SMF responds with Neasdf_DNSContext_Notify Response.

10.	If DNS message handling rule for the FQDN received in the report need to be updated, e.g. provide updates to information to build the EDNS Client Subnet option information, the SMF invokes Neasdf_DNSContext_Update Request (DNS message handling rules) to EASDF.

For Option A, the DNS handling rule includes corresponding IP address to be used to build the EDNS Client Subnet option. For Option B, the DNS handling rule includes corresponding Local DNS Server IP address. The EASDF may as well be instructed by the DNS handling rule to simply forward the DNS Query to a pre-configured DNS server/resolver.

11.	The EASDF responds with Neasdf_DNSContext_Update Response.

12.	The EASDF handles the DNS Query message received from the UE as the following:

-	For Option A, the EASDF adds the EDNS Client Subnet option into the DNS Query message as specified in RFC 7871[6] and sends it to C-DNS server;

-	For Option B, the EASDF sends the DNS Query message to the Local DNS server.

If no DNS message detection template within the DNS message handling rule provided by the SMF matches the requested FQDN in the DNS Query, the EASDF may simply send a DNS Query to a pre-configured DNS server/resolver.

13.	EASDF receives DNS Responses from the DNS system and determines that a DNS Response can be sent to the UE.

14.	The EASDF sends DNS message reporting to the SMF by invoking Neasdf_DNSContext_Notify request including EAS information if the EAS IP address or the FQDN in the DNS Response message matches the reporting condition provided by the SMF. The DNS message reporting may contain multiple EAS IP address if the EASDF has received multiple EAS IP address(es) from the DNS servers it has contacted. The DNS message reporting may contain the FQDN and the EDNS Client Subnet option received in the DNS Response message.

Per the received DNS message handling rule, the EASDF does not send the DNS Response message to the UE but waits for SMF instructions (in step 17), i.e. buffering the DNS Response message.

If the DNS response(s) is required to be buffered and reported to the SMF, when the reporting-once control information is set, EASDF only reports to SMF once by invoking Neasdf_DNSContext_Notify request for DNS responses matching with the DNS message detection template.

15.	The SMF invokes Neasdf_DNSContext_Notify Response service operation.

16.	The SMF may perform UL CL/BP and Local PSA selection and insert UL CL/BP and Local PSA.

Based on EAS information received from the EASDF in Neasdf_DNSContext_Notify, other UPF selection criteria, as specified in clause 6.3.3 in TS 23.501 [2], and possibly Service Experience or DN performance analytics for an Edge Application as described in TS 23.288 [10], the SMF may determine the DNAI and determine the associated N6 traffic routing information for the DNAI. The SMF may perform UL CL/BP and Local PSA selection and insertion as described in TS 23.502 [3]. In case of UL CL, the traffic detection rules and traffic routing rules are determined by the SMF based on IP address range(s) per DNAI included in the EAS Deployment Information or according to AF Request as defined in clause 5.6.7 in TS23.501[2] or according to preconfigured information.

17.	The SMF invokes Neasdf_DNSContext_Update Request (DNS message handling rules).

The DNS message handling rule indicates the EASDF to send a DNS Response buffered in step 14 to UE. Other DNS message handling rule may indicate the EASDF not to send further DNS Response message corresponding to FQDN ranges and/or EAS IP address ranges.

18.	The EASDF responds with Neasdf_DNSContext_Update Response.

19.	The EASDF sends the DNS Response to UE.

# * * * * Second change * * * *

#### 6.2.3.4	Node Level EAS Deployment Information Management

##### 6.2.3.4.1	General

Node Level EAS Deployment Information management refers to the capability to create, update or remove EAS Deployment Information in the UDR and the distribution from UDR to the SMF and to the EASDF. The NEF is in charge of the management of EAS Deployment Information in provision procedure from AF to UDR, and in distribution procedure from UDR to SMF.

The EAS Deployment Information indicates how edge services are deployed in each Local DN, the description of EAS Deployment Information is shown in Table 6.2.3.4-1.

Table 6.2.3.4-1 Description of EAS Deployment Information

| Parameters | Description |
| --- | --- |
| DNN | DNN for the EAS Deployment Information. [optional] |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [optional] |
| Application ID | Identifies the application for which the EAS Deployment Information corresponds to. |
| FQDN(s) | Supported FQDN(s) for application(s) deployed in the EDNs or Local DNs. |
| DNS Server Information | list of DNS server identifier (consisting of IP address and port) for each DNAI. [optional] |
| IP range Information | IP range(s) of the EASs in the local DN for each DNAI. [optional] |

The node level EAS Deployment Information management procedures are described in this clause, the procedures are independent of any PDU Session, including:

-	The procedure for EAS Deployment Information management from AF via the NEF.

-	The procedure for EAS Deployment Information management in the SMF;

-	The procedure for Node Level DNS Handling Information management in the EASDF; and

# * * * * Third change * * * *

### 7.1.3	Neasdf_NodeLevelDNSHandlingInfo Service

#### 7.1.3.1	General

This service provides the capability to create, update or remove Node Level DNS Handling Information in EASDF. See clause6.2.3.4.4 for detailed procedure.

#### 7.1.3.2	Neasdf_NodeLevelDNSHandlingInfo_Create Service Operation

Service operation name: Neasdf_NodeLevelDNSHandlingInfo_Create.

Description: Create the Node Level DNS Handling Information in EASDF.

Input, Required: Node Level DNS Handling Information.

Input, Optional: None.

Output, Required: Success or Failure.

Output, Optional:.

#### 7.1.3.3	Neasdf_NodeLevelDNSHandlingInfo_Update Service Operation

Service operation name: Neasdf_NodeLevelDNSHandlingInfo_Update.

Description: Update the Node Level DNS Handling Information in EASDF.

Input, Required: Updated Node Level DNS Handling Information.

Input, Optional: None.

Output, Required: Success or Failure.

Output, Optional: None.

#### 7.1.3.4	Neasdf_NodeLevelDNSHandlingInfo_Delete Service Operation

Service operation name: Neasdf_NodeLevelDNSHandlingInfo_Delete.

Description: Delete the Node Level DNS Handling Information in EASDF.

Input, Required: Node Level DNS message detection template ID and/or Node Level DNS handling actions ID .

Input, Optional: None.

Output, Required: Result.

Output, Optional: None.

# * * * * Fourth change * * * *

#### 6.2.3.4	Node Level EAS Deployment Information Management

##### 6.2.3.4.1	General

Node Level EAS Deployment Information management refers to the capability to create, update or remove EAS Deployment Information in the UDR and the distribution from UDR to the SMF and to the EASDF.

The NEF supports EAS Deployment information management function to retrieve EAS Deployment Information in the UDR and provide EAS Deployment Information to the SMF based on the subscription from SMF, The EAS Deployment Information management function also support EAS Deployment Information create/update/delete from AF to UDR.

The EAS Deployment Information indicates how edge services are deployed in each Local DN, including the following comment information:

-	The list of DNS server identifier (consisting of IP address and port) for each DNAI, and/or,

-	upported FQDN(s) for application(s) deployed in the EDNs or Local DNs.

-	IP ranges of the EASs in the local part of the DN for each DNAI.

The node level EAS Deployment Information management procedures are described in this clause, the procedures are independent of any PDU Session, including:

-	The procedure for EAS Deployment Information management from AF via the NEF.

-	The procedure for EAS Deployment Information management in the SMF;

-	The procedure for Node Level DNS Handling Information management in the EASDF; and

# * * * * End of changes * * * *