SA WG2 Meeting #147E	S2-2107400r03

Oct 18 - 22, 2021, Electronic

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0458 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on prepared analytics transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - There is an Editor's note in the procedure for prepared analytics transfer: Editor's note: It FFS how this aligns with the procedure when Analytics consumer changes. To resolve this EN, it is proposed that during the prepared analytics subscription transfer procedure, if the source NWDAF receives an Nnwdaf_AnalyticsInfo_ContextTransfer request service operation from an NWDAF which is different from any candidate target NWDAFs selected by the source NWDAF, the source NWDAF considers that NWDAF reselection is performed by NWDAF analytics consumer and cancels the prepared analytics subscription transfer to all the candidate target NWDAFs. - There are descriptions that only applies to UE related analytics subscription, e.g. "the source NWDAF estimates that the UE might enter an area which is not covered by the source NWDAF" and then initiates prepared analytics transfer; The target NWDAFs (e.g. NWDAFy and NWDAFz) may allocate a new subscription Id …"following the behaviour of AmfEventSubscription as specified in clause 6.2.6.2.2 of TS 29.518 [25] and UEContextTransfer as specified in clause 5.2.2.2.1.1 of TS 29.518 [25]". These should be clarified or removed. - In clause 6.1B.2.3 step 0b, it describes that "The source NWDAF…receive notifications on UE mobility events from AMF and/or SMF, respectively." However, the NWDAF does not receive notifications on UE mobility events from the SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Resolve the Editor's note on how the prepared analytics procedure aligns with the procedure when Analytics consumer changes. - Clarify that some descriptions only applies to UE related analytics transfer. - Remove the incorrect description that the source NWDAF receives notifications on UE mobility events from SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor's note unresolved. Unclear/incorrect specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1B.2.3 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

#### 6.1B.2.3	Prepared Analytics transfer

The procedure in Figure 6.1B.2.3-1 is used by an NWDAF instance to request another NWDAF instance for a prepared analytics subscription transfer from the source NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5.

NOTE 1:	The source NWDAF might determine that it needs to prepare to transfer analytics to another NWDAF instance, e.g. when the source NWDAF estimates for UE related analytics subscription that the UE might enter an area which is not covered by the source NWDAF (e.g., by subscribing to AMF event exposure service for UE mobility event notifications, by performing UE mobility analytics, or by subscribing to another NWDAF providing UE mobility analytics). If the procedure makes use of predictions to determine the candidate NWDAFs, care must be taken with regards to load and signalling cost when sending data to an NWDAF that will not eventually start serving the UE.

NOTE 2:	The source NWDAF might also determine that it needs to prepare to transfer analytics subscriptions to another NWDAF instance, as the source NWDAF wants to resolve an internal load situation or prepare for a graceful shutdown.

NOTE 3:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available.

Figure 6.1B.2.3-1: Prepared analytics transfer

0.	Analytics consumer subscribes to the source NWDAF for certain analytics as specified in clause 6.1.1.

0a.	Source NWDAF starts data collection from relevant data source(s) (e.g. NFs or OAM) as specified in clause 6.2. Source NWDAF starts generating requested analytics.

0b.	[Conditional] (Only if the source NWDAF does not serve the whole PLMN and the requested analytics involves UE related data) The source NWDAF subscribes, using Namf_EventExposure_Subscribe service operation, to receive notifications on UE mobility events from AMF.

1.	The source NWDAF determines that it needs to prepare to transfer analytics to another NWDAF instance.

2.	The source NWDAF discovers candidate target NWDAF instances (e.g. NWDAFy and NWDAFz) supporting the requested analytics information for the predicted target area(s). NWDAF discovery and selection is specified in clause 6.3.13 of TS 23.501 [2]. In the case of aggregated analytics from multiple NWDAFs, the source NWDAF may use the set of NWDAF identifiers related to aggregated analytics (see clause 6.1.3) to preferably select a target NWDAF that is already serving the consumer.

NOTE 4:	In this procedure, NWDAFy and NWDAFz are examples for target NWDAF instances that are candidates to take over those analytic subscriptions.

3-4.	In the case of a prepared analytics subscription transfer, the source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request, to the candidate target NWDAFs (e.g. NWDAFy and NWDAFz) to prepare for an analytics subscription transfer by including a "prepared analytics subscription transfer indication" in the request message. The request message includes information on the analytics subscriptions to be transferred. The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytics ID(s).

The candidate target NWDAFs (e.g. NWDAFy and NWDAFz) respond to the request from the source NWDAF using a Nnwdaf_AnalyticsSubscription_Transfer Response message.

5-6.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 4, the determined target NWDAFs (e.g. NWDAFy and NWDAFz) may request the "analytics context" from the source NWDAF. The analytics context information transfer procedure is specified in clause 6.1B.3.

NOTE 5:	The target NWDAFs (e.g. NWDAFy and NWDAFz) may allocate a new subscription Id to the received analytics subscriptions.

7.	[Optional] Based on the information received from the source NWDAF, the target NWDAFs (e.g. NWDAFy and NWDAFz) start data collection from NFs or OAM (as specified in clause 6.2) and analytics generation for the indicated analytics subscriptions.

NOTE 6:	After step 7, the source NWDAF initiates the analytics subscription transfer to the target NWDAF as specified in steps 4 to 12 of the analytics subscription transfer procedure illustrated in Figure 6.1B.2.2-1.

NOTE 7:	During the prepared analytics subscription transfer procedure, if the source NWDAF determines that it does not need to initiate the analytics subscription transfer by itself, e.g. upon receiving an Nnwdaf_AnalyticsInfo_ContextTransfer request service operation related to the affected analytics subscription from an NWDAF which is selected by the analytics consumer as described in clause 6.1B.2.1 and different from any candidate target NWDAFs selected by the source NWDAF, the source NWDAF cancels the prepared analytics subscription transfer to all the candidate target NWDAFs as described in step 8.

NOTE 8:	During the prepared analytics subscription transfer procedure, if the source NWDAF receives an Nnwdaf_AnalyticsInfo_ContextTransfer request service operation from an NWDAF which is selected by the analytics consumer as described in clause 6.1B.2.1 and is the same as one of the candidate target NWDAFs selected by the source NWDAF, the source NWDAF does not initiate analytics subscription transfer to the target NWDAF by itself, but proceeds as described in steps 3a to 4 of the analytics context transfer procedure illustrated in Figure 6.1B.2.1-1. It further cancels the prepared analytics subscription transfer for the other candidate target NWDAFs as described in step 8.

8.	[Conditional] If a candidate target NWDAF (e.g. NWDAFz) had accepted to prepare for an analytics subscription based on a prepared analytics subscription transfer request received from the source NWDAF in step 5, but had not been selected as the new target NWDAF, the source NWDAF cancels the prepared analytics subscription transfer to this candidate target NWDAF (e.g. NWDAFz), using Nnwdaf_AnalyticsSubscription_Transfer Request include an "analytics subscription transfer cancel indication". The target NWDAF (e.g. NWDAFz) confirms the cancelation to the source NWDAF and, if applicable, deletes any analytics data that is no longer needed. If the target NWDAF (e.g. NWDAFz), as part of the analytics subscription preparation, had already subscribed to entities to collect data or acquire ML model, it unsubscribes to those entities if the subscriptions are not needed for other active analytics subscriptions with the target NWDAF.

******************** End of Change ********************