3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107415r04

Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

# Title:	[DRAFT] Reply LS on network slice admission control for SNPN onboarding

# Response to:	LS (S2-2107028/C1-215159) on network slice admission control for SNPN onboarding from CT1

# Release:	Release 17

# Work Item:	eNPN

Source:	[Huawei to be] SA2

To:	CT1

Cc:

Contact Person:

Name:	zhuqianghua

Tel. Number:

E-mail Address:	zhuqianghua AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 thanks CT1 for the LS (C1-215159) on network slice admission control for SNPN onboarding.

CT1 Question: When the UE is registering or registered for onboarding services in SNPN or PLMN, whether the network slice admission control needs to be performed for slice(s) used for onboarding or not?

SA2 Answer: SA2 assumes that whether network slice admission control is applied or not depends on operator policies. However, SA2 discussed whether the UE can be aware of what service the rejected S-NSSAI relates to as the UE is not providing any Requested NSSAI. Therefore SA2 would like to understand from CT1 whether the UE is aware of what the rejected S-NSSAI relates to (i.e. such that the UE can apply required logic e.g. avoiding requests for the same service as required by the rejection) when the UE does not provide any Requested NSSAI (this applies to UE onboarding and emergency services). If the UE is not aware, then it was discussed whether it is better to make exemptions for such cases.

2. Actions:

To CT1 group.

ACTION: 	SA2 asks the CT1 group to answer the above question.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #148E		November 15 – 19, 2021			Elbonia

TSG-SA2 Meeting #149E   	February 14 – 25, 2022 			Elbonia