3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107441r07

# Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0010 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and Correction on AF and NEF authentication service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In the last meeting, the DNN/S-NSSAI has been added as an input paramater of Nnef_Authentication_Authenticate service operation in the UUAA-SM procedure. Thus, it needs to update the NEF service in clause 4.4.1.1 to cover the DNN/S-NSSAI. Also, UAV location is absent. - As the Nnef/Naf_authentication_notification servie operation is used for revoke, re-authentication, re-authorization or authorization data update, a “notify reason” is needed as an input parameter to indicate the purpose of this notification. - Notification endpoint is not needed as the input of Nnef/Naf_Authentication_notification service operation, as this was provided in Nnef_Authentication_authenticate service operation so that the Nnef knows the Notification endpoint to invoke the Nnef_Authentication_notification service operation. - TS 23.256 specifies in clause 5.2.2.2 (UUAA-MM) at step 5 that USS sends “an indication whether the UAS service releated network resource can be released in case of UUAA failure” with final authentication message to UAS NF. Then this is forwarded to AMF at step 6. But this output parameter is absent in Nnef_Authentication_authenticate_authorize and Naf_Authentication_authenticate service operations |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Addition of DNN,S-NSSAI and UAV location as input of Nnef_Authentication_Authenticate service operation. 2. Addition of Notify reason as input of Nnef/Naf_Authentication_Notification servie operation. 3. Remove notification endpoint from Nnef/Naf_Authentication_Notification service operation. 4. Addition of “an indication whether the UAS service releated network resource can be released in case of UUAA failure” as optional output parameter of Nnef_Authentication_Authenticate service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong NEF/AF implementation and incomplete specification to support UAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.2, 4.4.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.4.1.1.2	Nnef_Authentication service

4.4.1.1.2.1	General

Service Description: This service enables the consumer to either authenticate and authorise, or just authorize, the Service Level Device Identity. In case of UAS, the service is used to authenticate and/or authorize the UAV identified by a CAA-Level UAV ID.

4.4.1.1.2.2	Nnef_Authentication_Authenticate service operation

Service operation name: Nnef_Authentication_AuthenticateAuthorize

Description: Provides the authentication and authorization result of the Service Level device Identity.

Input, Required: Notification endpoint, Service Level Device Identity (i.e. CAA-Level UAV ID), GPSI.

Input, Conditional Required: Combination of DNN/S-NSSAI (in case the consumer NF is SMF)

Input, Optional: Authorization Server Address (i.e. USS Address), PEI, UE IP address (in case the consumer NF is SMF), authentication/authorization container provided by UE,  UAV location.

Output, Required: Success/Failure indication, Authorization Data container, Authentication Session Correlation Id

Output, Conditional Required: Indication whether the PDU sessions associated with the "DNN(s) subject to aerial services" can be released [Required for re-authentication failure].

Output, Optional: None.

4.4.1.1.2.3	Nnef_Authentication_Notification service operation

Service operation name: Nnef_Authentication_Notification

Description: Re-authenticate, update authorization data or revoke the UUAA authorization of a UAV.

Input, Required: Authentication Session Correlation Id, Service Level Device Identity, 3GPP UAV ID, Notify reason (revoke, re-authentication,  or authorization data update).

Input, Conditional Required: cause of revocation (in cased the notify reason is revoke), Authorization Data container (in case the Notify reason is authorization data update).

Input, Optional: None.

Output, Required: Acknowledge indication.

Output, Optional: None.

# * * * * Second change * * * *

4.4.1.2.2	Naf_Authentication service

4.4.1.2.2.1	General

Service Description: This service enables the consumer to authenticate and authorize the Service Level Device Identity. In case of UAS, the service is used to authenticate and authorize the UAV identified by a CAA-Level UAV ID.

4.4.1.2.2.2	Naf_Authentication_Authenticate service operation

Service operation name: Naf_Authentication_AuthenticateAuthorize

Description: Provides the Authentication and Authorization result of the Service Level Device Identity (i.e. CAA-Level UAV ID for UAS).

Input, Required: Notification endpoint, Service Level Device Identity, GPSI.

Input, Optional: PEI, UE IP address, authentication container provided by UE, UAV location.

Output, Required: Success/Failure indication, GPSI, Authorization Data container, Authentication Session Correlation Id.

Output, Conditional Required: Indication whether the UAS service related network resource can be released [Required for re-authentication failure]

Output, Optional: None.

4.4.1.2.2.3	Naf_Authentication_Notification service operation

Service operation name: Naf_Authentication_Notification

Description: Re-authenticate, update authorization data or revoke the UUAA authorization of a UAV.

Input, Required: Authentication Session Correlation Id, Service Level Device Identity, GPSI, Notify reason (revoke, re-authentication, or authorization data update).

Input, Conditional Required: cause of revocation (in case the Notify reason is revoke), Authorization Data container (in case the Notify reason is authorization data update).

Input, Optional: PDU Session IP address

Output, Required: Acknowledge indication.

Output, Optional: None.

# * * * * End of changes * * * *