3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107473r01

# Elbonia, October 18 – 22,  2021	(revision of S2-2100xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3187 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of NSAC and exposure service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon,ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to the LS on NSAC procedure from SA3(S3-213036), SA3 raises a question on whether NSSAA needs to be performed again if the NSSAA is successful but NSACF quota has been reached. It is proposed to clarify that if an S-NSSAI subject to the NSSAA is rejected due to Network Slice Admission Control (e.g., the maximum number of UEs has been reached), the NSSAA result for the S-NSSAI is not impacted. - There are still one EN related to the sub/notify operation for slice status event exposure in case multi NSACF instance. It is proposed to add the related description. Also the NEF is not just transparently forward the message from one side to another side. For example it may need change the related response destination to itself address. It is proposed to change that description. In the Figure 4.15.3.2.10-1, there are two step 5. It need be corrected. - As indicated by C3-214481, Nnsacf_slicestatus service can be totally covered by Nnsacf_ SliceEventExposure service. So the existing Nnsacf_slicestatus and Nnsacf_ SliceEventExposure can be combined into one service. And whether the separated Request/Response service operation is needed or not can be left to stage-3 decide. Thus it is proposed to merge the Nnsacf_ SliceEventExposure into the Nnsacf_slicestatus(as its name more general) service in Stage-2 specs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The NSSAA result for the S-NSSAI is not impacted by the NSAC. - Add the clarification related to the multiple NSACF instance for status event exposure procedure. Change the forwarding to others suitable wording. Update the 2nd step 5 step 6 in the figure. - Rename the Nnsacf_ SliceEventExposure as the Nnsacf_slicestatus service. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is unclear whether NSSAA needs to be performed again if the NSSAA is successful but NSACF quota has been reached. - The event exposure/status retrieval for multiple NSACF instances per S-NSSAI cannot be supported. Misunderstanding that the message from NSACF or AF is transparently transferred to other side. - There would be duplicated service operation for the same purpose. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.9.1, 4.15.3.2.10, 5.2.21.1, 5.2.21.4, 5.2.21.5.1, 5.2.21.5.x, 5.2.21.5.y, 5.2.21.5.z, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

5.2.21	Network Slice Admission Control Function (NSACF) services

5.2.21	NSACF services

5.2.21.1	General

The following table illustrates the NSACF services.

Table 5.2.21.1-1: List of NSACF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NSAC | NumOfUEsUpdate | Request/Response | AMF, SMF (NOTE 1) |
|  | NumOfPDUsUpdate |  | SMF |
|  | EACNotify |  | AMF |
| Nnsacf_SliceEventExposure | Subscribe | Subscribe/Notify | NEF, AF (NOTE 2) |
|  | Unsubscribe |  | NEF, AF |
|  | Notify |  | NEF, AF |
| NOTE: If EPS counting is required for the S-NSSAI, the SMF+PGW-C uses the Nnsacf_NumberOfUEs services operation at PDN connection establishment procedure. NOTE 2: The AF can access NSACF services either via NEF to NSACF in case of untrusted AF or directly in case of trusted AF. |  |  |  |

# * * * * 2nd change * * * *

5.2.21.5	Void

5.2.21.5.1	Void

5.2.21.5.2	Void

# * * * * End of changes * * * *