3GPP TSG-SA2 Meeting # 147E (e-meeting)	S2-2107474r06

# Elbonia, October 18 - 22, 2021													   (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0021 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the local MBS service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, , LG Electronics, Nokia, Nokia Shanghai-Bell, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For location dependent MBS session, the SMF get the MB-SMF profile include all related area session ID and MBS service area it support. Similar the UE also get the whole MBS servcie area for the MBS session based on the service announcment. However for the UE joining procedure only one area session ID and its related MBS service area need be included and sent to NG-RAN node. It is not need to send other Area Session ID and MBS service Area to the NG-RAN. Thus some unnecessary plural description need be removed. - To support the individual delivery, the SMF need subscribe the UE mobility event from AMF. - For the location dependent MBS session, the Area Session ID and MBS service area are also need be registered to NRF. So it is unsuitable to assume if the MBS session ID is TMGI, the registration can be skipped. - For the HOV procedure, it give some further clarificaiton on the location dependent MBS session and limited service area MBS session. It is proposed that target side determine whether the MBS session is kept or removed. - Add new clause about the local MBS session activation and termination of the local MBS session. - For the broadcast MBS service, there are no SMF need be involved. So the NOTE about the MB-SMF need be same for the one SMF service area is unclear on how these two are related. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove unnecessary plural description - Add the UE mobility event subscritption description - Remove the MB-SMF can skip the registration part description. - Add some more detail description of the HOV procedure - Add new clause about the local MBS session activation and termination of the local MBS session. - Remove unclear NOTE description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local MBS for individual delivery will not be supported for the HOV, Activtion, termination procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 6.2.1,6.2.2, 6.2.3, 7.2.4.1, 7.2.4.2.1, 7.2.4.2.2, 7.2.4.2.3, 7.2.4.2.4(new), 7.2.4.2.5(new), 7.2.4.26(new), 7.2.4.3.1, 7.2.4.3.2, 7.2.4.3.3, 7.2.4.3.4(new), 7.2.4.3.5(new), 7.2.4.3.6(new), 7.2.4.Y(new), 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

1st change

3.1	Terms

For the purposes of the present document, the terms and definitions defined in TR 21.905 [1] and the following apply:

5GC Individual MBS traffic delivery: 5G CN receives a single copy of MBS data packets and delivers separate copies of those MBS data packets to individual UEs via per-UE PDU sessions, hence for each such UE one PDU session is required to be associated with a multicast session.

5GC shared MBS traffic delivery: 5G CN receives a single copy of MBS data packets and delivers a single copy of those MBS data packets to a RAN node.

Area Session Identifier: A unique identifier within an MBS Session used for an MBS session with location dependent content. When present the Area Session ID, together with the TMGI, is used to uniquely identify the data flow of an MBS Session in a specific MBS service area.

Associated PDU Session: A PDU Session associated to a multicast session that is used for 5GC Individual MBS traffic delivery method and for signalling related to a user's participation in a multicast session such as join and leave requests.

Associated QoS Flow: A unicast QoS Flow that belongs to the associated PDU Session and is used for 5GC Individual MBS traffic delivery method. The associated QoS Flow is mapped from a multicast QoS Flow in a multicast MBS session.

Broadcast communication service: A 5GS communication service in which the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast coverage area are authorized to receive the data).

NOTE 1:	For the broadcast communication service, the content provider and network may not be aware whether the authorized UEs are actually receiving the data being delivered.

Broadcast MBS session: An MBS session to deliver the broadcast communication service. A broadcast MBS session is characterised by the content to send and the geographical area where to distribute it.

Broadcast service area: The area within which data of one or multiple Broadcast session(s) are sent.

MBS QoS Flow: The finest granularity for QoS forwarding treatment for MBS data. Providing different QoS forwarding treatment requires separate MBS QoS Flows in 5GS supporting MBS.

MBS Service Announcement: Mechanism to allow users to be informed about the available MBS services.

MBS session: A multicast session or a broadcast session.

Multicast communication service: A 5GS communication service in which the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the multicast coverage are authorized to receive the data).

NOTE 2:	For multicast communication service, the content provider and network can be aware whether the authorized UEs are actually receiving the data being delivered.

Multicast MBS session: An MBS session to deliver the multicast communication service. A multicast MBS session is characterised by the content to send, by the list of UEs that may receive the service and optionally by a multicast area where to distribute it.

MBS service area: The area within which data of one Multicast or Broadcast MBS session may be sent. The MBS service area consists of one or several complete tracking areas and/or, if only parts of tracking area(s) are within the MBS service area, one or several cell IDs. For location dependent MBS, the MBS service area is uniquely identified by the combination of Area Session ID and MBS Session ID and corresponds to the location dependent content data of the MBS Session ID.

2nd change

6.2	Local MBS service and Location dependent MBS service

6.2.1	General

A Local MBS service is an MBS service provided in one MBS service area. A location dependent MBS service is an MBS service provided in several MBS service area(s). An MBS service area is identified by a cell list or a tracking area list. The MBS service area could be geographical area information or civic address information, and NEF/MBSF translates the location information to Cell ID list or TAI list as MBS service area, see clause 7.1.1.2.

For multicast communication, local MBS and location dependent MBS services may be supported via 5GC Individual MBS traffic delivery towards RAN nodes not supporting MBS. If the multicast session relates to a Local MBS service or a location dependent MBS service and the SMF configures 5GC Individual MBS traffic delivery, the SMF subscribes at the AMF to notifications about the "UE location" or "UE moving in or out of an "Area Of Interest"(i.e. set by MBS service area)" event.

If the SMF obtains a notification about the UE location, it checks whether the UE is still in the MBS service area of the multicast session. For a local MBS service, if the UE is no longer in the MBS service area, the SMF terminates the 5GC Individual MBS traffic delivery towards the UE. For a location dependent MBS service, if the UE is no longer in the current MBS service area, the SMF determines whether the UE is in another MBS service area of the multicast session; if so, the SMF configures the UPF to send multicast data relating to the new MBS service area towards the UE. If the SMF terminates the 5GC Individual MBS traffic delivery towards the UE or removes the UE from the MBS session context, it unsubscribes at the AMF from the notifications about the UE location or "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

3rd change

6.2.2	Local MBS service

For a local MBS service, only UEs within the MBS service area may receive content data, while UEs outside the MBS service area are not allowed to receive location specific content. For multicast MBS service, UEs outside the MBS service area are not allowed to join the MBS service, and the network shall not deliver location specific content anymore to the UEs moved out of the MBS service area. Depending on policy, for the multicast MBS service the network may remove UEs outside the service area from the MBS session context after a grace period.

For the multicast MBS service, the SMF may subscribe to event notifications to the AMF about a UE leaving the MBS service area.

Editor´note: It is FFS whether, the RAN can defer the related notification for a grace period to avoid excessive notifications about UEs at the service area boundary and whether a new event is thus required.

The UE shall be able to obtain service area information of the local multicast service via MBS service announcement or via NAS signalling (UE Session Join Accept/Reject including Cell ID list or TAI list). If the UE Session Join procedure fails due to the UE being outside the MBS service area, the UE does not attempt to join the multicast session again until the UE moves inside the MBS service area. When the UE Session Join succeeds and if the multicast session is deactivated, the UE does not perform monitoring the session activation notification and any other information related to the multicast session identified by an MBS Session ID over the radio if outside the MBS service area.

NOTE:	Broadcast communication service is the service provided simultaneously to all UEs in a geographical area, therefore for broadcast it is naturally a local MBS service.

4th change

6.2.3	Location dependent MBS service

A location dependent MBS is identified by MBS Session ID, and provided in several MBS service areas. The location dependent MBS service enables distribution of different content data to different MBS service areas. The same MBS Session ID is used but a different Area Session ID is used for each MBS service area. The Area Session ID is used, in combination with MBS Session ID, to uniquely identify the service area specific part of the content data of the MBS service within 5GS. The network supports the location-dependent content distribution for the location dependent MBS services, while UEs are only aware of the MBS Session ID (i.e. UEs are not required to be aware of the Area Session IDs). When UE moves to a new MBS service area, content data from the new MBS service area shall be delivered to the UE, and the network ceases to deliver the content data from the old MBS service areas to the UE.  For multicast MBS service, UEs outside all MBS service area are not allowed to join the MBS service, When UE moves out of an MBS service area and there is no other MBS service area for the MBS session, the network ceases to deliver the content data to the UE. Depending on policy, for the multicast MBS service the network may remove UEs outside all MBS service areas from the MBS session context after a grace period.

The SMF may subscribe to event notifications to the AMF about a UE leaving the combination of all the MBS service areas.

Editor´note: It is FFS whether, the RAN can defer the related notification for a grace period to avoid excessive notifications about UEs at the service area boundary and whether a new event is thus required. It is also ffs whether a notification about UE mobility out of individual service areas is required and what additional information would be required in this notification.

Editor´note: For shared delivery it is FFS whether, the RAN node handles content switching due to mobility between service areas it serves autonomously.

Information about different MBS service areas for a location dependent MBS service may be provided by one or several AFs or may be configured. Different ingress points for location dependent points for the MBS session are supported for different MBS service area dependent content of the MBS session; different MB-SMFs and/or MB-UPF may be assigned for different MBS service areas in an MBS session. When the different MB-SMFs are assigned for different MBS service areas in an MBS session, the same TMGI is allocated for this MBS session.

The Area Session ID is allocated by MB-SMF in MBS Session configuration procedure. MB-SMF allocates Area Session ID for each MBS services area which is unique within the MBS session. MB-SMF needs to further ensure there is no MBS service area overlapping with other MBS service areas that share the same MBS session ID.

NOTE 1:	In this release, deployments topologies with specific SMF Service Areas are not supported, as a result, location dependent service using multicast communication is not supported when a UE moves outside its SMF service area.

NOTE 2:	For location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

NOTE 3:	An example of Location-dependent MBS is a nationwide weather forecast service with local weather reports.

NOTE 4:	Area Session ID is equivalent to Flow ID as specified in TS 23.246 [8].

5th change

7.2.4	Support of Local multicast service

7.2.4.1	General

The procedures for Local multicast service contains the ones for Local multicast service with the location-dependent content, and the ones for limited area local multicast service distribution, as described in clause 6.2.

The UE’s presence in an MBS service area (e.g. whether UE is in or out of an MBS service area, whether UE is located in a different MBS service area), is determined as follows:

- The NG-RAN determines whether a UE’s presence in an MBS service area based on the UE’s current location that is known in NG-RAN and the MBS service area provided by the 5GC.

Editor´note:	It is ffs whether the SMF needs to be informed at all times about the service area where the UE is located for location-dependent services

7.2.4.2	Support of multicast service with location-dependent content

7.2.4.2.1	UE join location dependent multicast session and establishment procedure

The local multicast session join and establishment procedure is performed as defined in clause 7.2.1 with the following additions:

-	The local multicast session is configured as described in clause 7.2.4.2.2.

-	If SMF has no information about the multicast MBS session context for the indicated MBS Session ID, the SMF requests MB-SMF information via Nnrf_NFDiscovery_Request Request (MBS Session ID, UE location), the NRF provides information about the MB-SMF(s) serving the multicast session at the indicated location and service areas and service area IDs for the multicast session, via Nnrf_NFDiscovery_Request Response (MB-SMF profile (Area Session ID(s), MBS service area(s)). The SMF selects the MB-SMF based on the location area where the UE is residing and interacts with MB-SMF to retrieve QoS information of the multicast QoS flow(s) for the MBS Session ID.

-	The SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID(s) and MBS service area(s) that the NG-RAN node belongs to..

Editor´note: It is ffs whether the SMF only provides the service area where the UE is residing, or all service areas served by the RAN node, and how the SMF determines those service areas.

-	If NG-RAN node supports MBS, the NG-RAN node uses the received MBS Session ID and Area Session ID to determine the local multicast session context and whether the user plane for the local multicast session is already established.

-	If the NG-RAN node serving the UE does not support MBS, the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session and service area via unicast delivery within a PDU session of the UE. The SMF subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes, or to notifications about the "UE moving in or out of an "Area Of Interest(i.e. set by MBS service area)"" event.

-	The SMF may subscribe to notifications about the UE leaving the combination of all the MBS service areas.

7.2.4.2.2	Configuration for local MBS

For local MBS, the configuration procedure is performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same multicast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the MBS Session ID, MBS service area and Area Session ID.

NOTE:	For a location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

-	The policy of Multicast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The whole MBS service area(s) for the MBS session are indicated to the UE in the Service Announcement as defined in clause 6.11.

7.2.4.2.3	Handover procedure

The handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

The following bullets apply if the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID before the Handover.

-	For the Xn Handover:

-	The Source RAN node includes MBS Session ID, Area Session ID and MBS service area to the Target RAN node.

-	If the Target RAN node supports MBS, it determines whether to establish the resources for multicast distribution for MBS Session ID and Area Session ID provided by Source RAN node, based on MBS Session ID, Area Session ID and MBS service area.

Editor’s note: Data forwarding issue needs the feedback of RAN WGs

-	If the Target RAN node supports MBS, it respondes to the Source RAN node, with the accepted MBS Session ID, Area Session ID. The Target RAN node perfomes the case when UE is no longer in the MBS service area as described in clause 7.2.4.3.3.

-	If the Target RAN node supports MBS, when it determines the shared delivery is not established for the MBS Session ID and Area Session ID, the Target RAN node initiates the shared delivery establishment as specified in clause 7.2.1.4.

-	The SMF determines MBS service area(s) that the NG-RAN node belongs to by comparing the RAN Node ID and current tracking area IDs of the RAN node received together with the path switch request with the MBS service areas received from the MB-SMF.

-	If the Target NG-RAN supports MBS, the SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID(s) and MBS service area(s) that the NG-RAN node belongs to.

-	If the Target RAN node does not support MBS the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session and service area via unicast delivery within a PDU session of the UE. The SMF subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes, or to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. In the later case the SMF supplies the service area of the multicast session as Area Of Interest.

-	For the N2 Handover:

-	The SMF includes MBS Session ID, Area Session ID, and MBS service area to the Target RAN node in Handover Request message.

-	If the Target RAN node supports MBS, it determines whether to establish the resources for multicast distribution for MBS Session ID and Area Session ID provided by SMF, based on MBS Session ID, Area Session ID, MBS service area and UE location. The Target RAN node performs the case when UE is no longer in the MBS service area as described in clause 7.2.4.3.3.

-	If the Target RAN node supports MBS, when it determines the shared delivery is not established for the MBS Session ID and Area Session ID, the Target RAN node initiates the shared delivery establishment as specified in clause 7.2.1.4.

- -	The SMF determines MBS service area(s) that the NG-RAN node belongs to by comparing the RAN Node ID and current tracking area IDs of the RAN node received in message 17 with the MBS service areas received from the MB-SMF.

- -	If the Target NG-RAN supports MBS, the SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID(s) and MBS service area(s) that the NG-RAN node belongs to.

- -	If the Target RAN node does not support MBS the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session and service area via unicast delivery within a PDU session of the UE. The SMF subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes, or to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. In the later case the SMF supplies the service area of the multicast session as Area Of Interest.

The following bullets apply if the UE is camping at Source RAN not supporting MBS before the Handover:

-	During handover the SMF serving the UE discovers via signaling from the RAN (via AMF) whether the target RAN node serving the UE supports MBS.

-	If the target RAN node serving the UE after a handover supports MBS the following applies:

-	If Multicast data for the multicast session are delivered to the UE via individual delivery the SMF configures the UPF to stop sending data related to the multicast session via unicast delivery within a PDU session of the UE

.	The SMF unsubscribes at the AMF from the notifications about the "UE location" or the "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

##### 7.2.4.2.4	Activation of location dependent MBS session

When the location dependent MBS session is activated or deactivated, the MBS session is activated or deactivated in the whole MBS service area of the MBS session. It is not supported that the same MBS session is in active state in one MBS service area but in inactive state in another MBS service area.

For the location dependent MBS session activation, the differences comparing to the procedure defined in clause 7.2.5.2 are as below.

- The SMF invokes Namf_MT_EnableGroupReachability service operation to AMF, which includes the whole MBS service area associated with the MBS session, i.e. the sum of all MBS service area associated with the MBS session ID regardless of the Area session ID.

- For the UE in CM-IDLE state, when the AMF triggers the paging, it takes the receiving MBS service area information into account. Only the NG-RAN node which is included in the MBS service area need trigger the paging.

- SMF checks whether the UE is in or out of the MBS service area based on the change notification or UE location information included in Nsmf_PDUSession_UpdateSMContext Request. Based on that information, the SMF determines how to activate the MBS session same as the handling after the handover procedure defined in clause 7.2.4.2.3

- When the SMF provides the MBS session information to the NG-RAN nodes, it includes the MBS service area, and Area Session ID, in the N2 SM information.

##### 7.2.4.2.5	SMF removing joined UEs from location dependent MBS session

Besides the condition as defined in clause 7.2.2.3, per operator's policy (e.g. when a local configured timer expires since the UE left the whole MBS service area) the SMF may remove the UE from the location dependent MBS session if the UE is out of the whole service area of the MBS session. When the UE is removed from the location dependent MBS session, the SMF also unsubscribes to the AMF from the notifications about the "UE moving in or out of a subscribed Area Of Interest" event.

7.2.4.2.6	UE moving between service areas during Individual delivery

Figure 7.2.4.2.6-1:	UE moving between service areas during Individual delivery for location-dependent multicast session

1.	The UE moves to another cell, either served by the same RAN node (intra-RAN node mobility) or by a different RAN node (handover).

2.	If the SMF subscribed to location changes, it obtains a notification about every cell change. If the notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event, it obtains notification when the UE enters or leaves the area of interest. If the SMF obtains a UE location change notification, it checks whether the new UE location is inside or outside the old service area of the MBS session where the UE was previously located (if any).

3.	If UE left a service area of the multicast session, the SMF decides to terminate individual delivery of MBS data for that old service area to UE. As detailed in the subsequent steps, the SMF also checks if UE is in another service area of multicast session. If so, the SMF configures individual delivers of MBS data for the new service area to UE. Otherwise the SMF may remove the UE from the MBS session

Steps 4 to 6 apply when the UE leaves the previous service area of the multicast session.

4.	The SMF configures the UPF to stop sending data related to the multicast session and service area via unicast delivery within a PDU session of the UE.

5.	If the UPF indicates back to SMF that it does not require data related to the multicast session and service area for other UEs, the SMF requests from the MB-SMF that data related to the multicast session and service area are no longer delivered to the UPF.

6.	If step 5 occurs, the MB-SMF configures the MB-UPF accordingly.

7.	If the SMF subscribed at the AMF to notifications related to the area of interest event and was only informed that the UE left the area of interest in step 2, the SMF may inquire the UE location from the AMF using the Namf_EventExposure service.

8.	If the SMF has no information about a service area for the multicast service for the new UE location, it may inquire a service area for the multicast service for the new UE location at the NRF,

Steps 9 to 12 apply when the UE enters a new service area of the multicast session.

9.	If the SMF subscribed at the AMF to notifications related to the area of interest event, the SMF updates the subscription by supplying the new service area as Area Of Interest.

10.	The SMF configures the UPF to send data related to the multicast session and service area via unicast delivery within a PDU session of the UE.

11.	If the UPF indicates back to SMF that it does not yet receive data related to the multicast session and service area, the SMF requests from the MB-SMF that data related to the multicast session and service area are delivered to the UPF.

12.	If step 11 occurs, the MB-SMF configures the MB-UPF accordingly.

If the UE is in no service area of the multicast session, the SMF may decide based on operator policy to remove the UE from the multicast session. Steps 13 and 14 then apply.

13.	the SMF updates the PDU session to inform the UE that it was removed from the multicast session (via a N1 container sent to the RAN node and then forwarded to the UE) and to remove information related to the multicast session from the PDU session (e.g. QoS flows to deliver multicast data, via N2 contained to be processed by RAN node)

14.	The SMF unsubscribes at the AMF from the notifications about the UE location or the "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

15.	If the UE is in no service area of the multicast session, and the SMF previously subscribed at the AMF to notifications related to the area of interest event, it updates the subscription to subscribe instead to notifications about the UE location.

# * * * * 3rd change * * * *

7.2.4.3	Support of multicast service available within a limited area

7.2.4.3.1	Local MBS service area information provided by AF

For local multicast services, the MBS service area information is provided to the UE and the 5GC as specified in clauses 7.1.1.2 and 7.1.1.3 with the following differences and clarifications:

-	For the Service Announcement, MBS service area information is included unless it is preconfigured.

-	When performing the MBS session request to the 5GC, the MBS service area information for a multicast session is provided by the AF unless it is preconfigured.

7.2.4.3.2	Multicast session join and session establishment procedure for multicast service available within a limited area

For the case that the multicast service is only available within a limited area, the UEs outside the MBS service area are not allowed to join the multicast service.

The multicast session join and session establishment for multicast service available within a limited area are performed as specified in clause 7.2.1.3 with the following differences and clarifications:

-	The local multicast session has been configured where the AF provided the MBS service area information as specified in clause 7.2.4.3.1.

-	The UE may have information about local multicast service including MBS service area and local MBS service indication via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the MBS service area based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside the MBS service area, the UE does not send the Join Request. The AMF provides the cell ID and current tracking area ID of the UE towards the SMF.

NOTE 1:	The AMF includes this information in all Nsmf_PDUSession_UpdateSMContext request if the DNN and S-NSSAI allow multicast.

-	In step 4, the SMF obtains the MBS service area (i.e. Cell ID list or TAI list) of the indicated MBS session from the MB-SMF, and the SMF determines the indicated MBS session corresponds to local multicast service based on the MBS service area.

-	In step 5, the SMF checks whether the UE is inside or outside the MBS service area.

-	The SMF determines whether the UE is inside the MBS service area by comparing the cell ID and current tracking area ID of the UE received together with the join request with the MBS service area received from the MB-SMF.

-	If the SMF detects that the UE is inside the MBS service area, the SMF continues the process of multicast session join as specified in clause 7.2.1.3 with the following additions:

-	The SMF sends the UE a PDU Session Modification Command indicating a Join Accept as a response to the Join Request. The Joint Accept includes the MBS service area (i.e. Cell ID list or TAI list).

-	The multicast session information sent by the SMF to the NG-RAN includes the MBS service area (i.e. Cell ID list or TAI list).

-	If the RAN node serving the UE does not support MBS the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session via unicast delivery within a PDU session of the UE. The SMF subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes, or to notifications about the "UE moving in or out of an "Area Of Interest(i.e. set by MBS service area)"" event.

-	If the SMF detects that the UE is outside the MBS service area, the SMF rejects the multicast session join, so sends a Join Reject to the UE indicating that the MBS join is rejected. The Joint Reject includes the reject reason (outside of local service area) and the MBS service area (i.e. Cell ID list or TAI list).

In this case, the MBS Session establishment (i.e. resources establishment for MBS traffic delivery) for the UE is not performed.

NOTE 2:	There may be the case that the UE determines that it is inside the MBS service area based on the geographical area information or civic address information as MBS service area information provided by the AF, so sends a Join Request but the UE is outside the MBS service area.

NOTE 3:	Which SM NAS message is used to deliver the Join Reject (e.g. PDU Session Modification Reject) is defined in stage 3 specifications.

-	If the Join Request from the UE is accepted and 5GC individual MBS traffic delivery is used, the SMF subscribes to the UE mobility event notification from the AMF (e.g. UE moving into or out of Area Of Interest, which is set by MBS service area), by invoking Namf_EventExposure_Subscribe service operation as specified in clause 5.2.2.3.2 of TS 23.502 [6].

-	For the UE that received the Join Reject from the SMF, later, if the UE detects that it is inside the MBS service area based on the MBS service area provided in the Join Reject, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group.

-	The SMF may subscribe to notifications about the UE leaving the service area.

##### 7.2.4.3.3	Handover procedure with limited area MBS session

The handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

For Xn handover, the The AMF provides the cell ID and current tracking area ID of the UE towards the SMF in message 1 in the handover preparation phase. ForN2 handover, the AMF provides the cell ID and current tracking area in message 17

NOTE 1:	The AMF includes this information in all Nsmf_PDUSession_UpdateSMContext request if the DNN and S-NSSAI allow multicast.

The following bullets apply if the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID before the Handover.

-	For Xn based handover in clause 7.2.3.2, the Source RAN node includes MBS Session ID and MBS service area to the Target RAN node during Handover Preparation phase. For N2 based handover in clause 7.2.3.3, this step corresponds to Handover Request message and Handover Required message, respectively.

-	When Target RAN supports MBS but  the UE is no longer in the MBS service area, the Target RAN node does not allocate RAN resources for the multicast MBS Session to the UE.

-	For N2 handover, if the UE is handed over to a target cell outside to the MBS service area, the SMF does not provide the MBS session related information in N2 SM Info to the target RAN.

-

-	The SMF determines whether the UE is inside the MBS service area by comparing the cell ID and current tracking area ID of the UE received together in the path switch request (for Xn based handover) or message 17 (for N2 based handover)  with the MBS service area received from the MB-SMF.

-	If the RAN node serving the UE does not support MBS the SMF applies individual delivery towards the UE. If the UE is inside the MBS service area, the SMF configures the UPF to send data related to the multicast session via unicast delivery within a PDU session of the UE. The SMF subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes, or to notifications about the "UE moving in or out of a "Area Of Interest(i.e. set by MBS session ID)"" event.

-	If the UE is outside the MBS service area, per operator’s policy the SMF may remove the UE from the multicast MBS session as defined in clause 7.2.2.3, possibly after a grace period.

The following bullets apply if the UE is camping at Source RAN not supporting MBS before the Handover:

-	During handover the SMF serving the UE discovers via signaling from the RAN (via AMF) whether the target RAN node serving the UE supports MBS.

-	The SMF determines whether the UE is inside the MBS service area by comparing the cell ID and current tracking area ID of the UE received together in the path switch request (for Xn based handover) or message 17 (for N2 based handover)  with the MBS service area received from the MB-SMF.

-	If the UE is outside the MBS service area, per operator’s policy the SMF may remove the UE from the multicast MBS session as defined in clause 7.2.2.3, possibly after a grace period.

-	If the target RAN node serving the UE after a handover supports MBS the following applies:

-	If Multicast data for the multicast session are delivered to the UE via individual delivery the SMF configures the UPF to stop sending data related to the multicast session and service area via unicast delivery within a PDU session of the UE

.	The SMF unsubscribes at the AMF from the notifications about the "UE location" or the "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

-

For the N2 Handover, t

he SMF includes MBS Session ID and MBS service area to the Target RAN node in Handover Request message.

-	For the Xn Handover, the SMF includes MBS Session ID and MBS service area to the Target RAN node in the Nsmf_PDUSession_UpdazteSmContext Response (message 11).

-	The Target RAN determines whether to establish the resources for multicast distribution for MBS Session ID provided by SMF, based on MBS Session ID, MBS service area and UE location. If the UE is no longer in the MBS service area provided by the SMF, the Target RAN node does not allocate RAN resources for the multicast MBS Session to the UE.

-	If the target RAN node serving the UE after a handover does not supports MBS the following applies:

- If the UE is inside of the service area of the MBS session, the SMF provides associated QoS flows.

- If the UE is ouside of the service area of the MBS session,

- the SMF does not provide associated QoS flows.

##### 7.2.4.3.4	Activation of limited area MBS session

For the limited area MBS session activation, the differences comparing to the procedure defined in clause 7.2.5.2 are as below.

- The SMF invokes Namf_MT_EnableGroupReachability service operation to AMF, which include the MBS service area associated with the MBS session.

- For the UE in CM-IDLE state, when the AMF triggers the paging, it takes the receiving MBS service area information into account. Only the NG-RAN node which is included in the MBS service area need trigger the paging.

- SMF checks whether the UE is in or out of the MBS service area based on the change notification or UE location information included in Nsmf_PDUSession_UpdateSMContext Request.Based on that information, the SMF determines how to activate the MBS session same as the handling after the handover procedure defined in clause 7.2.4.3.3.

- When the SMF provides the MBS session information to the NG-RAN nodes, it includes the MBS service area in the N2 SM information.

##### 7.2.4.3.5	SMF removing joined UEs from limited area MBS session

Besides the condition as defined in clause 7.2.2.3, per operator's policy (e.g. when a local configured timer expires since the UE left the MBS service area) the SMF may remove the UE from the limited areaMBS session if the UE is out of the service area of the MBS session. When the UE is removed from the limited area MBS session, the SMF also unsubscribes to the AMF from the notifications about the "UE moving in or out of a subscribed Area Of Interest" event.

7.2.4.3.6	UE moving in or out of service areas during Individual delivery

Figure 7.2.4.3.6-1:	UE moving in or out of service area during Individual delivery for local multicast session

1.	The UE moves to another cell, either served by the same RAN node (intra-RAN node mobility) or by a different RAN node (handover).

2.	If the SMF subscribed to location changes, it obtains a notification about every cell change. If the notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event, it obtains notification when the UE enters or leaves the area of interest. If the SMF obtains a UE location change notification, it checks whether the new UE location is inside or outside the service area of the MBS session.

Steps 3 to 8 apply when the UE leaves the service area of the multicast session.

3.	The SMF decides to stop individual delivery towards the UE, i.e. the SMF decides to stop sending data related to the multicast session via unicast delivery within a PDU session of the UE.

4.	The SMF configures the UPF to stop sending data related to the multicast session via unicast delivery within a PDU session of the UE.

5.	If the UPF indicates back to SMF that it does not require data related to the multicast session for other UEs, the SMF requests from the MB-SMF that data related to the multicast session are no longer delivered to the UPF.

6.	If step 5 occurs, the MB-SMF configures the MB-UPF accordingly.

The SMF may decide based on operatot policy to remove the UE from the multicast session. Steps 7 and 8 then apply.

7.	The SMF updates the PDU session to inform the UE that it was removed from the multicast session (via a N1 container sent to the NG-RAN node and then forwarded to the UE) and to remove information related to the multicast session from the PDU session (e.g. QoS flows to deliver multicast data, via N2 contained to be processed by RAN node)

8.	The SMF unsubscribes at the AMF from the notifications about the UE location or "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

Steps 9 to 11 apply when the UE enters the service area of the multicast session.

9.	The SMF decides to apply individual delivery towards the UE, i.e. the SMF decides to send data related to the multicast session via unicast delivery within a PDU session of the UE.

10.	The SMF configures the UPF to send data related to the multicast session via unicast delivery within a PDU session of the UE.

11.	If the UPF indicates back to SMF that it does not yet receive data related to the multicast session, the SMF requests from the MB-SMF that data related to the multicast session are delivered to the UPF.

12.	If step 11 occurs, the MB-SMF configures the MB-UPF accordingly.

# * * * * 4rd change * * * *

7.2.4.Y	Idle/inactive mobility procedure

If a UE that has joined into a multicast MBS session moves into the MBS service area from outside:

-	If the UE is in RRC-IDLE state, the UE initiates Service Request procedure to activate the associated PDU Session.

-	If the UE is in CM-CONNECTED with RRC-INACTIVE state, when the associated PDU Session is activated, the UE resumes RRC connection with the NG-RAN, when the associated PDU Session is inactive, the UE initiates Service Request procedure to activate the associated PDU Session.

-	If the MBS service area has been changed and has not been provided to the UE, the SMF performs MBS Session Update for the MBS service area towards the UE.

-	For the case UE comes back to the MBS service area of the multicast MBS session after out of the MBS service area of the multicast MBS session before,

-	For location-dependent content, if the multicast MBS session is in the "Active" state, the SMF try to activate the multicast MBS session towards the UE per UE’s current location.

-	For a limited area content, if the multicast MBS session is in the "Active" state, the SMF try to activate the multicast MBS session towards the UE.

# * * * * 5th change * * * *

7.3.4	Support for Local Broadcast Service

The procedures for Local broadcast service contains the ones for local broadcast service with the location-dependent content as described in clause 6.2.

The configuration procedure is performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same broadcast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the TMGI and Area Session ID.

-	The policy of broadcast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The MBS service area(s) are indicated to the UE in the Service Announcement as defined in clause 6.11.

The MBS session establishment procedure is performed as defined in clause 7.3.1 with the following additions:

-	MB-SMF requests the AMF to transfer an N2 message (i.e. MBS Session Resource Setup Request) to the NG-RAN nodes of the MBS service area with broadcast session information which additionally includes the Area Session ID and MBS service area.

-	The NG-RAN uses the received MBS Session ID and Area Session ID to determine the local broadcast session context.

# * * * * End of change * * * *