3GPP TSG-SA2 Meeting # 147E (e-meeting)	S2-2107474r06

# Elbonia, October 18 - 22, 2021													   (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0021 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the local MBS service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, , LG Electronics, Nokia, Nokia Shanghai-Bell, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For location dependent MBS session, the SMF get the MB-SMF profile include all related area session ID and MBS service area it support. Similar the UE also get the whole MBS servcie area for the MBS session based on the service announcment. However for the UE joining procedure only one area session ID and its related MBS service area need be included and sent to NG-RAN node. It is not need to send other Area Session ID and MBS service Area to the NG-RAN. Thus some unnecessary plural description need be removed. - To support the individual delivery, the SMF need subscribe the UE mobility event from AMF. - For the location dependent MBS session, the Area Session ID and MBS service area are also need be registered to NRF. So it is unsuitable to assume if the MBS session ID is TMGI, the registration can be skipped. - Add new clause about the local MBS session activation and termination of the local MBS session. - For the broadcast MBS service, there are no SMF need be involved. So the NOTE about the MB-SMF need be same for the one SMF service area is unclear on how these two are related. Clause 6.2 (for local MBS service and location dependent service) is the high-level feature description. However, some description is quite procedure oriented, e.g. how to determine a joined UE is IN or OUT of an MBS service area, what service operations are more for procedures, how the NG-RAN behave. It’s proposed to move the details to the procedures. How to determine UE’s presence in an MBS service area is common for multiple procedures related to multicast MBS, and it is proposed to introduce a new sub-clause to describe the system behavior. In clause 7.2.4.2.2 (see below), the following text “for the UE” is incorrect and it is proposed to say “…procedure is performed as defined….” - For local MBS, the configuration procedure for the UE is optional and performed as defined in clause 7.1.1.2 with the following additions |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove unnecessary plural description - Add the UE mobility event subscritption description - Remove the MB-SMF can skip the registration part description. - Add new clause about the local MBS session activation and termination of the local MBS session. - Remove unclear NOTE description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local MBS for individual delivery will not be supported for the Activtion, procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.2.3, 7.2.4.1, 7.2.4.2.1, 7.2.4.2.2, 7.2.4.3.1, 7.2.4.3.2, , 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

3rd change

6.2.2	Local MBS service

For a local MBS service, only UEs within the MBS service area may receive content data, while UEs outside the MBS service area are not allowed to receive location specific content. For multicast MBS service, UEs outside the MBS service area are not allowed to join the MBS service, and the network shall not deliver location specific content anymore to the UEs moved out of the MBS service area. Depending on policy, for the multicast MBS service the network may remove UEs outside the MBS service area of the MBS session from the MBS session context after a grace period (e.g. by using a local configured timer).

The UE shall be able to obtain service area information of the local multicast service via MBS service announcement or via NAS signalling (UE Session Join Accept/Reject including Cell ID list or TAI list). If the UE Session Join procedure fails due to the UE being outside the MBS service area, the UE does not attempt to join the multicast session again until the UE moves inside the MBS service area. When the UE Session Join succeeds and if the multicast session is deactivated, the UE does not perform monitoring the session activation notification and any other information related to the multicast session identified by an MBS Session ID over the radio if outside the MBS service area.

NOTE:	Broadcast communication service is the service provided simultaneously to all UEs in a geographical area, therefore for broadcast it is naturally a local MBS service.

For more details, refer to clause 7.2.4 for multicast MBS Session and refer to clause 7.3.4 for broadcast MBS Session.

4th change

6.2.3	Location dependent MBS service

A location dependent MBS is identified by MBS Session ID, and provided in several MBS service areas. The location dependent MBS service enables distribution of different content data to different MBS service areas. The same MBS Session ID is used but a different Area Session ID is used for each MBS service area. The Area Session ID is used, in combination with MBS Session ID, to uniquely identify the service area specific part of the content data of the MBS service within 5GS. The network supports the location-dependent content distribution for the location dependent MBS services, while UEs are only aware of the MBS Session ID (i.e. UEs are not required to be aware of the Area Session IDs). When UE moves to a new MBS service area, content data from the new MBS service area shall be delivered to the UE, and the network ceases to deliver the content data from the old MBS service areas to the UE. For multicast MBS service, UEs outside all MBS service area of the MBS Session are not allowed to join the MBS service, When UE moves out of an MBS service area and there is no other MBS service area for the MBS session, the network ceases to deliver the content data to the UE. Depending on policy, for the multicast MBS service the network may remove UEs outside all MBS service areas of the MBS Session from the multicast MBS session context after a grace period (e.g. by using a local configured timer).

Information about different MBS service areas for a location dependent MBS service may be provided by one or several AFs or may be configured. Different ingress points for location dependent points for the MBS session are supported for different MBS service area dependent content of the MBS session; different MB-SMFs and/or MB-UPF may be assigned for different MBS service areas in an MBS session. When the different MB-SMFs are assigned for different MBS service areas in an MBS session, the same TMGI is allocated for this MBS session.

The Area Session ID is allocated by MB-SMF in MBS Session configuration procedure. MB-SMF allocates Area Session ID for each MBS services area which is unique within the MBS session. MB-SMF needs to further ensure there is no MBS service area overlapping with other MBS service areas that share the same MBS session ID.

NOTE 1:	In this release, deployments topologies with specific SMF Service Areas are not supported, as a result, location dependent service using multicast communication is not supported when a UE moves outside its SMF service area.

NOTE 2:	For location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

NOTE 3:	An example of Location-dependent MBS is a nationwide weather forecast service with local weather reports.

NOTE 4:	Area Session ID is equivalent to Flow ID as specified in TS 23.246 [8].

For more details, refer to clause 7.2.4 for multicast MBS Session and refer to clause 7.3.4 for broadcast MBS Session.

5th change

7.2.4	Support of Local multicast service and Location dependent multicast service

7.2.4.1	General

The clause captures the procedural enhancement to supportLocation-dependent MBS service and the Local MBS service described in clause 6.2.

Editor’s Note: Procedures for the UE mobility between cells of one RAN node, e.g. when the UE leaves, enters or switches service areas due to mobility are ffs. For shared delivery it is FFS whether, the RAN node handles content switching due to mobility between service areas it serves autonomously or the core network needs notifications about this mobility to select new service areas. For the UE leaving a service area it is ffs whether the RAN node defers related notification for a grace period or the SMF defers removing UEs from the multicast session. Details require coordination with RAN

7.2.4.2	Support of location-dependent multicast service

7.2.4.2.1	UE join location dependent multicast session and establishment procedure

The local multicast session join and establishment procedure is performed as defined in clause 7.2.1 with the following additions:

-	The local multicast session is configured as described in clause 7.2.4.2.2.

-	If SMF has no information about the multicast MBS session context for the indicated MBS Session ID, the SMF requests MB-SMF information via Nnrf_NFDiscovery_Request Request (MBS Session ID, UE location), the NRF provides information about the MB-SMF(s) serving the multicast session at the indicated location and service areas and service area IDs for the multicast session, via Nnrf_NFDiscovery_Request Response (MB-SMF profile (Area Session ID(s), MBS service area(s)). The SMF selects the MB-SMF based on the location area where the UE is residing and interacts with MB-SMF to retrieve QoS information of the multicast QoS flow(s) for the MBS Session ID.

-	The SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID(s) and MBS service area(s) that the NG-RAN node belongs to..

Editor´note: It is ffs whether the SMF only provides the service area where the UE is residing, or all service areas served by the RAN node, and how the SMF determines those service areas for the later case.

-	If NG-RAN node supports MBS, the NG-RAN uses the received MBS Session ID(s) and Area Session ID(s) to determine the local multicast session context and whether the user plane for the local multicast session is already established.

-	If the NG-RAN node serving the UE does not support MBS, the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session and service area via individual delivery within a PDU session of the UE. The SMF subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes, or to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. In the later case the SMF supplies the service area of the multicast session as Area Of Interest.

7.2.4.2.2	Configuration for local MBS

For local MBS, the configuration procedure is performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same multicast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the MBS Session ID, MBS service area and Area Session ID.

NOTE:	For a location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

-	The policy of Multicast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The whole MBS service area(s) for the MBS session are indicated to the UE in the Service Announcement as defined in clause 6.11.

# * * * * Next  change * * * *

7.2.4.2.4	Activation of location dependent MBS session

When the location dependent MBS session is activated, the MBS session is activated in the whole MBS service area of the MBS session. It is not supported that the same MBS session is in active state in one MBS service area but in inactive state in another MBS service area.

For the location dependent MBS session activation, the differences comparing to the procedure defined in clause 7.2.5.2 are as below.

- The SMF invokes Namf_MT_EnableGroupReachability service operation to AMF, which includes the whole MBS service area associated with the MBS session, i.e. the sum of all MBS service area associated with the MBS session ID regardless of the Area session ID.

- For the UE in CM-IDLE state, when the AMF triggers the paging, it take the receiving MBS service area information into account. Only the NG-RAN node which is included in the MBS service area need trigger the paging.

- SMF checks whether the UE is in or out of the MBS service area based on the change notification or UE location information included in Nsmf_PDUSession_UpdateSMContext Request. Based on that information, the SMF determines how to activate the MBS session same as the handling after the handover procedure defined in clause 7.2.4.2.3

- When the SMF provides the MBS session information to the NG-RAN nodes, it includes the MBS service area, and Area Session ID, in the N2 SM information.

# * * * * Next change * * * *

7.2.4.3	Support of multicast service available within a limited area

7.2.4.3.1	Local MBS service area information provided by AF

For local multicast services, the MBS service area information is provided to the UE and the 5GC as specified in clauses 7.1.1.2 and 7.1.1.3 with the following differences and clarifications:

-	For the Service Announcement, MBS service area information is included unless it is preconfigured.

-	When performing the MBS session request to the 5GC, the MBS service area information for a multicast session is provided by the AF unless it is preconfigured.

7.2.4.3.2	Multicast session join and session establishment procedure for multicast service available within a limited area

For the case that the multicast service is only available within a limited area, the UEs outside the MBS service area are not allowed to join the multicast service.

The multicast session join and session establishment for multicast service available within a limited area are performed as specified in clause 7.2.1.3 with the following differences and clarifications:

-	The local multicast session has been configured where the AF provided the MBS service area information as specified in clause 7.2.4.3.1.

-	The UE may have information about local multicast service including MBS service area and local MBS service indication via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the MBS service area based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside the MBS service area, the UE does not send the Join Request. The AMF provides the User Location Information of the UE towards the SMF.

NOTE 1:	The AMF includes this information in all Nsmf_PDUSession_UpdateSMContext request if the DNN and S-NSSAI allow multicast.

-	In step 4, the SMF obtains the MBS service area (i.e. Cell ID list or TAI list) of the indicated MBS session from the MB-SMF, and the SMF determines the indicated MBS session corresponds to local multicast service based on the MBS service area.

-	In step 5, the SMF checks whether the UE is inside or outside the MBS service area.

-	The SMF determines whether the UE is inside the MBS service area by comparing the User Location Information of the UE received together with the join request with the MBS service area received from the MB-SMF.

-	If the SMF detects that the UE is inside the MBS service area, the SMF continues the process of multicast session join as specified in clause 7.2.1.3 with the following additions:

-	The SMF sends the UE a PDU Session Modification Command indicating a Join Accept as a response to the Join Request. The Joint Accept includes the MBS service area (i.e. Cell ID list or TAI list).

-	The multicast session information sent by the SMF to the NG-RAN includes the MBS service area (i.e. Cell ID list or TAI list).

-	If the RAN node serving the UE does not support MBS the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session via individual delivery within a PDU session of the UE. The SMF subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes, or to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. In the later case the SMF supplies the service area of the multicast session as Area Of Interest.

-	If the SMF detects that the UE is outside the MBS service area, the SMF rejects the multicast session join, so sends a Join Reject to the UE indicating that the MBS join is rejected. The Joint Reject includes the reject reason (outside of local service area) and the MBS service area (i.e. Cell ID list or TAI list).

In this case, the MBS Session establishment (i.e. resources establishment for MBS traffic delivery) for the UE is not performed.

NOTE 2:	There may be the case that the UE determines that it is inside the MBS service area based on the geographical area information or civic address information as MBS service area information provided by the AF, so sends a Join Request but the UE is outside the MBS service area.

NOTE 3:	Which SM NAS message is used to deliver the Join Reject (e.g. PDU Session Modification Reject) is defined in stage 3 specifications.

-	If the Join Request from the UE is accepted and 5GC individual MBS traffic delivery is used, the SMF subscribes to the UE mobility event notification from the AMF (e.g. UE moving into or out of Area Of Interest, which is set by MBS service area), by invoking Namf_EventExposure_Subscribe service operation as specified in clause 5.2.2.3.2 of TS 23.502 [6].

-	For the UE that received the Join Reject from the SMF, later, if the UE detects that it is inside the MBS service area based on the MBS service area provided in the Join Reject, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group.

# * * * * Next change * * * *

##### 7.2.4.3.4	Activation of limited area MBS session

For the limited area MBS session activation, the differences comparing to the procedure defined in clause 7.2.5.2 are as below.

- The SMF invokes Namf_MT_EnableGroupReachability service operation to AMF, which include the MBS service area associated with the MBS session.

- For the UE in CM-IDLE state, when the AMF triggers the paging, it take the receiving MBS service area information into account. Only the NG-RAN node which is included in the MBS service area need trigger the paging.

- SMF checks whether the UE is in or out of the MBS service area based on the change notification or UE location information included in Nsmf_PDUSession_UpdateSMContext Request.Based on that information, the SMF determines how to activate the MBS session same as the handling after the handover procedure defined in clause 7.2.4.3.3.

- When the SMF provides the MBS session information to the NG-RAN nodes, it includes the MBS service area in the N2 SM information.

# * * * * 5th change * * * *

7.3.4	Support for Local Broadcast Service

The procedures for Local broadcast service contains the ones for local broadcast service with the location-dependent content as described in clause 6.2.

The configuration procedure is performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same broadcast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the TMGI and Area Session ID.

-	The policy of broadcast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The MBS service area(s) are indicated to the UE in the Service Announcement as defined in clause 6.11.

The MBS session establishment procedure is performed as defined in clause 7.3.1 with the following additions:

-	MB-SMF requests the AMF to transfer an N2 message (i.e. MBS Session Resource Setup Request) to the NG-RAN nodes of the MBS service area with broadcast session information which additionally includes the Area Session ID and MBS service area.

-	The NG-RAN uses the received MBS Session ID and Area Session ID to determine the local broadcast session context.

# * * * * End of change * * * *