3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107595r01

# Elbonia, October 18 – 22, 2021	(revision of S2- 210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3320 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous correction for eNA_Ph2 on TS 23.501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Huawei, HiSilicon, Nokia, LG Electronics, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Based on the agreements in S2-2106645 (SA2#146E), the Analytics Filter information for ML model provisioning had been renamed as “ML model Filter information” in TS 23.288. The same modification should also be applied to TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Rename the Analytics Filter information for the ML model provisioning as “ML model Filter information” for alignment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The terminology is not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.13, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.3.13	NWDAF discovery and selection

Multiple instances of NWDAF may be deployed in a network.

The NF consumers shall utilize the NRF to discover NWDAF instance(s) unless NWDAF information is available by other means, e.g. locally configured on NF consumers. NF consumers may make an additional query to UDM, when supported, as detailed below. The NWDAF selection function in NF consumers selects an NWDAF instance based on the available NWDAF instances.

The NRF may return one or more candidate NWDAF instance(s) and each candidate NWDAF instance (based on its registered profile) supports the Analytics ID with a time that is less than or equal to the Supported Analytics Delay.

The following factors may be considered by the NF consumer for NWDAF selection:

-	S-NSSAI.

-	Analytics ID(s).

-	Supported service(s), possibly with their associated Analytics IDs.

-	NWDAF Serving Area information, i.e. list of TAIs for which the NWDAF can provide analytics, trained ML models and/or data.

NOTE 1:	If all services provided by one NWDAF do not support the same Analytics ID, the NWDAF registers the Analytics IDs of the services at the service level.

NOTE 2:	Analytics ID(s) at service level take precedence over Analytics ID(s) at NF level.

NOTE 3:	For discovery of NWDAF supporting Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo services, the Analytics IDs at the NWDAF NF profile are used.

-	NWDAF location information, if the selection is to decide candidates for analytics subscription transfer

-	(only when DCCF is hosted by NWDAF):

-	NF type of the data source.

-	NF Set ID of the data source.

NOTE 4:	Can be used when the NWDAF determines that it needs to discover another NWDAF which is responsible for co-ordinating the collection of required data. The NWDAF does a new discovery for a target NWDAF via NRF using NF Set ID of the data source.

NOTE 5:	For discovery of NWDAF supporting Nnwdaf_DataManagement service, at least the and NWDAF Serving Area information from the NWDAF profile are used.

NOTE 6:	The presence of NF type of data source or NF set ID of the data source denotes that the NWDAF can collect data from such NF Sets or NF Types.

-	Supported Analytics Delay of the requested Analytics ID(s) (see clause 6.2.6.2).

In the case of multiple instances of NWDAFs deployment, following factors may also be considered:

-	NWDAF Capabilities:

-	Analytics aggregation capability.

-	Analytics metadata provisioning capability.

Editor's note:	Whether analytics metadata provisioning capability to be registered in NRF or to be queried from NRF is FFS.

Applicable when NF consumer cannot determine a suitable NWDAF instance based on NRF discovery response, and when NWDAF registration in UDM is supported, as defined in clause 5.2 of TS 23.288 [86]: NF consumers may query UDM (Nudm_UECM_Get service operation) for determining the ID of the NWDAF serving the UE. The following factors may be considered by NF consumers to select an NWDAF instance already serving a UE for an Analytics ID:

-	SUPI.

-	Analytics ID(s).

When selecting an NWDAF for ML model provisioning, the following additional factors may be considered by the NWDAF:

-	The ML model Filter information (see clause 5.2, TS 23.288 [86]) for the trained ML model(s) per Analytics ID(s), if available.

# * * * * End of changes * * * *