3GPP TSG-SA2 Meeting #147-e	S2-2107607r02

# Online, , 18th-22nd Oct 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 034 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification on MBS Session update procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo Mobile Communications Co. LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | [7.2.6] 1. The procedure is only for update of QoS parameter. 2. Step 8 is not always needed. 3. Other QoS parameters than ARP for associated QoS flow also can be updated by RAN node if associated QoS flow is known. 4. Notification in step 7 uses correlation info instead of TMGI. 5. Step 8 only for individual delivery. [7.2.7] 6. Duplicated description with 7.2.6 7. AF does not know the UE connection state, AF informs area update is not a good practice. 8. Use different method for UE location query. 9. PDU session update is needed when previously outside but now inside. [7.2.X] 10. Current specification does not specify when the session is updated to add new QoS flows or remove QoS flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [7.2.6] 1. Change title for QoS parameters update. 2. Make step 8 optional in figure 3. Specify that RAN update associated QoS flows with the updated QoS parameters. 4. Change TMGI to correlation info in step 7. 5. Change step 8 only for individual delivery. [7.2.7] 6. Reuse 7.2.6 as possible 7. Remove the AF informs area update to UE in figure. 8. Reuse UE location event subscription for UE location query. 9. Specify that PDU session update is also needed when previously outside but now inside. [7.2.X] 10. Add new clause for QoS flow update, and the PDU session modification is triggered after HO to avoid massive PDU session modification.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not clear and complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.6, 7.2.7, 7.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.2.6	Multicast MBS session QoS update procedure

Multicast MBS session QoS update procedure is invoked by the AF, and to update the QoS parameters (e.g., 5QI, ARP), or add/remove multicast QoS flows for an ongoing multicast session. For the interaction between AF and MB-SMF, see clause 7.1.1.6 and 7.1.1.7.

Figure 7.2.6-1: Multicast MBS session QoS update procedure.

1.	This procedure is triggered by the MB-SMF receiving the updated policy for a multicast MBS session, see clauses 7.1.1.6 and 7.1.1.7.

2.	The MB-SMF generates the QoS profile for the multicast MBS session, and invokes Namf_MBSCommunication_N2MessageTransfer service operation (MBS Session ID, N2 SM information (TMGI, QoS profiles for multicast MBS session)) to the AMF(s).

3.	The involved AMF sends N2 MBS session request (N2 SM information) to the NG-RAN nodes based on the MBS Session ID and RAN node IDs stored in the AMF for the multicast MBS session.

4.	The NG-RAN node updates the multicast MBS session context based on the received N2 SM information, and if only QoS parameters are updated without multicast QoS flows added/removed, the NG-RAN also updates the QoS parameters of the associated QoS flows in the associated PDU Sessions.

5.	The NG-RAN node(s) acknowledges N2 MBS session request by sending an N2 MBS session Response message to the AMF.

6.	The AMF invokes the Nmbsmf_MBSSession_ContextUpdate () to the MB-SMF.

7.	The MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify request (subscription correlation info, Event ID, QoS profiles for multicast MBS session) to the SMFs.

8.	[Conditional] The SMF determines the UE list with 5GC Individual MBS traffic delivery regarding the MBS Session ID related to the subscription correlation info. If the associated QoS flows is to be added/removed, SMF triggers PDU Session Modification procedure for each UE in the UE list as defined in TS 23.502 [6].

# * * * * End of changes * * * *