3GPP TSG-SA2 Meeting #147e 	S2-2107782

# Elbonia, Elbonia, Oct 18 - Oct 22, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0052 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Multicast session service area update procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | "Editor´s note: Whether this procedure can be combined with clause 7.2.6 will be addressed in the next meeting" needs to be addressed. 7.2.6 contains the QoS update procedure but there are substantial differences: + The main part in 7.2.6 is direct signalling between MB-SMF, AMF and RAN nodes, but this signalling is optional in 7.2.7 because it is not suitable to address the addition of additional RAN nodes to the updated multicast area. + 7.2.7 contains a step for location inquiry and signalling towards the UE which is not present in 7.2.6. + The handling of the received information in RAN nodes differs substantially. However, if service area change and location change occur together, both update procedures can be combined. For the sake of clarity, it is still proposed to maintain separate description and only add a NOTE that the procedures can be combined. For the PDU session modification details are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A separate description of QoS updates and service area updates is maintained but a NOTE is added that both procedures can be executed together. For the PDU session modification details are provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unreolved editor´s note and missing details for PDU session modification for service area change |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.2.7	Multicast session service area update procedure

NOTE:	If the service area and QoS of a multicast service needs to be updated at the same time, the procedures in clause 7.2.6. and 7.2.7 will be exceuted together, i.e. using combined signalling interactions.

9. Determine affected UEs

9. Determine affected UEs

Figure 7.2.7-1: MBS service area update procedure for multicast

This procedure is triggered by the MB-SMF receiving the updated service area for multicast MBS session, see clauses 7.1.1.6 and 7.1.1.7.

1.	The AF providing the updated service area may also inform UEs at application level about the new service area via a service announcement. If a UE is located in a radio cell which was previously outside the service area and is now inside the updated service area, the UE may join the multicast service.

2.	[Conditional] This step is optional and can be omitted. However, it allows to terminate data transmission towards cells served by the RAN node previously in the multicast service area and now outside the updated multicast service area as quickly as possible. The MB-SMF may alternatively inform only NG-RAN nodes handling the multicast session but where the MBS service area was restricted in such a way that some or all cells served by the RAN node previously in the multicast service area are now outside the updated MBS service area

Same as described in steps 2-6 of figure 7.1.1.6-1 with the difference that the MB-SMF 	invokes the Namf_MBSCommunication_N2MessageTransfer request (TMGI, [Area Session ID], N2 SM message container (TMGI, MBS service area, [Area Session ID]).

The involved AMFs forward the N2 SM information received from MB-SMF to the RAN nodes handling the multicast MBS session and possiple Area Session ID

. The 	NG-RAN nodes updates the MBS session context with the new MBS service area. If an NG-RAN node is informed via MB-SMF about an updated MBS service area of the multicast MBS session and some cells it serves previously within the old MBS service area are now outside the updated MBS service area, it ends transmission of the related multicast data towards those cells. If the RAN node no longer serves any cells within the updated MBS service area with users inside the multicast MBS session, it requests (via AMF) from the MB-SMF that the distribution of the multicast data towards the NG-RAN node is terminated, see clause 7.2.2.4.

3.	The MB-SMF invokes Nmbsmf_MBSSession_ContextStatusNotify (subscription correlation info, Event ID, MBS service area, [Area Session ID]) to each SMF with a related subscription. If Area Session ID exists, the MB-SMF notifies the MBS service area corresponding to the Area Session ID to the SMFs involved in the multicast MBS session identified by the MBS Session ID. It is assumed that each SMF serving UEs within the multicast MBS session performs a related subscription for the multicast MBS session at the MB-SMF when the first such UE joins.

4.	The SMF determines the affected UEs it serves based on the subscription correlation identifying the multicast MBS session and Area Session ID (if exists) included in the step 3.

The subsequent steps 5 to 7 are executed separately for each affected UE.

5.	[Conditional] If the SMF previously subscribed at the AMF to notifications about the UE moving in or out of a subscribed "Area Of Interest" (for individual delivery), the SMF updates the subscription with the new MBS service area as area of intrest.

6.	The SMF may query AMF or NG-RAN for the current location of the UE (e.g. cell ID and/or TAI) to determine whether the UE is within the updated MBS service area.

7.	The SMF informs each UE involved in the multicast session that it serves about the changed service area by providing information about the multicast session ID and updated service area in a N1 container. For a UE previously inside the MBS service area but now outside the updated MBS service area of the multicast MBS session, the SMF may alternatively signal towards the UE that it has been removed from the multicast session to trigger UE resources for the reception of multicast data to be released and/or a related notification towards the user

.

The SMF also updates the PDU session resources associated to the multicast MBS session with the new MBS service area in an N2 container. The RAN node serving the PDU session starts or terminates transmission of multicast content in cells which are added or removed in the updated service area, respectively, and if necessary interacts with the MB-SMF to start ot terminate the distribution of multicast data to the RAN node. For a local multicast session and a UE previously outside the service area but now inside the updated service area, the SMF may also add associated unicast QoS flows for the multicast MBS session to the PDU session resources.