SA WG2 Meeting #S2-148E	S2-2108299

15 - 22 November, 2021, Electronic meeting

# Broadband Forum Liaison To:

3GPP Liaison Coordinator <3GPPLiaison@etsi.org>

3GPP TSG SA WG2

Puneet Jain, 3GPP SA2 Chairman <puneet.jain@intel.com>

3GPP TSG CT WG4 
Peter Schmitt, 3GPP CT4 Chairman <peter.schmitt@huawei.com>

CC:

Belal Hamzeh, CableLabs CTO and SVP <b.hamzeh@cablelabs.com>

# From:

Lincoln Lavoie, Broadband Forum (BBF) Technical Committee Chair <lylavoie@iol.unh.edu>

Liaison Communicated By:

Manuel Paul, BBF liaison officer to 3GPP <manuel.paul@telekom.de>

Date: September 16th, 2021

Subject: EAP-5G changes

Dear colleagues,

During the course of our deliberations, we have come to realize that the use of EAP-5G over PPP for W-CP initiation is an unsuitable choice for 5G-RG support over wireline. The issue we have identified is that EAP over PPP does not provide a fragmentation mechanism, and previous liaisons have advised us that the wireline MTU will be inadequate to communicate some SUCI encodings without fragmentation and reassembly support.

As such we have modified our call flows to initiate the VSNP procedures to establish reliable NAS transport with fragmentation and reassembly support earlier in the call flow sequence, and communicate the key elements of the EAP-5G exchange as part of W-CP initiation dialog within our framework of VSNP messaging.

We note the above change will have a number of impacts on TS23.316. We will document the updated procedures in issue 2 of TR-456, which is currently in straw ballot.

We look forward to continuing our fruitful relationship.

Thanks,

Lincoln Lavoie

Broadband Forum Technical Committee Chair

CC:

Liaisons at BBF <liaisons@broadband-forum.org>

Lincoln Lavoie, Broadband Forum Technical Committee Chair <lylavoie@iol.unh.edu>

Ken Ko, Broadband Forum Managing Director <kko@broadband-forum.org>

April Nowicki, Broadband Forum Member Support Manager <anowicki@broadband-forum.org>

David Allan, Broadband Forum WWC Work Area Director <david.i.allan@ericsson.com>

# Broadband Forum Reference: LIAISE-477

# Date of Upcoming Broadband Forum Meetings: A list of upcoming meetings can be found at https://www.broadband-forum.org/what-s-happening/meetings-events-webinars/upcoming-bbf-meetings