SA WG2 Meeting #146e	S2-2108312

15 – 22 November 2021, e-Meeting

Source:	Qualcomm Incorporated

Title:	Discussion of Open-Source-Defined Multi-Access Edge Computing

Document for:	Discussion / Approval

Agenda Item:

Work Item / Release:

Abstract of the contribution: Discusses the architecture and the principles of open-source-defined multi-access edge computing, include the network decoupling and reconfiguration of the network functions and resources.

# 1. Introduction

At recent SA2 meetings, there were different views concerning the usefulness of, or necessity for, cells as seen by the Multi-Access Edge Computing (MEC) whose reference architecture is built upon special hardware and its software functions are closely integrated with the hardware, which is inconvenient in the face of rapidly evolving scenarios and requirements of wireless network[1][2]. As a remedy, the above challenges may be met by introducing the open-source-defined (OSD) cellular network concept into MEC for allowing operators to adaptively customize their users’ networks[3].

S2-2108312

The development of Open-source software: - The organizations of SHARE and DECUS in 1950s. - ARPANET, UNIX, Linux and Android were developed for altruistic public sharing. OSD cellular networks are still in their infancy. Explicitly, an open infrastructure would enable flexible network reconfiguration based on the specific requirements of diverse scenarios. Such an OSD cellular network has to rely on the integration of beyond fifth generation (B5G)/6G candidate technologies, such as network function virtualization (NFV) , software-defined networking (SDN) , network slicing and MEC, etc. Among these candidate technologies, SDN/NFV serve as the core drivers, which should additionally be integrated with MEC for cell-edge performance enhancement in support of customized radio access networks (RANs) conceived for IoT and ultra Reliable Low Latency Communications (uRLLC)[4]. Substantial efforts have been invested in promoting the progress of OSD cellular networks: - The Open-Air-Interface (OAI) community founded in Europe aims for developing open source software for cellular networks, as exemplified by a fully-fledged fourth generation (4G) protocol stack based on commercial off-the-shelf (COTS) hardware. - The Open5G community founded in China developed the concepts, technologies and platforms of open source fifth generation (5G). - The O-RAN alliance founded by the global industry supported by academics has endeavoured to evolve RANs around the world. - The Linux Foundation (LF) launched the open network automation platform (ONAP) for orchestration[5]. - The Akraino project built by LF offers a pair of blueprints, namely the 5G MECice and the micro-MEC system

# 2. Support of OpenMEC Framework

Proposal 1	Using ETSI's MEC architecture as a spring-board, an OpenMEC framework  is introduced to facilitate customized services for any potential scenario, which is built on a pair of key principles, namely on network decoupling and reconfiguration.

Fig. 1: OpenMEC framework.

The OpenMEC framework conceived is shown in Fig. 1. The entire framework includes the application plane and MANO plane, where the former is mainly responsible for data processing and transmission, involving the infrastructure layer, virtualization layer, service-based MEC layer, and application layer. By contrast, the latter is composed of the virtualized infrastructure manager (VIM) and MANO, which completes the coordination and resource management of the OpenMEC system.

The infrastructure layer covers all computing, caching and communication resources of the system. Specifically, the central processing unit (CPU) provides high-performance computing power for the service-based MEC layer. The memory and the hard disk drive (HDD)/solid state drive (SSD) are the main components of the caching resources in the infrastructure layer. The communications resources of this layer include the band-width as well as the eNB, next-generation node base station (gNB) and access point (AP).

The underlying three-dimensional resources may be decoupled from the dedicated hardware and abstracted into a resource pool that could be shared by various NFs at the service-based MEC layer based on the NFV concept. Then, OpenMEC creates multiple Docker containers or virtual machines (VMs), which are run on the underlying resource pool and could simultaneously support different NFs for providing customized services [38], as seen at the virtualization layer.

The service-based MEC layer constitutes the core part of OpenMEC, which includes a unified SBI and diverse NFs, where the SBI can connect these NFs together based on the unified stateless hypertext transfer protocol (HTTP) for ensuring that they can communicate directly with each other when needed. Furthermore, we decouple the originally centralized service functions into independent NFs. These NFs are transparent to each other and can also be combined at will, so that they could be activated and released by all users instantly in support of their customized services.

The MANO plane is responsible for managing these NFs as well as APPs, and for scheduling the VIM to allocate resources. As mentioned earlier, MANO is in charge of MEC reconfiguration, where the OpenMEC’s templates and instances are proposed for implementing flexible reconfiguration, as and when required. The VIM of the MANO plane manages the virtualized resources according to MANO’s commands, so as to ensure that appropriate computing, caching and communication resources are supplied for the upper layers.

# 3. Support of MEC Decoupling

Proposal 2	With reference to the service-based architecture (SBA) of the 5G core network (5GC), a service-based MEC layer is developed for the OpenMEC scheme for decomposing the tightly coupled service functions into multiple independent network functions (NFs), thereby achieving MEC decoupling.

Fig. 2: A detailed view of the service-based MEC layer of Fig. 1.

In order to decouple the MEC functions, we conceive the service-based MEC layer, which evolved from the traditional MEC layer, mainly composed of the SBI and the service-oriented NFs. The service-based MEC layer is extracted from Fig. 1, as seen in the detailed view of Fig. 2.

Firstly, the SBI connects all the NFs together for facilitating their interface programming and implementation. The north-bound interface (NBI) is an application program interface (API) provided for the application layer to interact with the service-based MEC Layer in order to obtain services. The east-bound interface (EBI) is an API for NF’s MANO to interact with the service-based MEC Layer, which is mainly used to manage and orchestrate both the resource allocation and life cycle of NFs.

Secondly, all service-oriented NFs are independently deployed in the Docker containers of Fig. 1 to get virtualized. There are six basic NFs in our proposed OpenMEC of Fig. 1. Specifically, the MEC NFs of 5GC include the unified data management (UDM), the NF repository function (NRF) and UPF seen in Fig. 2. Furthermore, the communication protocol conversion function (CPCF), the service registry function (SRF) and the application selection function (ASF) of Fig. 2 are newly developed after considering the context of OpenMEC.

Then, the NF’s MANO of Fig. 2 can flexibly schedule the NFs according to the specific service types, since the unified HTTP is used as EBI between the service-based MEC layer and NF’s MANO, and also used as SBI in the service-based MEC layer. With all the NFs deployed in the Docker containers of Fig. 1 in our solution, Kubernetes, the open-source implementation of the container cluster management, is advocated for centrally managing all the containers playing the role of MANO.

Finally, based on the above NFs, we are ready for characterizing the basic functionalities of OpenMEC. Hence anyone could develop his/her own NFs for supporting new applications.

# 4. Support of MEC Reconfiguration

Proposal 3	In order to integrate these decoupled NFs as required and to reconfigure a MEC system, the concept of templates and instances is implemented as part of the NF's management and orchestration (MANO) scheme, eventually generating a customized entity for supporting complete services.

Fig. 3: A detailed view of the NF’s MANO of Fig. 1.

As the MEC management is not directly related to the reconfiguration of OpenMEC, the joint management of MEC and NFV proposed by ETSI can still be used in OpenMEC, albeit the conception of service-based MEC layers is bound to impose challenges on the orchestration scheme. Therefore, by means of Kubernetes, we present OpenMEC’s templates and instances of NF’s MANO, which supports the flexible scheduling of NFs, and any further MEC reconfiguration.

More explicitly, some templates are pre-defined for the corresponding applications. However, when no user asks for this application, we do not allocate the related resources to the template, hence, the template is empty. By contrast, if a user requests this application, we shall activate the template by assigning computing, caching and communication resources as an instantiation.

A detailed view of the NF’s MANO extracted from Fig. 1 is shown in the middle (blue) part of Fig. 3, where the right side represents the predefinition and selection of templates, while the left side represents the templates’ instantiation based upon Kubernetes, which is a container-based cluster management platform. The VIM seen at the bottom right of Fig. 3 is used for scheduling the underlying resource pool to arrange for the immediate resource deployment for instantiation, so as to avoid the assignment of excessive resources.

Observe in Fig. 3 that there is a master node and several slave nodes in Kubernetes, where the former is in charge of managing Kubernetes, while the latter is in charge of the orchestration of NFs. In a slave node, there are two basic components dedicated to instantiation in NF’s MANO, namely Pod and Kubelet as seen in Fig. 3. As defined in, Pod is the smallest unit of deploying and monitoring NFs in Kubernetes, which can run several Docker containers simultaneously. Kubelet of Fig. 3 maintains the life cycle of Pods and the communication between the master and the slave nodes. In the master node, there are three basic components, namely the scheduler, controller and API server. The API server provides the only entry for slave nodes to the application layer, i.e., Pods communicate with the given templates selected by the user through the API server for implementing MEC reconfiguration. The Scheduler and Controller of Fig. 3 manage and monitor all slave nodes, e.g., for fault detection. Moreover, there is a Kubernetes’ data center represented by Etcd of Fig. 3 for storing the states of the above components.

The template seen close to the right of Fig. 3 is provided for extracting the common characteristics of a class of problems. Given this idea, a static OpenMEC template is predefined, e.g., for selecting appropriate NFs and for predefining the related parameters according to the specific type of APPs. Again, we consider two APPs, namely high throughput and intensive computation, as seen at the right of Fig. 3. These are examples of describing the OpenMEC templates, where the former is provided for high-throughput multimedia video services, while the latter is mainly for computing intensive services.

The OpenMEC template cannot directly provide services and we do not allocate the related resources for the template. As a result, users can only gain access to the edge services through instances. The instantiation determines whether the OpenMEC system can run stably, which is the key for reconfiguring MEC.

The instantiation process mainly includes the following three steps. Firstly, when a user requests edge services, such as intensive computation and high throughput, the MANO of Fig. 3 configures the operating environment of NFs, i.e. the API Server of Fig. 3 locates the idle Pod and updates the relevant information to Etcd of Fig. 3, which provides basic environmental support for instantiation. Secondly, the MANO of Fig. 3 schedules VIM for allocating the specified virtual computing, caching and communication resources to users. Finally, under the specifically configured operating environment and the resources allocated, MANO actually runs the predefined NFs and APPs of the template, i.e., Kubelet runs the Pod that specifically deploys NFs and APPs. Therefore, the dedicated MEC instance has been created.

At this point, we have succeeded in creating the OpenMEC scheme by completing the MEC decoupling and reconfiguration.

# 5. Conclusions

The following proposals have been made.

Proposal 1	Using ETSI's MEC architecture as a spring-board, an OpenMEC framework  is introduced to facilitate customized services for any potential scenario, which is built on a pair of key principles, namely on network decoupling and reconfiguration.

Proposal 2	With reference to the service-based architecture (SBA) of the 5G core network (5GC), a service-based MEC layer is developed for the OpenMEC scheme for decomposing the tightly coupled service functions into multiple independent network functions (NFs), thereby achieving MEC decoupling.

Proposal 3	In order to integrate these decoupled NFs as required and to reconfigure a MEC system, the concept of templates and instances is implemented as part of the NF's management and orchestration (MANO) scheme, eventually generating a customized entity for supporting complete services.

# 7. References

[1]	S2-2102332: “Using URSP Validation Criteria for Edge Computing”, LS from SA2#143e.

[2]	S2-2105303: “LS reply to SA on GSMA 3GPP Edge Computing coordination”, LS from SA2#146e.

[3]	S2-2105361: “LS from GSMA Operator Platform Group on edge computing definition and integration”, LS from SA2#146e.

[4]	S2-2106255: “New SID on Enhancement of support for Edge Computing in 5G Core network - phase 2” LS from SA2#146e.

[5]	S2-2106397: “Study on enhancement to support Edge Computing in 5GC” LS from SA2#146e.