3GPP TSG-SA WG2 Meeting #148E 	S2-2108313

# 15 - 22 November, 2021, Electronic meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3220 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for the figure of Number of UEs and PDU Sessions per network slice status retrieval by AF procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The figure of “Number of UEs and PDU Sessions per network slice status retrieval by AF procedure” is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the figure of Number of UEs and PDU Sessions per network slice status retrieval by AF procedure of clause 4.15.3.2.12 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

4.15.3.2.12	Number of UEs and PDU Sessions per network slice status retrieval by AF procedure

AF

NSACF

1. Nnef_SliceStatus_Retrieval Request

NEF

3. Nnsacf_SliceStatus_Retrieval Request

4. Nnsacf_SliceStatus_Retrieval Response

5. Nnef_SliceStatus_Retrieval Response

2. AF authentication

Figure 4.15.3.2.12-1: Number of UEs and PDU Sessions per network slice status retrieval by AF procedure

1.	To retrieve information about the number of the UEs registered with a network slice or the number of the PDU Sessions established on a network slice or both, the AF sends Nnef_SliceStatus_Retrieval Request (Event ID, Event Filter) message to the NEF. The Event ID parameter defines the information to be reported, i.e. the number of registered UEs with a network slice or the number of the PDU sessions with a network slice or both. The Event Filter parameter defines the S-NSSAI for which reporting is required.

2.	The NEF checks whether the AF is authorised. If authorised, the NEF may query the NRF to find the NSACF responsible for the requested S-NSSAI.

3.	The NEF invokes the request to the NSACF with Nnsacf_SliceStatus_Retrieval Request (Event ID, Event Filter).

4.	The NSACF returns the Nnsacf_SliceStatus_Retrieval Response (Event ID, Event Filter, Event Reporting information) message to the NEF. In the Event Reporting information parameter the NSACF provides information for the current number of the UEs registered with a network slice (e.g. represented in percentage of the maximum number of the UEs registered with the network slice) or information for the current number of the PDU Sessions on a network slice (e.g. represented in percentage of the maximum number of the PDU Sessions established on the network slice) or both depending on the Event ID.

5.	The NEF invokes the message to the AF with the Nnef_SliceStatus_Retrieval Response (Event ID, Event Filter, Event Reporting information) message.

For one S-NSSAI if multiple of NSACF is deployed in the network, the procedure has following difference:

-	Step 2: The NEF finds that there are several different NSACFs served the indicated S-NSSAI.

-	Step 3: The NEF invokes the request to all related NSACF. If the NSACF Set is configured in the network, the NEF selects the NSACF per NSACF Set, i.e. only one NSACF instance is selected from the NSACF Set and this NSACF instance retrieves the slice status of this Set.

-	Step 5: The NEF aggregates the event reporting information from all selected NSACF before invokes the response message to the AF.

* * * * end of change * * *