3GPP TSG-WG SA2 Meeting #148E e-meeting 	S2-2108353

# Elbonia, November 15 – 22, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.032 | CR | 0021 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing new high accuracy GAD shape with scalable uncertainty |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, LCS_LTE_acc_enh-Core |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In 23.032 CR 0015 R1 two new high accuracy shapes was added. The shapes are used by various EPS protocols such as the LPP protocol (TS 37.355). The LPP protocol do however also support inclusions of Information Elements defined by the OMA LPPe protocol (ref [20] in 37.355). The LPPe protocol defines uncertainty ranges that are larger than what is defined in the high accuracy shapes of 23.032 (in 23.032 maximum uncertainty is approximately 46.5 m while LPPe allows up to 205 m). This has the implication that location information from the UE using LPP/LPPe protocol in common scenarios cannot be conveyed using the high accuracy shapes. This leads to a loss of accuracy, especially in cases when horizontal uncertainty is above 46.4 m and vertical location is of high accuracy. This as the legacy shapes for vertical component has a granularity of 1 m and minimum uncertainty of 1.13 m. Thus, it is needed to in a backward compatible way extend the uncertainty range of high accuracy shapes to align with the LPPe protocol. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing two high accuracy GAD shapes where uncertainty range can be configured to align with existing shapes or align with uncertainty ranges of LPPe. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In EPS, if LPPe is used, for some ranges of reported uncertainties the high accuracy shapes cannot be used which leads to a loss in accuracy. This may imply challenges to fulfil e.g., regulatory requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5, 5.3x (new), 5.6x (new), 6.2x (new), 7.2, 7.3.3x (new), 7.3.6x (new), 7.3.3x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** BEGIN 1st CHANGE *****

# 5	Shapes

The intention is to incorporate a number of different shapes, that can be chosen according to need.

-	Ellipsoid Point;

-	Ellipsoid point with uncertainty circle;

-	Ellipsoid point with uncertainty ellipse;

-	Polygon;

-	Ellipsoid point with altitude;

-	Ellipsoid point with altitude and uncertainty ellipsoid;

-	Ellipsoid Arc;

-	High Accuracy Ellipsoid point with uncertainty ellipse;

-    High Accuracy Ellipsoid point with scalable uncertainty ellipse.

-	High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid.

-	High Accuracy Ellipsoid point with altitude and scalable uncertainty ellipsoid.

Shapes relevant to Local Co-ordinates:

-	Local 2D point with uncertainty ellipse (only in 5GS);

-	Local 3D point with uncertainty ellipsoid (only in 5GS).

Each shape is discussed individually.

***** NEXT CHANGE *****

## 5.3x High Accuracy Ellipsoid point with scalable uncertainty ellipse

The "high accuracy ellipsoid point with scalable uncertainty ellipse" is characterised by the co-ordinates of an ellipsoid point (the origin), distances r1 and r2 and an angle of orientation A, as described in clause 5.3. Compared to the "ellipsoid point with uncertainty ellipse", the "high accuracy ellipsoid point with scalable uncertainty ellipse" provides finer resolution for the co-ordinates, and distances r1 and r2, and additionally provides prossibility to choose uncertainty range compared to "high accuracy ellipsoid point with uncertainty ellipse".

***** NEXT CHANGE *****

## 5.6x	High Accuracy Ellipsoid point with scalable altitude and uncertainty ellipsoid

The "high accuracy ellipsoid point with altitude and scalable uncertainty ellipsoid" is characterised by the co-ordinates of an ellipsoid point with altitude, distances r1 (the "semi-major uncertainty"), r2 (the "semi-minor uncertainty") and r3 (the "vertical uncertainty") and an angle of orientation A (the "angle of the major axis"), as described in clause 5.6. Compared to the "ellipsoid point with altitude and uncertainty ellipsoid", the "high accuracy ellipsoid point with altitude and scalable uncertainty ellipsoid" provides finer resolution for the co-ordinates, and distances r1, r2, and r3, and additionally provides prossibility to choose uncertainty range compared to “high accuracy ellipsoid point with altitude and uncertainty ellipse".

***** NEXT CHANGE *****

## 6.2x	High Accuracy Extended Uncertainty

The high accuracy extended uncertainty r, expressed in metres, is mapped to a number K, with the following formula:

with C = 0.3 and x = 0.02594, with 0  K  253, and r = 200 m with K=254, and r > 200 m with K=255 a suitably useful range between 0 and 200 metres is achieved for the high accuracy uncertainty, while still being able to code down to values as small as 8 millimeters. The uncertainty can then be coded on 8 bits, as the binary encoding of K.

Table 6.2a-1: Example values for the high accuracy uncertainty function

| Value of K | Value of uncertainty |
| --- | --- |
| 0 | 0 m |
| 1 | 0.00778 m |
| 2 | 0.01577 m |
| - | - |
| 20 | 0.20068 m |
| - | - |
| 40 | 0.53560 m |
| - | - |
| 60 | 1.09457 m |
| - | - |
| 80 | 2.02744 m |
| - | - |
| 100 | 3.58434 m |
| - | - |
| 120 | 6.18271 m |
| - | - |
| 127 | 7.45551 m |
| - | - |
| 253 | 195.12396 m |
| 254 | 200 m |
| 255 | > 200 m |

***** NEXT CHANGE *****

## 7.2	Type of Shape

The Type of Shape information field identifies the type which is being coded in the Shape Description. The Type of Shape is coded as shown in table 2a.

Table 2a: Coding of Type of Shape

| Bits |  |
| --- | --- |
| 4 3 2 1 |  |
| 0 0 0 0 | Ellipsoid Point |
| 0 0 0 1 | Ellipsoid point with uncertainty Circle |
| 0 0 1 1 | Ellipsoid point with uncertainty Ellipse |
| 0 1 0 1 | Polygon |
| 1 0 0 0 | Ellipsoid point with altitude |
| 1 0 0 1 | Ellipsoid point with altitude and uncertainty Ellipsoid |
| 1 0 1 0 | Ellipsoid Arc |
| 1 0 1 1 | High Accuracy Ellipsoid point with uncertainty ellipse |
| 1101 | High Accuracy Ellipsoid point with scalable uncertainty ellipse |
| 1 1 0 0 | High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid |
| 1111 | High Accuracy Ellipsoid point with altitude and scalable uncertainty ellipsoid |
| other values | reserved for future use |

***** NEXT CHANGE *****

### 7.3.3x	High Accuracy Ellipsoid point with scalable uncertainty ellipse

Figure 7.3.3x-1: Shape description of a high accuracy ellipsoid point with uncertainty ellipse

High Accuracy Degrees of Latitude:

Bit 8 of octet 2 is the high order bit.

Bit 1 of octet 5 is the low order bit.

High Accuracy Degrees of Longitude:

Bit 8 of octet 6 is the high order bit.

Bit 1 of octet 9 is the low order bit.

Orientation of major axis:

angle in degrees between the major axis and north

(0 = north, 90 = east, values of 180 and above are not used).

U:	Uncertainty Range

Bit value 0	High Accuracy default uncertainty range used

Bit value 1	High Accuracy Extended Uncertainty Range used

***** NEXT CHANGE *****

### 7.3.6x	High Accuracy Ellipsoid point with altitude and scalable uncertainty ellipse

Figure 7.3.6x-1: Shape description of a High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid

High Accuracy Degrees of Latitude:

Bit 8 of octet 2 is the high order bit.

Bit 1 of octet 5 is the low order bit.

High Accuracy Degrees of Longitude:

Bit 8 of octet 6 is the high order bit.

Bit 1 of octet 9 is the low order bit.

High Accuracy Altitude:

Bit 6 of octet 10 is the high order bit.

Bit 1 of octet 12 is the low order bit.

Orientation of major axis:

angle in degrees between the major axis and north

(0 = north, 90 = east, values of 180 and above are not used).

HU: Horizontal Uncertainty Range

Bit value 0	High Accuracy default uncertainty range used

Bit value 1	High Accuracy Extended Uncertainty Range used

VU: Vertical Uncertainty Range

Bit value 0	High Accuracy default uncertainty range used

Bit value 1	High Accuracy Extended Uncertainty Range used

NOTE:	The same "High Accuracy Uncertainty" (defined in clause 6.2a) or "High Accuracy Extended Uncertainty" (defined in clause 6.2x) is used for both horizontal and vertical location.

***** END OF CHANGES *****