3GPP TSG SA WG2 Meeting SA2#148E 		S2-2108362

November 15 – 22, 2021, Electronic, Elbonia

Title:	[DRAFT] Reply LS on MTU and other topics

Response to:	Reply to your liaison S2-2106577 (LIAISE-474 / S2-2108290)

Release:	Rel-16

Work Item:	5WWC

Source:	Ericsson [to be: SA2]

To:	BBF

Cc:	CT4, Cablelabs

Contact person: 	Stefan Rommer

stefan.rommer@ericsson.com

Attachment: 	TS 23.316 CR2061

# 1	Overall description

SA2 would like to thank BBF for the replies to our liaison.

Q1)	SA2 agrees that the MTU can be locally configured on SMF, e.g. per DNN, and has approved the attached CR to clarify this.

Q2 and Q3) 	SA2 takes note of the replies.

Q4)	SA2 would like to inform that TS 23.503 has been updated for Rel-17 to remove the restriction with respect to the simultaneous use of a DNN in both a URSP Traffic descriptor and Route Selection Descriptor. Please see TS 23.503, v17.2.0, clause 6.6.2.1 (excerpt provided below).

-	DNN Selection: Indicates that the traffic of the matching application shall be routed via a PDU Session supporting any of the included DNNs. It includes one or more DNN(s). If a DNN Selection component is provided in the Route Selection Descriptor then the UE shall use any of the DNNs of the DNN Selection component, instead of the DNN requested by the application for the PDU Session that is used to route the traffic of the matching application. If there is no DNN Selection component in the Route Selection Descriptor, then the UE shall use the DNN requested by the application for the PDU Session that is used to route the traffic of the matching application.

NOTE 2:	To provide uniform service experience for UEs from earlier Releases, when a USRP rule with a DNN in both, Traffic descriptor and Route Selection Descriptor, is provided to the UEs, the DNN(s) used in the Traffic descriptor would also need to be included in the policy for DNN replacement in the network. In addition, a lower priority Route Selection Descriptor without a DNN would also need to be provided to the UEs.

Q5 and Q6) 	SA2 takes note of the replies.

# 2	Actions

To BBF

ACTION: 	SA2 kindly requests BBF to take the above information into account.

# 3	Dates of next TSG SA WG2 meetings

SA2 WG Meeting #149 E		February 14 – 25, 2022		Elbonia

SA2 WG Meeting #150		April 4 – 8, 2022 		TBD