3GPP TSG SA WG2 Meeting SA2#148E 		S2-2108368

November 15 – 22, 2021, Electronic, Elbonia

Title:	[DRAFT] Reply LS on mandatory SSC modes supported by UE

Response to:	LS to SA2 on mandatory SSC modes supported by UE (C6-210258 / S2-2108269)

Release:	Rel-15 / Rel-16

Work Item:	5GS_Ph1_UEConTest

Source:	Ericsson [to be: SA2]

To:	CT6

Cc:	CT1

Contact person: 	Stefan Rommer

stefan.rommer@ericsson.com

Attachment: 	none

# 1	Overall description

SA2 would like to thank CT6 for the LS on mandatory SSC modes supported by UE.

SA2 would like to clarify the following aspects related to SSC modes:

-	SSC mode 1 does not require any specific UE functionality, as it is defined to support PDU Sessions where the network does not trigger any PDU Session release with the purpose to relocate the PSA UPF.

-	SSC mode 2 relies on UE supporting the reception of a NAS PDU Session Release Command with cause #39 "reactivation requested”, and act on that cause code to trigger a PDU Session Establishment.

-	SSC mode 3 relies on UE supporting the reception of a NAS PDU Session Modification Command with cause #39 "reactivation requested”, and act on that cause code to trigger a PDU Session Establishment.

It can be noted that these are regular SM procedures that have been defined as “SSC modes” to make the handling more explicit. In 4G/EPC, functionality corresponding to SSC modes 1 and 2 exist without specific terms.

The above UE behaviour for SSC modes 2 and 3 are described in TS 23.501 and further detailed in TS 24.501. The NAS procedures and cause codes required for SSC modes 2 and 3 are assumed to be supported by all UEs since Rel-15. It can also be noted that the use of cause #39 "reactivation requested” is not limited to SSC modes but is a generic cause. It can be e.g. used when a PDU Session Establishment is not accepted by the network (6.4.1.4 in 24.501) or in certain cases for CIoT (6.3.3.2 in 24.501). The UE is thus assumed to support such cause code for other scenarios not related to SSC modes.

Stage 3 (TS 24.501) further specifies the UE behaviour in relation to SSC modes 2 and 3 as a “should”. Excerpts from TS 24.501 are copied below.

For SSC mode 2 (clause 6.3.3.3):

If the PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested", then after completion of the network-requested PDU session release procedure, the UE should re-initiate the UE-requested PDU session establishment procedure…<snip>

For SSC mode 3 (clause 6.3.2.3):

If the selected SSC mode of the PDU session is "SSC mode 3" and the PDU SESSION MODIFICATION COMMAND message includes 5GSM cause #39 "reactivation requested", the UE can provide to the upper layers the PDU session address lifetime if received in the PDU session address lifetime PCO parameter of the Extended protocol configuration options IE of the PDU SESSION MODIFICATION COMMAND message. After the completion of the network-requested PDU session modification procedure, the UE should re-initiate the UE-requested PDU session establishment procedure with a new PDU session ID…<snip>

The UE may not be able to initiate PDU Session Establishment, e.g. if the UE has insufficient resources for doing so.

It can also be noted that SSC mode 3 is only applicable for IP based PDU Session types, while SSC modes 1 and 2 are applicable for any PDU Session Type.

Based on the above, SA2 would like to reply to CT6’s questions as follows:

Q1)	Is it correct, if we handle SSC modes 2 and 3 as optional?

SA2 reply: No, the related NAS procedures and cause codes are assumed to be supported by all UEs.

Q2)	Is it possible to modify 3GPP TS 23.501 to explicitly state which SSC modes have to be supported?

SA2 reply: Please see reply to Q1.

Q3)	If the support of SSC mode 2 and/or 3 is not optional but conditional, can you please specify the conditions?

SA2 reply: Please see reply to Q1.

# 2	Actions

To CT6

ACTION: 	SA2 kindly requests CT6 to take the above information into account

# 3	Dates of next TSG SA WG2 meetings

SA2 WG Meeting #149 E		February 14 – 25, 2022		Elbonia

SA2 WG Meeting #150		April 4 – 8, 2022 		TBD