3GPP TSG-SA2 Meeting # 148E (e-meeting)	S2-2108394

# Elbonia, November 15 - 22, 2021												(revision of S2-2107197 )

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0007 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing MBS SAI and updating MBS service area |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | #1 RAN2 LS S2-2107037(R2-2108914) asked to introduce an MBMS SAI-like identifiers for the MBS service area. Introduction of MBS SAI is discussed in S2-2108406 which also proposes to remove TAI for simplicity. In addition to the discussion in S2-2108406. #2 In service announcement, if geographical area information or civic address information is used MBS service area, it cannot be applied to multi-carrier deployment which requires MBS SAI. Therefore, the following is proposed if the AF is not aware of MBS SAI list: the AF provides the geographical area information or civic address information to NEF/MBSF which translates the geographical area or civic address into MBS SAI list. The NEF/MBSF provides the MBS SAI list to AF in MBS session creation procedure or TMGI allocation procedure. After that, AF can use MBS SAI list in the service announcement. #3 In clause 7.1.1.2 & 7.1.1.3, MBS service area is not provided in TMGI allocation but provided in MBS session create (step 8), and consequently the MB-SMF selected during TMGI allocation may be different from the MB-SMF selected during MBS Session creation, resulting MBS Session creation failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #1 Introduce 3-octet MBS SAI to identify a group of cells within a PLMN and is frequency agnostic and MBS SAI replaces TAI. Update description related to MBS service area as follows: MBS service area represented by MBS SAI list, cell ID list, geographical area information or civic address information can be used by AF, and MBS service area represented by MBS SAI list and/or cell ID list is used in 5GC and NG-RAN. - MBS service area (MBS SAI list, cell ID list, geographical area information or civic address information) may be included in the service announcement for local multicast MBS service or broadcast MBS service. - NEF translates the geographical area information or civic address information to MBS SAI list. - MBS service area (MBS SAI list and/or cell ID list) will be used by 5GC and NG-RAN for area restriction for local multicast MBS service or broadcast MBS service. - MBS service area (MBS SAI list and/or cell ID list) may be included in the Join Accept or Join Reject NAS messages #2 In MBS session create or TMGI allocate request, AF may provide geographical area information or civic address information as MBS service area. In this case, NEF/MBSF translates it into MBS SAI list and include MBS SAI list in in MBS session create or TMGI allocate response as MBS service area. #3 In clause 7.1.1.2 & 7.1.1.3, MBS service area should be provided in TMGI allocation when needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For multi-carrier deployment, the MBS solution cannot work properly, as UE cannot find the right frequency from SIB and service announcement. Risk of MBS Session creation failure due to different MB-SMFs selected for TMGI allocation and for MBS Session creation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 6.2.1, 6.2.2, 6.2.5, 6.9.1, 6.11, 7.1.1.2, 7.1.2, 7.1.3, 7.2.4.2.2, 7.2.4.3.2, 9.4.2.2, 9.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR3139 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR needs to be implemented after 23.247 CR0026 is implemented due to overlapping in clause 6.9.1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms and definitions defined in TR 21.905 [1] and the following apply:

5GC Individual MBS traffic delivery: 5G CN receives a single copy of MBS data packets and delivers separate copies of those MBS data packets to individual UEs via per-UE PDU sessions, hence for each such UE one PDU session is required to be associated with a multicast session.

5GC shared MBS traffic delivery: 5G CN receives a single copy of MBS data packets and delivers a single copy of those MBS data packets to a RAN node.

Area Session Identifier: A unique identifier within an MBS Session used for an MBS session with location dependent content. When present the Area Session ID, together with the TMGI, is used to uniquely identify the data flow of an MBS Session in a specific MBS service area.

Associated PDU Session: A PDU Session associated to a multicast session that is used for 5GC Individual MBS traffic delivery method and for signalling related to a user's participation in a multicast session such as join and leave requests.

Associated QoS Flow: A unicast QoS Flow that belongs to the associated PDU Session and is used for 5GC Individual MBS traffic delivery method. The associated QoS Flow is mapped from a multicast QoS Flow in a multicast MBS session.

Broadcast communication service: A 5GS communication service in which the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e., all UEs in the broadcast coverage area are authorized to receive the data).

NOTE 1:	For the broadcast communication service, the content provider and network may not be aware whether the authorized UEs are actually receiving the data being delivered.

Broadcast MBS session: An MBS session to deliver the broadcast communication service. A broadcast MBS session is characterised by the content to send and the geographical area where to distribute it.

Broadcast service area: The area within which data of one or multiple Broadcast session(s) are sent.

MBS QoS Flow: The finest granularity for QoS forwarding treatment for MBS data. Providing different QoS forwarding treatment requires separate MBS QoS Flows in 5GS supporting MBS.

MBS SAI:  An MBS SAI identifies a group of cells within a PLMN. The MBS SAI is independent of physical location of the cell(s) and is frequency agnostic. The MBS SAI is 3-octet long, and value 0 denotes all cells in the PLMN.

MBS Service Announcement: Mechanism to allow users to be informed about the available MBS services.

MBS session: A multicast session or a broadcast session.

Multicast communication service: A 5GS communication service in which the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e., not all UEs in the multicast coverage are authorized to receive the data).

NOTE 2:	For multicast communication service, the content provider and network can be aware whether the authorized UEs are actually receiving the data being delivered.

Multicast MBS session: An MBS session to deliver the multicast communication service. A multicast MBS session is characterised by the content to send, by the list of UEs that may receive the service and optionally by a multicast area where to distribute it.

MBS service area: The area within which data of one Multicast or Broadcast MBS session may be sent. For location dependent MBS, the MBS service area is uniquely identified by the combination of Area Session ID and MBS Session ID and corresponds to the location dependent content data of the MBS Session ID.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [5] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

FEC	Forward Error Correction

CDN	Content Delivery Network

LL MC	Lower Layer Multicast

MBMS	Multimedia Broadcast/Multicast Service

MBS	Multicast/Broadcast Service.

MB-SMF	Multicast/Broadcast Session Management Function

MBSF	Multicast/Broadcast Service Function

MBSTF	Multicast/Broadcast Service Transport Function

MB-UPF	Multicast/Broadcast User Plane Function

PTM	Point To Multipoint

PTP	Point To Point

SAI	Service Area Identity

SSM	Source Specific Multicast address

TMGI	Temporary Mobile Group Identity

****************** NEXT CHANGES ***************

## 6.2	Local MBS service and Location dependent MBS service

### 6.2.1	General

A Local MBS service is an MBS service provided in one MBS service area. A location dependent MBS service is an MBS service provided in several MBS service area(s). An MBS service area is identified by a cell list or an MBS SAI list. The MBS service area could be geographical area information or civic address information, and NEF/MBSF translates the location information to Cell ID list or MBS SAI list as MBS service area, see clause 7.1.1.2.

If the multicast session relates to a Local MBS service or a location dependent MBS service and the SMF configures 5GC Individual MBS traffic delivery, the SMF subscribes at the AMF to notifications about the "UE location" or UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

For multicast communication, local MBS and location dependent MBS services may be supported via 5GC Individual MBS traffic delivery towards RAN nodes not supporting MBS. If the SMF obtains a notification about the UE location, it checks whether the UE is still in the MBS service area of the multicast session. For a local MBS service, if the UE is no longer in the MBS service area, the SMF terminates the 5GC Individual MBS traffic delivery towards the UE. For a location dependent MBS service, if the UE is no longer in the current MBS service area, the SMF determines whether the UE is in another MBS service area of the multicast session; if so, the SMF configures the UPF to send multicast data relating to the new MBS service area towards the UE. If the SMF terminates the 5GC Individual MBS traffic delivery towards the UE, it unsubscribes at the AMF from the notifications about the UE location or "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

### 6.2.2	Local MBS service

For a local MBS service, only UEs within the MBS service area may receive content data, while UEs outside the MBS service area are not allowed to receive location specific content. For multicast MBS service, UEs outside the MBS service area are not allowed to join the MBS service, and the network shall not deliver location specific content anymore to the UEs moved out of the MBS service area. The UE shall be able to obtain service area information of the local multicast service via MBS service announcement or via NAS signalling (UE Session Join Accept/Reject including Cell ID list or MBS SAI list). If the UE Session Join procedure fails due to the UE being outside the MBS service area, the UE does not attempt to join the multicast session again until the UE moves inside the MBS service area. When the UE Session Join succeeds and if the multicast session is deactivated, the UE does not perform monitoring the session activation notification and any other information related to the multicast session identified by an MBS Session ID over the radio if outside the MBS service area.

NOTE:	Broadcast communication service is the service provided simultaneously to all UEs in a geographical area, therefore for broadcast it is naturally a local MBS service.

****************** NEXT CHANGES ***************

### 6.2.5	Update of multicast local MBS service and location dependent MBS service

If an MBS service area is updated via configuration for multicast communication, the MB-SMF provides the updated MBS service area to:

- SMFs,

- and AMF(s), and the AMF(s) forward this information to the relevant NG-RAN(s) that update the MBS service area.

NG-RAN node configures the UE not to receive the MBS data over air interface if it detects the UE was in the previous service area but is outside the updated MBS service area NG-RAN node may release the tunnel for the shared delivery if none of the cells/MBS SAIs of the NG-RAN node belongs to the MBS service area any more. NG-RAN node configures the UE to receive the MBS data over air interface if it detects the UE was outside the previous service area but is inside the updated MBS service area, if part of cells/MBS SAIs of the NG-RAN node belongs to MBS service area and others outside the MBS service area.

****************** NEXT CHANGES ***************

## 6.9	MBS Session Context

### 6.9.1	MBS Session Context

The MBS Session Context contains all information describing a particular MBS session in the 5GS and is created in each node involved in the delivery of the MBS data.

The content of the Multicast MBS Session Context is described in Table 6.9.1-1.

Table 6.9.1-1: Multicast MBS Session context

| Parameter | Description | NG-RAN | MB-SMF | SMF | AMF |
| --- | --- | --- | --- | --- | --- |
| State | State of MBS session ('Active multicast session' or 'Inactive multicast session' or 'Configured multicast session') | X (note 2) | X | X (note 2) |  |
| SSM (source specific IP multicast address) | IP multicast address identifying the MBS session. |  | X (note 1) | X (note 1) |  |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | x | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) | X |
| MB-SMF | The MB-SMF that handles the MBS session. | X |  | X | X |
| QoS information | QoS information of the MBS session. | X | X | X |  |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. MBS SAI list and/or Cell ID list). | X (note 1) | X (note 1) | X (note 1) | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are involved in the multicast session |  |  |  | X |
| AMF | The AMF(s) which are selected for the MBS session | X | X |  |  |
| IP multicast and source address for data distribution | IP addresses identifying the SSM user plane transport for shared delivery between MB-UPF and NG-RAN and for individual delivery between MB-UPF and UPF when the IP multicast transport is used. | X (note 1) | X (note 1) | X (note 1) |  |
| SMF | The SMF(s) that manages the associated PDU session. |  | X |  |  |
| UE ID | ID identifying the UE that successfully join the Multicast MBS Session. For NG-RAN it is NGAP UE ID and for SMF it is SUPI. | X (note 3) |  | X (note 3) |  |
| IP address for distribution | The IP addresses and TEID of NG-RAN used for the user plane between NG-RAN and MB-UPF and between MB-UPF and UPF when Point to Point tunnel is used. | X (note 1) | X (note 1) | X (note 1) |  |
| TEID for data distribution | The tunnel ID used for receiving the multicast data for shared delivery by NG-RAN and for individual delivery by UPF | X | X | X |  |
| MB-PCF | The MB-PCF that provides policy control for the MBS session. |  | X (note 1) |  |  |
| NOTE 1: It is an optional parameter. NOTE 2: The value 'Configured multicast session' is not applicable for NG-RAN and SMF. NOTE 3: the UE ID is available within the UE Context which contains the MBS information. |  |  |  |  |  |

In Broadcast MBMS mode, an MBS Session Context is created in the NG-RAN, AMF, MB-SMF and MBSF as a result of the MBS Session Start procedure.

The content of the Broadcast MBS Session Context is described in Table 6.9.1-2.

Table 6.9.1-2: Broadcast MBS Session context

| Parameter | Description | NG-RAN | AMF | MB-SMF |
| --- | --- | --- | --- | --- |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) |
| AMF | The AMF(s) which are selected for the MBS session | X |  | X |
| QoS information | QoS information for the MBS Session, including the QoS parameters of QoS flows. | X |  | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. MBS SAI list and/or Cell ID list). | X | X | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are selected for the broadcast session |  | X |  |
| IP multicast address for data distribution | IP addresses identifying the user plane transport used for shared delivery between MB-UPF and NG-RAN when the IP multicast transport is used. | X (note 1) |  | X (note 1) |
| NG-RAN IP Address for data distribution | The IP address of NG-RAN used for the user plane between NG-RAN and MB-UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) |
| TEID for data distribution | The tunnel ID used for receiving the broadcast data for shared delivery by NG-RAN | X |  | X |
| MB-PCF | The MB-PCF that provides policy control for the MBS session. |  |  | X (note 1) |
| NOTE 1: It is an optional parameter. |  |  |  |  |

****************** NEXT CHANGES ***************

## 6.11	Service Announcement

Service Announcement provides the UE with descriptions specifying the multicast or broadcast services to be delivered as part of MBS Session.

The Service Announcement includes the MBS Session ID(s), which is represented by TMGI or a Source Specific IP Multicast Address, for the service. When the MBS Session ID is Source Specific IP Multicast Address, the Service Announcement may include the PLMN ID of the PLMN and NID for an SNPN in which the service is delivered.

The Service Announcement includes an MBS Service Type, which indicates whether the MBS Session for the service is multicast or broadcast.

NOTE 1:	A Source Specific IP Multicast Address as MBS Session ID indicates a multicast session.

For local MBS service, the Service Announcement may include the MBS service area. The MBS service area used by AF can be Cell ID list, MBS SAI list, geographical area information or civic address information. Amongst them, Cell ID list shall only be used by AFs who reside in trust domain, and when the AFs are aware of such information.

The Service Announcement may include the radio frequency information for the multi-carrier deployment as specified in TS 26.346 [13].

If the MBS Session is multicast, the Service Announcement may include the DNN and S-NSSAI of the PDU Session to indicate which PDU Session is associated with the MBS Session.

NOTE 2:	For multicast, AF or MBSF provides Service Announcement only after the MBS information is available to 5GC or the start time need be included, to avoid potential rejection sent by SMF of the MBS session join request.

NOTE 3:	The MBS Service related information, e.g. default PLMN ID, DNN and S-NSSAI can be pre-configured in the UE.

NOTE 4:	If DNN and S-NSSAI information is not provided in the service announcement or pre-configured, how UE determines the PDU session to join the MBS Session is implementation specific.

The Service Announcement may be provided to a UE by AF or MBSF, or may be retrieved by the UE from those entities.

NOTE 5:	How the UE can get the Service Announcement from other entities is not specified.

****************** NEXT CHANGES ***************

#### 7.1.1.2	Initial MBS session configuration without PCC

This procedure is used by the AF to start the MBS Session towards 5GC and consist of TMGI allocation, and MBS session creation, and they apply to both multicast and broadcast communications unless otherwise stated.

For multicast, MBS session establishment procedure triggered by UE join requests may follow the initial MBS session configuration procedure to reserve resources towards NG-RAN. For broadcast, the MBS session start procedure to reserve resources towards NG-RAN is triggered by the MBS session configuration procedure.

For both broadcast and multicast communication, the TMGI allocation may be separated from the MBS Session creation request.

For multicast communication, TMGI allocation procedure is applicable if TMGI is used as MBS Session ID.

Figure 7.1.1.2-1: Initial Configuration for MBS Session without PCC

Steps 1 to 6 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1.	AF sends Nnef_TMGI_Allocate Request (TMGI number, [MBS service area]) message to NEF/MBSF to request allocation of a TMGI(s) to identify new MBS session(s).

The MBS service area information can be cell ID list, MBS SAI list, geographical area information or civic address information. Among them, Cell ID list shall only be used by AFs who reside within trusted domain, and when the AFs are aware of such information.

NOTE 1:	Depending on the configuration, MB-SMF may receive requests from AF directly, or via NEF, or via MBSF, or via NEF and MBSF.

2.	NEF/MBSF checks authorization of AF. If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area into MBS SAI list.

NOTE 2:	NEF is not required if AF is in trusted domain.

3.	NEF/MBSF discovers and selects an MB-SMF using NRF or based on local configuration,

4.	NEF/MBSF sends an Nmbsmf_TMGI_Allocate Request (TMGI number) message to the MB-SMF.

5.	MB-SMF allocates TMGI(s) and returns the TMGI(s) to the NEF/MBSF via the Nmbsmf_TMGI_Allocate response (TMGI(s), expiration time).

6.	The NEF or MBSF responds to the AF by sending an Nnef_TMGI_Allocate Response (TMGI(s), expiration time, MBS service area).

If geographical area information or civic address information is provided by AF as MBS service area in Nnef_TMGI_Allocate Request in step 1, NEF/MBSF should include MBS SAI list as MBS service area in Nnef_TMGI_Allocate Response.

7.	The AF may perform a Service Announcement towards UEs. The AF informs UEs about MBS Session information with MBS Session ID, e.g., TMGI, source specific multicast address, and possibly other information e.g., MBS service area, session description information, etc.

The MBS service area information can be Cell ID list, MBS SAI list, geographical area information or civic address information. Amongst them, Cell ID list shall only be used by AFs who reside in trust domain, and when the AFs are aware of such information.

The UE needs to be aware if the service is broadcast or multicast to decide if JOIN is to be performed.

Editor's note:	How to do service announcements requires SA WG4 /WG6 coordination.

8.	AF of content provider may provide description for an MBS session (possibly providing information for a previously allocated TMGI to NEF via a Nnef_MBSSession_Create request (([MBS Session ID], service type, MBS information, [TMGI allocation indication])). If step 1-6 has not been executed before, the AF may provide a source specific multicast address or it may request that the network allocates an identifier for the MBS session (i.e., TMGI). The AF provides the service type (i.e. either multicast service or broadcast service). MBS information may further include QoS requirements, for a multicast session MBS authorization information (e.g., or an indication that any UE may join), MBS service area, start and end time of the MBS session and MBS session state (active/inactive). In addition, MBS information may also indicate whether the allocation of an ingress transport address is requested.

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to MBS SAI list.

Editor's note:	What other information is to be sent by AF is FFS.

9	NEF/MBSF checks authorization of content provider.

10.	NEF/MBSF discovers MB-SMF candidates and selects MB-SMF as ingress control node, possibly based on MBS service area. If a TMGI is included in step 8, NEF/MBSF finds MB-SMF based on that TMGI.

11.	NEF/MBSF sends Nmbsmf_MBSSession_Create Request (MBS Session ID, service type, TMGI allocation indication, MBS service area information, ingress transport address request indication) to MB-SMF, to request MB-SMF to reserve ingress resources for a MBS distribution session, The NEF/MBSF provides MBS Session ID or request allocation of a TMGI, and indicate the requested service type (either multicast service or broadcast service) and MBS session state (active/inactive). It also indicates that the allocation of an ingress transport address is requested if this was requested in step 8, or if the MBSF decides to insert an MBSTF into the user plane for the MBS session. If provided in step 8, the request includes the indication that any UE may join.

The MBS service area is provided by NEF/MBSF to the MB-SMF if provided by the AF in step 8.

12.	If requested to do so, or if a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF allocates a TMGI.

If a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF updates its NF profile at the NRF with the serving MBS Session ID. If an MBS service area information was received in step 11, the MB-SMF updates its NF profile at the NRF with that information.

NOTE 3:	If TMGI is used to represent an MBS Session, MB-SMF does not need to update NRF if the TMGI range(s) supported by an MB-SMF is already included in the MB-SMF profile when MB-SMF register itself into NRF.

13.	The MB-SMF derives the required QoS parameters locally.

14	MB-SMF selects the MB-UPF. If the allocation of an ingress transport address was requested in step 11, the MB-SMF requests the MB-UPF to reserve user plane ingress resources. If multicast transport of the MBS data towards RAN nodes is to be used, the MB-SMF also request the MB-UPF to reserve for the outgoing data a tunnel endpoint and the related identifiers (source IP address, source specific multicast address and GTP Tunnel ID) and to forward data received at the user plane ingress resource using that tunnel endpoint.

If the allocation of an ingress transport address was not requested in step 11, the MB-SMF provides the source specific multicast address received as MBS session ID to the MB-UPF and requests the MB-UPF to join the corresponding multicast tree from the content provider. The MB-SMF may also defer the configuration to join the corresponding multicast tree e.g. based on information that the session is inactive, QoS requirements and MBS start/end time until receiving the first query for the MBS session as part of the establishment procedure in clause 7.2.1.3, or until receiving a request to activate the MBS session via the MBS Session Update procedure in clause 7.1.1.6 or 7.1.1.7..

15.	If requested, MB-UPF selects an ingress address (IP address and port) and a tunnel endpoint for the outgoing data and provides it to MB-SMF.

16.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.1.

17.	MB-SMF indicates the possibly allocated ingress address to the NEF/MBSF. MB-SMF may include TMGI if it is allocated in step 9. It also indicates the success or failure of reserving transmission resources.

18.	[Optional] If the MBSF decides to use an MBSTF, the NEF/MBSF provides the ingress address received in step 18 towards the MBSTF as DL destination. If the allocation of an ingress transport address was requested in step 8, the MBSF requests the MBSTF to allocate the user plane ingress resources. If the allocation of an ingress transport address was not requested in step 8, the MBSF provides the source specific multicast address received as Multicast session ID in step 8 and requests the MBSTF to join the corresponding multicast tree from the content provider.

19.	[Conditional on step 19]	If requested, the MBSTF selects an ingress address (IP address and port) and provides it to NEF/MBSF.

20.	The NEF/MBSF-C indicates the possibly allocated ingress address and other parameters (e.g. TMGI) to the AF via an Nnef_MBSSession_Create response ([TMGI], [Allocated ingress address])). If MBS Session ID is not provided in step 8, or the MBS Session ID is source specific multicast address, the NEF/MBSF provides the allocated TMGI. If AF requests the allocation of an ingress transport address, the message also includes the allocated ingress address

21.	Same as step 7. The AF may also perform a service announcement at this stage.

22.	For multicast communication, depending on configuration UEs can join the MBS Session as specified in clause 7.2.1.

****************** NEXT CHANGES ***************

### 7.1.2	MB-SMF discovery and selection for multicast/broadcast session

To facilitate the MB-SMF discovery/selection for one multicast/broadcast MBS session, the following mechanism is used:

-	The MB-SMF registers its capability related to multicast/broadcast session management (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s), TMGI range, MBS SAI list) as part of its profile to the NRF by invoking Nnrf_NFManagement_NFRegister. In addition, when an multicast session is configured, and the MBS session ID is not yet included in the MB-SMF profile, the MB-SMF updates its profile towards the NRF with the MB Session ID (i.e. TMGI or source specific IP multicast address).

NOTE:	The operator can preconfigure MB-SMF for specific source IP multicast address range or TMGI range.

-	When the UE joins the multicast MBS session via PDU session modification procedures, the SMF serving the PDU session invokes the Nnrf_NFDiscovery_Request Request including the MBS session ID for multicast provided by the UE and optionally other information, i.e. the S-NSSAI and the associated NSI ID (if available), DNN, etc., to query the NRF for MB-SMF information. Based on the MBS session ID and other information for query, the NRF decides whether an MB-SMF serving the MBS session exists. If so, the NRF provides in Nnrf_NFDiscovery_Request Response the information of the MB-SMF currently serving the MBS session. The SMF serving the PDU session selects the MB-SMF currently serving the multicast MBS session, based on the MB-SMF information provided by the NRF. For local MBS services, the SMF takes MB-SMF service area, UE location into account when selecting the MB-SMF. If no MB-SMF serving the multicast session exists, the NRF provides MB-SMF profiles based on the other query information.

-	When the multicast MBS session context is deleted from the MB-SMF, e.g. due to MBS session release, the MB-SMF updates its profile towards the NRF, i.e., removing the MBS session ID which is no longer served by the MB-SMF.

-	During MBS session information provisioning procedures defined in clause 7.1.1.2, unless the MB-SMF information is available by other means, e.g. locally configured in the NEF/MBSF/AF, the NEF/MBSF/AF queries the NRF with information of the multicast/broadcast session (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s)), and selects the MB-SMF(s) based on the MB-SMF information provided by the NRF. For local MBS services, the NEF/MBSF/AF takes MB-SMF service area into account when selecting the MB-SMF(s).

### 7.1.3	MB-UPF discovery and selection for multicast/broadcast session

The selection and reselection of the MB-UPF are performed by the MB-SMF by considering MB-UPF deployment. For the local broadcast/multicast MBS session and location-dependent MBS session, the supported MBS SAI list is taken into consideration for MB-UPF selection.

MB-SMF may be locally configured with the information about the available MB-UPFs, e.g. by OA&M system when MB-UPF is instantiated or removed.

The MB-UPF selection functionality in the MB-SMF may optionally utilize the NRF to discover MB-UPF instance(s) which is similar with UPF selection with NRF defined in the TS 23.501 [5] clause 6.3.3.2.

****************** NEXT CHANGES ***************

##### 7.2.4.2.2	Configuration for local MBS

For local MBS, the configuration procedure for the UE is optional and performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same multicast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps civic address information or geographical area information for MBS service areas to MBS SAI list.

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the MBS Session ID, session area and Area Session ID.

NOTE:	For a location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session. If the MBS Session ID is TMGI, the MB-SMF updating NF profile can be skipped.

-	The policy of Multicast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The MBS service area(s) are indicated to the UE in the Service Announcement as defined in clause 6.11.

****************** NEXT CHANGES ***************

##### 7.2.4.3.2	Multicast session join and session establishment procedure for multicast service available within a limited area

For the case that the multicast service is only available within a limited area, the UEs outside the MBS service area are not allowed to join the multicast service.

The multicast session join and session establishment for multicast service available within a limited area are performed as specified in clause 7.2.1.3 with the following differences and clarifications:

-	The local multicast session has been configured where the AF provided the MBS service area information as specified in clause 7.2.4.3.1.

-	The UE may have information about local multicast service including MBS service area and local MBS service indication via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the MBS service area based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside the MBS service area, the UE does not send the Join Request.

-	In step 4, the SMF obtains the MBS service area (i.e. MBS SAI list and/or Cell ID list) of the indicated MBS session from the MB-SMF, and the SMF determines the indicated MBS session corresponds to local multicast service based on the MBS service area.

-	In step 5, the SMF checks whether the UE is inside or outside the MBS service area.

-	If the SMF detects that the UE is inside the MBS service area, the SMF continues the process of multicast session join as specified in clause 7.2.1.3 with the following additions:

-	The SMF sends the UE a PDU Session Modification Command indicating a Join Accept as a response to the Join Request. The Joint Accept includes the MBS service area (i.e. MBS SAI list and/or Cell ID list).

-	The multicast session information sent by the SMF to the NG-RAN includes the MBS service area (i.e. MBS SAI list and/or Cell ID list).

-	If the SMF detects that the UE is outside the MBS service area, the SMF rejects the multicast session join, so sends a Join Reject to the UE indicating that the MBS join is rejected. The Joint Reject includes the reject reason (outside of local service area) and the MBS service area (i.e. MBS SAI list and/or Cell ID list).

In this case, the MBS Session establishment (i.e. resources establishment for MBS traffic delivery) for the UE is not performed.

NOTE 1:	There may be the case that the UE determines that it is inside the MBS service area based on the geographical area information or civic address information as MBS service area information provided by the AF, so sends a Join Request but the UE is outside the MBS service area.

NOTE 2:	Which SM NAS message is used to deliver the Join Reject (e.g. PDU Session Modification Reject) is defined in stage 3 specifications.

-	If the Join Request from the UE is accepted and 5GC individual MBS traffic delivery is used, the SMF subscribes to the UE mobility event notification from the AMF (e.g. UE moving into or out of Area Of Interest, which is set by MBS service area), by invoking Namf_EventExposure_Subscribe service operation as specified in clause 5.2.2.3.2 of TS 23.502 [6].

-	For the UE that received the Join Reject from the SMF, later, if the UE detects that it is inside the MBS service area based on the MBS service area provided in the Join Reject, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group.

****************** NEXT CHANGES ***************

#### 9.4.2.2	Nnef_MBSTMGI_Allocation service operation

Service operation name: Nnef_ MBSTMGI_Allocation

Description: This service is used by the NF Service Consumer to request allocation of TMGI(s), or to refresh the expiry time for already allocated TMGI(s).

Inputs, Required: Number of TMGIs (may be zero if only a refresh of expiry time is requested).

Inputs, Optional: TMGI(s) to be refreshed, MBS service area (can be MBS SAI list and/or Cell ID list, or geographical area information, or civic address information).

Outputs, Required: TMGIs, Expiry Time, Success or not.

Outputs, Optional: MBS service area (i.e. MBS SAI list).

****************** NEXT CHANGES ***************

#### 9.4.3.2	Nnef_MBSSession_Create service operation

Service operation name: Nnef_MBSSession_Create

Description: Create a new multicast session or broadcast session. Optionally implicitly subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (source specific multicast address or TMGI) or TMGI request, Service Type (broadcast or multicast).

Input, Optional: MBS service area (mandatory for broadcast. Can be MBS SAI list and/or Cell ID list, or geographical area information, or civic address information), MBS start time, MBS stop time, service requirement (e.g. Flow Description, QoS), Input Transport Address Request, session activity status (active/inactive), for subscription to notifications notification target address, For a multicast session, indication that any UE may join the multicast session.

Output, Required: Result Indication.

Output, Optional: TMGI, Cause, MB-UPF tunnel info, MBS service area (i.e. MBS SAI list).

****************** END CHANGES ***************