3GPP TSG-WG SA2 Meeting #148E e-meeting 	S2-2108396

Elbonia, November 15 – 22, 2021	(Revision of S2-2107193 )

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0004 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Policy Control for MBS Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Ther are issues with the MBS PCC solution (see more discussin in S2-2108407): - The proposed PCC solution for MBS Session is not aligned with PDU Session’s PCC framework as intended - AF, NEF or MBSF need to split the request to create MBS Session into two steps (i.e. create + update), which adds complexity to NEF, MBSF or AF. - Unclear policy handling when MB-SMF first contact PCF when service requirement is not provided to PCF. Note that the service requirement from the AF is always on service data flow level or MBS QoS Flow level (instead of MBS Session level) - Such splitting may cause procedure failure at AF due to long delay of execution time. - NEF/MBSF/AF needs to determine whether to send MBS Session update to PCF or to MB-SMF or to both depending on the deployment of dynamic PCC and what parameter to update (i.e. QoS or MBS service area, or both). - Proposal “if no real policy update is required, the PCF repeats previous policies or sends an empty update message” is an inefficient and patch-like solution. To reduce complexity in both 5GC and AF, to avoid the inefficient&patch like solution, to avoid the unclarity on what policy rule will be provided by PCF when MB-SMF creates policy association, and to reduce the risk of procedure failure due to split procedures, it is proposed that MB-SMF interacts with PCF for policy control. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 7.1.1: Replace the PCC interactions with MB-SMF interaction with PCF, describe only the difference to the procedure without PCC. For MBS Session update with updated service requirement (e.g. QoS, packet filter), new policy trigger for MBS Resource Request is introduced. Clause 5.3.2.1, remove “The PCF can receive MBS information from AF, NEF or MBSF…” Clause 6.6, remove “Editor's note: The following description depends on the ongoing PCF discussion and needs revisit” Clause 6.10, Remove text related to MBS Session binding Clause 9.2, add service operation “Npcf_MBSPolicyControl_Update”, remove Npcf_MBSPolicyAuthorization Service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PCC solution may not work properly (e.g. EN not resolved, procedure failure due to splitting of the request message into 2). Unncessary complexity in 5GC (i.e. MB-SMF, MBSF and NEF) and in AF without clear benefit. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1, 6.6, 6.10, 7.1.1.3, 7.1.1.5, 7.1.1.7, 9.2.1, 9.2.2.2, 9.2.2.5 (new), 9.2.3 (void) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START OF CHANGES ***************

#### 5.3.2.1	PCF

In addition to the functions defined in TS 23.501 [5], the PCF performs the following functions to support MBS if dynamic PCC for 5MBS is needed:

-	Supporting QoS handling for MBS Session.

-	Providing policy information regarding the MBS Session to MB-SMF for authorizing the related QoS profiles.

-	Interacting with UDR for QoS information retrieval.

****************** NEXT CHANGES ***************

## 6.6	QoS Handling for Multicast and Broadcast services

For MBS services, the network shall support QoS control per MBS session.

The 5G QoS model and parameters as defined in TS 23.501 [5] clause 5.7 also apply to multicast/broadcast communication services with the following differences:

-	Reflective QoS is not applicable;

-	Wireline access network specific 5G QoS parameters do not apply to MBS services;

-	Alternative QoS Profile is not applicable;

-	QoS Notification Control is not applicable;

-	UE-AMBR is not applicable;

NOTE 1:	For multicast communication service, the UE-AMBR applies for associated PDU Session.

-	Session-AMBR if provided is enforced at MB-UPF but not communicated to NG-RAN.

NOTE 2:	Whether Session-AMBR is required in addition to the MBS service data flow bit rate is determined by operator policy and/or agreement with the service provider.

-	For broadcast MBS session, the QoS rule and QoS Flow level QoS parameters are not provided to UE.

NOTE 3: For broadcast MBS session, the associated QoS Flow(s) are not applicable.

Editor's Note: For multicast MBS Session, whether UE needs to be aware that a QoS Flow within the PDU Session is mapped from MBS QoS Flow is FFS.

-	For multicast MBS session, the handling of QoS rule and QoS Flow level QoS parameters of the associated QoS Flow(s) is the same as for other QoS Flow without UL in a PDU Session.

The network shall support one or multiple QoS flows, which can be either GBR or non-GBR, for an MBS session.

If 5GC Individual MBS traffic delivery method is used to deliver multicast data packets, the network may use dedicated QoS Flows for multicast data packets in a PDU session.

NOTE 4:	When there is a need to apply 5GC individual MBS traffic delivery, the Session-AMBR of the associated PDU Session can be configured with a sufficiently high value to cater for MBS Session-AMBR.

Editor´s Note: Whether any policy control at the SMF for individual delivery QoS flows is required is FFS.

The MB-SMF obtains QoS information for multicast and broadcast MBS session in different ways depending on the deployment and use cases.

If dynamic PCC is not deployed:

-	When an MBS session is started, the MB-SMF is provided with service requirements including QoS information. If MBSF is not used, the service requirement is provided to the MB-SMF by the AF (directly or via the NEF). If the MBSF is used, the MBSF receives request from the AF (or via the NEF) and decides the related QoS requirements (e.g. considering support for FEC) and provides them to the MB-SMF. The MB-SMF determines the QoS profiles and QoS for N4 rules for the MBS session with QoS parameters of the MBS QoS flows, and provides related information to the RAN and the MB-UPF respectively.

NOTE 5:	What information is included in the request from AF to MBSF requires collaboration with SA WG4.

If dynamic PCC is deployed:

-	It is the PCF that generates policy rules for MBS Session based on the received service requirement and provides the policy rules to the MB-SMF. The MB-SMF, based on the policy rules from the PCF, determines to create, and/or modify MBS QoS Flow(s) including providing QoS information to NG-RAN and MB-UPF, and providing packet detection and forwarding information to MB-UPF.

****************** NEXT CHANGES ***************

## 6.10	Policy control for Multicast and Broadcast services

The policy and charging control framework as defined in TS 23.503 [7] applies to Multicast and Broadcast services in the following aspects:

-	QoS Flow binding: For an MBS Session, QoS Flow binding is the association of a PCC rule to a QoS Flow within an MBS Session. The MB-SMF performs QoS Flow binding for an MBS Session in the same way as the SMF for a PDU Session.

-	PCC rules for MBS Session are used to provide policy for QoS flows: The following PCC rule parameters defined in Table 6.3.1 of TS 23.503 [7] are applicable for MBS:

-	Rule identifier.

-	Service data flow detection: Precedence, Service data flow template (only for IP PDU traffic).

-	5G QoS Identifier (5QI), DL-maximum bitrate, DL-guaranteed bitrate, ARP, Priority Level, Averaging Window, Maximum Data Burst Volume.

-	Policy information can also be applicable for an entire MBS session. The following parameters defined for a PDU session in Table 6.4.1 of TS 23.503 [7] are applicable for an entire MBS session:

-	Authorized Session-AMBR.

Editor's note:	It is FFS whether any other aspects or differences compared to existing PCC framework are required for MBS policy control.

****************** NEXT CHANGES ***************

#### 7.1.1.2	Initial MBS session configuration without PCC

This procedure is used by the AF to start the MBS Session towards 5GC and consist of TMGI allocation, and MBS session creation, and they apply to both multicast and broadcast communications unless otherwise stated.

For multicast, MBS session establishment procedure triggered by UE join requests may follow the initial MBS session configuration procedure to reserve resources towards NG-RAN. For broadcast, the MBS session start procedure to reserve resources towards NG-RAN is triggered by the MBS session configuration procedure.

For both broadcast and multicast communication, the TMGI allocation may be separated from the MBS Session creation request.

For multicast communication, TMGI allocation procedure is applicable if TMGI is used as MBS Session ID.

Figure 7.1.1.2-1: Initial Configuration for MBS Session without PCC

Steps 1 to 6 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1.	AF sends Nnef_TMGI_Allocate Request (TMGI number) message to NEF/MBSF to request allocation of a TMGI(s) to identify new MBS session(s).

NOTE 1:	Depending on the configuration, MB-SMF may receive requests from AF directly, or via NEF, or via MBSF, or via NEF and MBSF.

2.	NEF checks authorization of AF.

NOTE 2:	NEF is not required if AF is in trusted domain.

3.	NEF/MBSF discovers and selects an MB-SMF using NRF or based on local configuration,

4.	NEF/MBSF sends an Nmbsmf_TMGI_Allocate Request (TMGI number) message to the MB-SMF.

5.	MB-SMF allocates TMGI(s) and returns the TMGI(s) to the NEF/MBSF via the Nmbsmf_TMGI_Allocate response (TMGI(s), expiration time).

6.	The NEF or MBSF responds to the AF by sending an Nnef_TMGI_Allocate Response (TMGI(s), expiration time).

7.	The AF may perform a Service Announcement towards UEs. The AF informs UEs about MBS Session information with MBS Session ID, e.g., TMGI, source specific multicast address, and possibly other information e.g., MBS service area, session description information, etc.

The MBS service area information can be Cell ID list, TAI list, geographical area information or civic address information. Amongst them, Cell ID list and TAI list shall only be used by AFs who reside in trust domain, and when the AFs are aware of such information.

The UE needs to be aware if the service is broadcast or multicast to decide if JOIN is to be performed.

Editor's note:	How to do service announcements requires SA WG4 /WG6 coordination.

8.	AF of content provider may provide description of an for an MBS session (possibly providing information for a previously allocated TMGI to NEF via a Nnef_MBSSession_Create request (([MBS Session ID], service type, MBS information, [TMGI allocation indication]). If step 1-6 has not been executed before, the AF may provide a source specific multicast address or it may request that the network allocates an identifier for the MBS session (i.e., TMGI). The AF provides the service type (i.e. either multicast service or broadcast service). MBS information may further include QoS requirements, for a multicast session MBS authorization information (e.g., or an indication that any UE may join), MBS service area, start and end time of the MBS session and MBS session state (active/inactive). In addition, MBS information may also indicate whether the allocation of an ingress transport address is requested.

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to Cell ID list or TAI list.

Editor's note:	What other information is to be sent by AF is FFS.

9	NEF/MBSF checks authorization of content provider.

10.	NEF/MBSF discovers MB-SMF candidates and selects MB-SMF as ingress control node, possibly based on MBS service area. If a TMGI is included in step 8, NEF/MBSF finds MB-SMF based on that TMGI.

11.	NEF/MBSF sends Nmbsmf_MBSSession_Create Request (MBS Session ID, service type, TMGI allocation indication, MBS service area information, ingress transport address request indication) to MB-SMF, to request MB-SMF to reserve ingress resources for a MBS distribution session, The NEF/MBSF provides MBS Session ID or request allocation of a TMGI, and indicate the requested service type (either multicast service or broadcast service) and MBS session state (active/inactive). It also indicates that the allocation of an ingress transport address is requested if this was requested in step 8, or if the MBSF decides to insert an MBSTF into the user plane for the MBS session. If provided in step 8, the request includes the indication that any UE may join.

The MBS service area is provided by NEF/MBSF to the MB-SMF if provided by the AF in step 8.

12.	If requested to do so, or if a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF allocates a TMGI.

If a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF updates its NF profile at the NRF with the serving MBS Session ID. If an MBS service area information was received in step 11, the MB-SMF updates its NF profile at the NRF with that information.

NOTE 3:	If TMGI is used to represent an MBS Session, MB-SMF does not need to update NRF if the TMGI range(s) supported by an MB-SMF is already included in the MB-SMF profile when MB-SMF register itself into NRF.

13.	The MB-SMF derives the required QoS parameters locally.

14	MB-SMF selects the MB-UPF. If the allocation of an ingress transport address was requested in step 11, the MB-SMF requests the MB-UPF to reserve user plane ingress resources. If multicast transport of the MBS data towards RAN nodes is to be used, the MB-SMF also request the MB-UPF to reserve for the outgoing data a tunnel endpoint and the related identifiers (source IP address, source specific multicast address and GTP Tunnel ID) and to forward data received at the user plane ingress resource using that tunnel endpoint.

If the allocation of an ingress transport address was not requested in step 11, the MB-SMF provides the source specific multicast address received as MBS session ID to the MB-UPF and requests the MB-UPF to join the corresponding multicast tree from the content provider. The MB-SMF may also defer the configuration to join the corresponding multicast tree e.g. based on information that the session is inactive, QoS requirements and MBS start/end time until receiving the first query for the MBS session as part of the establishment procedure in clause 7.2.1.3, or until receiving a request to activate the MBS session via the MBS Session Update procedure in clause 7.1.1.6 or 7.1.1.7..

15.	If requested, MB-UPF selects an ingress address (IP address and port) and a tunnel endpoint for the outgoing data and provides it to MB-SMF.

16.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.1.

17.	MB-SMF indicates the possibly allocated ingress address to the NEF/MBSF. MB-SMF may include TMGI if it is allocated in step 9. It also indicates the success or failure of reserving transmission resources.

18.	[Optional] If the MBSF decides to use an MBSTF, the NEF/MBSF provides the ingress address received in step 18 towards the MBSTF as DL destination. If the allocation of an ingress transport address was requested in step 8, the MBSF requests the MBSTF to allocate the user plane ingress resources. If the allocation of an ingress transport address was not requested in step 8, the MBSF provides the source specific multicast address received as Multicast session ID in step 8 and requests the MBSTF to join the corresponding multicast tree from the content provider.

19.	[Conditional on step 19]	If requested, the MBSTF selects an ingress address (IP address and port) and provides it to NEF/MBSF.

20.	The NEF/MBSF-C indicates the possibly allocated ingress address and other parameters (e.g. TMGI) to the AF via an Nnef_MBSSession_Create response ([TMGI], [Allocated ingress address])). If MBS Session ID is not provided in step 8, or the MBS Session ID is source specific multicast address, the NEF/MBSF provides the allocated TMGI. If AF requests the allocation of an ingress transport address, the message also includes the allocated ingress address

21.	Same as step 7. The AF may also perform a service announcement at this stage.

22.	For multicast communication, depending on configuration UEs can join the MBS Session as specified in clause 7.2.1.

#### 7.1.1.3	Initial MBS session configuration with PCC

Figure 7.1.1.3-1: Configuration for MBS Session with PCC

1.	Same as steps 1-11 in clause 7.1.1.2.

2. [Optional] The MB-SMF sends Npcf_MBSPolicyControl_Create Request to PCF with the MBS Session ID, service information (e.g. QoS information, packet filters), and PCF derives the policy rules for the MBS service data flow.

3. [Optional] The PCF provide policy rules in Npcf_MBSPolicyControl_Create Response. The MB-SMF determines, based on the policy rule(s), the QoS profile(s) of the MBS QoS Flow(s) and derives the packet detection rule used to detect the MBS packet over N6mb.

4. Same as steps 12-22 in clause 7.1.1.2.

****************** NEXT CHANGE ***************

#### 7.1.1.4	Removal of MBS session configuration without PCC

This procedure is used by the AF to release the MBS Session. This procedure may also include TMGI de-allocation. The procedures apply to both multicast and broadcast communications unless otherwise stated. This procedure releases the reserved resources in both 5GC and NG-RAN.

Figure 7.1.1.4-1: Removal of MBS Session configuration without PCC

1.	AF of content provider may request to release the MBS session (MBS Session ID).

2/3.	If an MBSTF was inserted into the user plane, the MBSF request the MBSTF to release user plane resources.

4.	NEF/MBSF requests MB-SMF to release resources for the MBS session.

5.	For broadcast session, the MB-SMF triggers resource release towards the AMFs as specified in clause 7.3.2. For multicast session, the MB-SMF triggers resource release towards the SMFs as specified in clause 7.2.2.3.

6/7.	MB-SMF requests the MB-UPF to release user plane resources.

8.	[Conditional] If MB-SMF configured the profile with an MBS session ID when the MBS session was configured, the MB-SMF updates its NF profile at NRF to release the MBS Session ID.

9.	MB-SMF responds to the NEF/MBSF.

10.	The NEF/MBSF responds to the AF.

11.	[Optional] AF requests NEF/MBSF to de-allocate TMGI(s),

12.	[Conditional on step 11] NEF/MBSF forwards request to de-allocate TMGI(s) to MB-SMF.

13.	[Conditional on step 12] The MB-SMF responds to the NEF or MBSF by sending a de-allocate TMGI Response message.

14.	[Conditional on step 13] NEF or MBSF forwards de-allocate TMGI Response message to AF.

#### 7.1.1.5	Removal of MBS session configuration with PCC

This procedure is used by the AF to release the MBS Session. This procedure may also include TMGI de-allocation. The procedures apply to both multicast and broadcast communications unless otherwise stated. This procedure releases the reserved resources in both 5GC and NG-RAN.

Figure 7.1.1.5-1: Removal of configuration for MBS Session with PCC

1.		Same as steps 1-4 in clause 7.1.1.4.

2. The MB-SMF sends Npcf_MBSPolicyControl_Delete request to PCF

3. The PCF responds with Npcf_MBSPolicyControl_Delete response.

4. Same as steps 5-14 in clause 7.1.1.4

****************** NEXT CHANGE ***************

#### 7.1.1.6	MBS Session Update without PCC

This procedure is used by the AF to update the MB service area and/or update QoS of an MBS Session. Updating QoS of an MBS Session may lead to addition of new MBS QoS Flow(s). The procedure applies to both multicast and broadcast communications unless otherwise stated.

Figure 7.1.1.6-1: Update of MBS Session without PCC

1.	AF of content provider initiates MBS Session Update to a NEF/MBSF, e.g. to update MBS service area and/or update service requirement, or to activate or deactivate an MBS session. AF may provide updated information for an MBS session (identified by MBS session ID) by sending MBS Session update request ([MBS Session ID], MBS information, AF Identifier). MBS information may include service requirements, MBS service area information, and media information.  The service requirements adjustment may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s).

2.	NEF checks authorization of AF.

3.	NEF/MBSF forward the MBS Session Update request to MB-SMF.

4.	The MB-SMF derives any updated QoS parameters locally.

5-6.MB-SMF may need to update MB-UPF, e.g. if new MBS QoS Flow is to be created, or existing MBS QoS Flow is to be deleted.

7.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.3. For multicast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.2.5 (for service activation/deactivation), 7.2.6 (for QoS updates) and 7.2.x (for service area updates).

8.	If an MBS service area is being updated, the MB-SMF stores the new service area in its profile at the NRF.

9.	MB-SMF responds to the MBS Session Update.

10.	NEF/MBSF responds to the MBS Session Update.

#### 7.1.1.7	MBS session configuration Update with PCC

Figure 7.1.1.7-1: MBS session configuration update with PCC

1.	Same as steps 1-3 in clause 7.1.1.6.

For MBS service area update, and/or MBS session activation/deactivation which is only applicable for multicast MBS Session, steps 2-3 does not apply.

2. If the MBS Session update involves update of service requirement (e.g. QoS, packet filters), the MB-SMF performs Npcf_MBSPolicyControl_Update with policy trigger of MBS Resource Request.

3. The PCF derives upated policy rules based on the updated service requirements. The updated policy rules are sent to the MB-SMF which may result in creation of new MBS QoS Flow(s), and update and/or deletion of existing MBS QoS Flow(s).

4.	Same as steps 5-10 in clause 7.1.1.6.

****************** NEXT CHANGE ***************

## 9.2	PCF Services

### 9.2.1	General

The following table illustrates the PCF Services for MBS.

Table 9.2.1-1: NF services provided by PCF for MBS

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Npcf_MBSPolicyControl | Create | Request/Response | MB-SMF |
|  | Update | Request/Response | MB-SMF |
|  | UpdateNotify | Subscribe/Notify | MB-SMF |
|  | Delete | Request/Response | MB-SMF |

### 9.2.2	Npcf_MBSPolicyControl service

#### 9.2.2.1	General

Service description: NF Service Consumer, e.g. MB-SMF can create and manage a MBS Policy Association in the PCF through which the NF Service Consumer receives policy information for a MBS Session.

As part of this service, the PCF may provide the NF Service Consumer, e.g. MB-SMF with policy information about the MBS Session that may contain:

-	MBS Session related policy information.

-	PCC rule information.

-	Policy Control Request Trigger information. When a Policy Control Request Trigger condition is met the NF Service Consumer, e.g. MB_SMF shall contact the PCF and provide information on the Policy Control Request Trigger condition that has been met.

Editor's note:	Details of MBS Session related policy information, PCC rule information, and Policy Control Request Trigger information need to be defined in an appropriate location of the TS and referenced.

At MBS Session establishment the NF Service Consumer, e.g. MB-SMF requests the creation of a corresponding MBS Policy Association with the PCF (Npcf_MBSSMPolicyControl_Create) and provides relevant parameters about the MBS Session to the PCF.

-	When the PCF has created the "MBS Policy Association", the PCF may provide policy information as defined above.

When a Policy Control Request Trigger condition is met the NF Service Consumer, e.g. MB-SMF requests the update (Npcf_MBSPolicyControl_Update) of the MBS Policy Association by providing information on the condition(s) that have been met. The PCF may provide updated policy information to the NF Service Consumer.

The PCF may at any time provide updated policy information (Npcf_MBSPolicyControl_UpdateNotify).

At MBS Session Release the NF Service Consumer, e.g. MB_SMF requests the deletion of the corresponding MBS Policy Association.

Editor's note:	Parameters of the service operations are FFS.

****************** NEXT CHANGE ***************

#### 9.2.2.2	Npcf_MBSPolicyControl_Create service operation

Service operation name: Npcf_MBSPolicyControl_Create

Description: The NF Service Consumer can request the creation of a MBS Policy Association and provide relevant parameters about the MBS Session to the PCF.

Inputs, Required: MBS session ID, DNN, S-NSSAI, service information.

Inputs, Optional: None.

Outputs, Required: Success or Failure. For Success MBS Policy Association ID.

Outputs, Optional: Policy information for the MBS Session.

****************** NEXT CHANGE ***************

#### 9.2.2.5	Npcf_MBSPolicyControl_Update service operation

Service operation name: Npcf_MBSPolicyControl_Update

Description: The NF Service Consumer can request the update of an MBS Policy Association and provide the updated service information of MBS Session to the PCF.

Inputs, Required: MBS session ID, service information.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: Policy information for the MBS Session.

****************** NEXT CHANGE ***************

### 9.2.3	void

****************** END OF CHANGES ***************