SA WG2 Meeting #148E	 S2-2108405

November 15 – 22, 2021, E-Meeting

# Title:	[DRAFT] LS on AF/MBSF handling MBS Session when dynamic PCC is deployed

# Response to:	-

# Release:	Rel-17

# Work Item:	5MBS

Source:	[Ericsson, will be] SA2

To:	SA4, SA6

Cc:	CT3

Contact Person:

Name:	Judy Gan Juying

Tel. Number:

E-mail Address:	Judy.Gan.Juying@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

- Overall Description:

SA2 would like to inform SA4, SA6 and CT3 WGs of the following:

SA2 has specified that dynamic PCC may be deployed for MBS Session handling, see clause 7.1.1.3, 7.1.1.5 and 7.1.1.7 of TS 23.247 v17.0.0.

When dynamic PCC is deployed, if the AF is in trusted domain and if the Transport Only Mode is applied,

- For MBS Session creation, the AF is required to split the MBS Session creation procedure into two procedures:

  - First the AF interacts with MB-SMF without providing service requirement (e.g., QoS, packet filters) in Nmbsmf_MBSSession_Create request, and

  - Then the AF interacts with PCF providing the service requirement in Npcf_MBSPolicyAuthorization_Create request

- For MBS Session update, the AF is required to determine whether to update MBS Service Area, or update service requirement, or update both:

  - If the AF updates only MBS service area, the AF interacts with MB-SMF in Nmbsmf_MBSSession_Update request

  - If the AF updates only service requirement, the AF interacts with PCF in Npcf_MBSPolicyAuthorization_Update.

  - If the AF updates both MBS service area and service requirement, the AF needs to perform the MBS Session update in two procedures, i.e.

    - AF interacts with MB-SMF in Nmbsmf_MBSSession_Update and

    - AF interacts with PCF in Npcf_MBSPolicyAuthorization_Update

In the above, if the Service Mode (instead of Transport Only Mode) is applied, then “AF” in the MBS Session creation procedure and MBS Session update procedure above will be replaced by “MBSF”, that is, the MBSF needs to act differently, depending on the availability of dynamic PCC.

SA2 kindly asks SA6 and SA4 whether there is any comment or feedback on the above.

2. Actions:

To SA4, SA6:

ACTION: 	SA2 kindly asks SA6 and SA4 to provide feedback, if needed.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #149E		February 14 – 25, 2022			Elbonia