3GPP TSG-WG SA2 Meeting #148E 	S2-2108472

# E-meeting, November 15 – 22, 2021	(revision of S2-2107118)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3241 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network slice admission control for SNPN onboarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, MediaTek Inc, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN, eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CT1 asked in LS S2-2107028/ C1-215159 whether the network slice admission control needs to be performed for slice(s) used for onboarding or not. SA2 needs to clarify this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that network slice admission control is optionally, depending on operator policies, applied to slice(s) used for onboarding and emergency services and 24.501 describes how the UE associates the rejected S-NSSAI with the usage of SNPN Onboarding or emergency service as to avoid that the UE makes same request again. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear whether the network slice admission control needs to be performed for slice(s) used for onboarding or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes * * * *

#### 5.15.11.0	General

The Network Slice Admission Control Function (NSACF) monitors and controls the number of registered UEs per network slice and the number of PDU Sessions per network slice for the network slices that are subject to Network Slice Admission Control (NSAC). The NSACF is also configured with the information indicating which access type is specified for the S-NSSAI subject to NSAC (i.e. 3GPP Access Type, Non-3GPP Access Type, or both).

The NSACF also provides event based Network Slice status notifications and reports to the consumer NF (e.g. AF).

Multiple NSACFs may be deployed in a network. One NSACF may be responsible for one or more S-NSSAIs in a service area. Within a service area, one S-NSSAI is handled by one NSACF, which can be one NSACF instance or one NSACF Set. A PLMN may have one or more service areas.

Each NSACF is configured with the maximum number of UEs per network slice and the maximum number of PDU Sessions per network slice which are allowed to be served by each network slice that is subject to NSAC.

Subject to operator policy and national/regional regulations, the AMF may exempt from NSAC the network slices that the AMF has determined to include Emergency, Critical and Priority services (e.g. MCS, MPS). When the Registration Type indicates Emergency Registration or the establishment cause is associated with priority services (e.g. MPS, MCS), for the S-NSSAI to be subject to NSAC, or has been included in the Allowed NSSAI and is subject to NSAC, if the S-NSSAI is included in the Emergency Configuration Data, the AMF decides that the S-NSSAI may be exempt from NSAC. When the emergency PDU Session is requested to be established or a priority header is received from the AMF during PDU Session establishment, if the S-NSSAI is subject to NSAC, the SMF decides that NSAC may be skipped for the S-NSSAI. When NSAC is exempted for the S-NSSAI, the AMF and the SMF skip the corresponding NSAC procedure for the S-NSSAI, i.e. this UE (respectively PDU Session) is not counted towards the maximum number of UEs (respectively PDU Sessions).

The support of NSAC for the S-NSSAI used for SNPN Onboarding as described in clause 5.30.2.10 is optional and subject to Onboarding Network operator policies.

If the S-NSSAI used for SNPN Onboarding or Emergency services is rejected due to NSAC, the UE associates the rejected S-NSSAI for NSAC to SNPN Onboarding or to Emergency services as described in TS 24.501 [47].

# * * * * End of changes * * * *