3GPP TSG-SA WG2 Meeting #148E (e-meeting)	S2-2108488

# November 15 – 22, 2021, Elbonia										     (revision of S2-21xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0045 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TAI announcement procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2021-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Relay TAI announcement procedure is needed to follow up the agreement made in SA2#147E with S2-2107974. That is, TAI can be provided by the Layer-3 UE-to-Network Relay to the Layer-3 Remote UE by using the Relay Discovery Additional Information message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add TAI announcement procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How Layer-3 UE-to-Network Relay announces its TAI is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1.2.2, 6.5.1.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.5.1.2.2	N3IWF selection procedure for 5G ProSe Layer-3 Remote UE

When the 5G ProSe Layer-3 Remote UE relays traffic over 5G ProSe Layer-3 UE-to-Network Relay that supports N3IWF, the 5G ProSe Layer-3 Remote UE selects the N3IWF using the N3IWF selection procedure that is specified in clause 6.3.6.2 of TS 23.501 [4] for untrusted non-3GPP access with the following differences.

To support the N3IWF selection for 5G ProSe Layer-3 Remote UE, a 5G ProSe Layer-3 Remote UE is configured by HPLMN with N3IWF identifier configuration for 5G ProSe Layer-3 Remote UE and 5G ProSe Layer-3 UE-to-Network Relay access node selection information as described in clause 5.1.4.1.

When the 5G ProSe Layer-3 Remote UE decides to select an N3IWF in the HPLMN, the 5G ProSe Layer-3 Remote UE uses the N3IWF identifier configuration for 5G ProSe Layer-3 Remote UE, if configured, to find the IP address of the N3IWF in the HPLMN. Otherwise, 5G ProSe Layer-3 Remote UE constructs N3IWF FQDN based on either the Tracking Area Identity FQDN or on Operator Identifier FQDN of the 5G ProSe Layer-3 UE-to-Network Relay node selection information.

To assist the 5G ProSe Layer-3 Remote UE with N3IWF selection, the 5G ProSe Layer-3 UE-to-Network Relay supporting N3IWF access advertises the 5GS TAI corresponding to the serving cell, as defined in clause 5.8.3.2, in the TAI announcement procedure as defined in clause 6.5.1.X.

A 5G ProSe Layer-3 Remote UE constructs the FQDN using either Tracking Area Identity FQDN or on Operator Identifier FQDN and selects the N3IWF using the procedures of N3IWF selection in clause 6.3.6.2 of TS 23.501 [4].

* * * * Start of Next Change * * * *

6.5.1.X	TAI announcement procedure

TAI announcement procedure outlined in Figure 6.5.1.X-1 allows an authorized 5G ProSe Layer-3 Remote UE to request a 5G ProSe Layer-3 UE-to-Network Relay to announce the TAI of the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay. The announced TAI may be used by the 5G ProSe Layer-3 Remote UE to select a N3IWF.

Figure 6.5.1.X-1: TAI announcement procedure

1.	5G ProSe Layer-3 Remote UE requires the TAI of the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay. The 5G ProSe Layer-3 Remote UE may need the TAI of the 5G ProSe Layer-3 UE-to-Network Relay to select a N3IWF, e.g. after the 5G ProSe Layer-3 Remote UE selects a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support and establishes a connection for unicast mode communication as specified in clause 6.5.1.2.1.

2.	The 5G ProSe Layer-3 Remote UE sends to the 5G ProSe Layer-3 UE-to-Network Relay a TAI Announcement Request to obtain the TAI of the 5G ProSe Layer-3 UE-to-Network Relay's serving cell.

3.	The 5G ProSe Layer-3 UE-to-Network Relay acknowledges receipt of the request in step 2 with a TAI Announcement Response.

4.	The 5G ProSe Layer-3 UE-to-Network Relay announces the TAI of the serving cell by sending Relay Discovery Additional Information message as defined in clause 5.8.3.2.

NOTE:	The 5G ProSe Layer-3 UE-to-Network Relay can send the Relay Discovery Additional Information message more than one-time if there is other 5G ProSe Layer-3 Remote UE that has connected to the 5G ProSe Layer-3 UE-to-Network Relay but not yet requested to announce the TAI to the 5G ProSe Layer-3 UE-to-Network Relay. This can make the other 5G ProSe Layer-3 Remote UE obtain the TAI of the 5G ProSe Layer-3 UE-to-Network Relay without requesting it.

* * * * End of Changes * * * *