3GPP TSG-SA WG2 Meeting #148E (e-meeting)	S2-2108490

# November 15 – 22, 2021, Elbonia										     (revision of S2-21xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0054 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on local multicast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | S2-2108006 (TS 23.247 CR#0012Rev1) agreed at SA2#147E updates Figure 7.2.1.3-1 and adds multicast join via PDU Session Establishment Request to clause 7.2.1.3. These updates can be reflected to some related description in clause 7.2.4.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 7.2.4.3.2: - Add description about Join Request via PDU Session Establishment Request and Join Accept via PDU Session Establishment Accept. - Fix step# from step 4 to step 3 about interaction between SMF and MB-SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description about local multicast is not aligned with the Multicast session join procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

7.2.4.3.2	Multicast session join and session establishment procedure for multicast service available within a limited area

For the case that the multicast service is only available within a limited area, the UEs outside the MBS service area are not allowed to join the multicast service.

The multicast session join and session establishment for multicast service available within a limited area are performed as specified in clause 7.2.1.3 with the following differences and clarifications:

-	The local multicast session has been configured where the AF provided the MBS service area information as specified in clause 7.2.4.3.1.

-	The UE may have information about local multicast service including MBS service area and local MBS service indication via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the MBS service area based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) or the PDU Session Establishment Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside the MBS service area, the UE does not send the Join Request.

-	In step 3, the SMF obtains the MBS service area (i.e. Cell ID list or TAI list) of the indicated MBS session from the MB-SMF, and the SMF determines the indicated MBS session corresponds to local multicast service based on the MBS service area.

-	In step 5, the SMF checks whether the UE is inside or outside the MBS service area.

-	If the SMF detects that the UE is inside the MBS service area, the SMF continues the process of multicast session join as specified in clause 7.2.1.3 with the following additions:

-	The SMF sends the UE a PDU Session Modification Command or a PDU Session Establishment Accept indicating a Join Accept as a response to the Join Request. The Joint Accept includes the MBS service area (i.e. Cell ID list or TAI list).

-	The multicast session information sent by the SMF to the NG-RAN includes the MBS service area (i.e. Cell ID list or TAI list).

-	If the SMF detects that the UE is outside the MBS service area, the SMF rejects the multicast session join, so sends a Join Reject to the UE indicating that the MBS join is rejected. The Joint Reject includes the reject reason (outside of local service area) and the MBS service area (i.e. Cell ID list or TAI list).

In this case, the MBS Session establishment (i.e. resources establishment for MBS traffic delivery) for the UE is not performed.

NOTE 1:	There may be the case that the UE determines that it is inside the MBS service area based on the geographical area information or civic address information as MBS service area information provided by the AF, so sends a Join Request but the UE is outside the MBS service area.

NOTE 2:	Which SM NAS message is used to deliver the Join Reject (e.g. PDU Session Modification Reject) is defined in stage 3 specifications.

-	If the Join Request from the UE is accepted and 5GC individual MBS traffic delivery is used, the SMF subscribes to the UE mobility event notification from the AMF (e.g. UE moving into or out of Area Of Interest, which is set by MBS service area), by invoking Namf_EventExposure_Subscribe service operation as specified in clause 5.2.2.3.2 of TS 23.502 [6].

-	For the UE that received the Join Reject from the SMF, later, if the UE detects that it is inside the MBS service area based on the MBS service area provided in the Join Reject, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group.

* * * * End of Changes * * * *