3GPP TSG-SA WG2 Meeting #148E (e-meeting)	S2-2108496

15 - 22 November 2021, Elbonia	(revision of)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3381 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of TAI indication in a ULI for NR Satellite Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2021-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A solution for including a TAI in the ULI over N2 for NR satellite access, where one or multiple TACs may be indicated per PLMN in a serving radio cell, has been evaluated as Option C+D in S2-210xxxx. This has the following definition: Option C+D A gNB includes a TAI in the ULI for the TA within which the UE is geographically located. When a TAI cannot be determined (e.g. no UE location information), a gNB includes a reserved value indicating the TAI is not available. The gNB also includes all TACs broadcast in the serving radio cell. Changes to TS 23.501 are needed to support this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add impacts to support the Option C+D solution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to support UE Registration for NR satellite access in an unambigous consistent manner, which will prevent operation of NR satellite access in Release 17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 5.4.11.X	User Location Information over N2

An NG RAN node shall indicate the TAI in which a UE is located, when available, in the User Location Information passed over N2 to the AMF. An NG RAN node shall also include in the User Location Information the TACs for the RPLMN that are currently indicated in the serving radio cell.

NOTE:	When the TAI in which a UE is located cannot be determined by NG-RAN, a PLMN operator can configure a reserved TAI value to indicate to AMF that the TAI of the UE cannot be determined.

**** End of Changes ****