3GPP TSG-SA WG2 Meeting #148E	S2-2108509

# E-Meeting, 15th – 22nd November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0057 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multicast MBS service for SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | -description on multicast MBS service for SNPN is missing. -UE operation for UE join is unclear when the appropriate PDU session is not established yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Multicast service is applied to SNPN as well as PLMN. - When the appropriate PDU session is not established yet, for UE join we may establish a PDN session establishment including UE join request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multicast service in SNPN is not valid |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1.2, 7.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******Start of 1st change*******

7.2.1.2	Establishment of a PDU Session that can be associated with multicast session(s)

The PDU Session associated with multicast session(s) (i.e., the associated PDU Session) is established using the procedures as specified in TS 23.502 [6] clause 4.3.2.2 with the following differences:

NOTE 1:	The DNN and S-NSSAI are used to establish the PDU session which can carry the Multicast operation, i.e. join/leave, and can be associated with multicast MBS session(s).

-	In step 2, based on the DNN and S-NSSAI use for establishing a PDU Session associated with multicast session(s), the AMF selects an SMF capable of handling multicast sessions based on DNN and S-NSSAI, locally configured data or a corresponding SMF profile stored in the NRF. For indirect discovery, the AMF requests the SCP to select an SMF capable of handling multicast sessions.

-	In step 4, if MBS subscription data for the UE (i.e. corresponding SUPI), DNN and S-NSSAI of the HPLMN or subscribed SNPN is not available, the SMF retrieves the MBS subscription data using Nudm_SDM_Get (SUPI, MBS subscription data, selected DNN, S-NSSAI of the HPLMN or subscribed SNPN, Serving PLMN ID(or PLMN ID and NID)) and subscribes to be notified when this subscription data is modified using Nudm_SDM_Subscribe (SUPI, MBS subscription data, selected DNN, S-NSSAI of the HPLMN or SNPN, Serving PLMN ID(or PLMN ID andNID)). UDM may get this information from UDR by Nudr_DM_Query (SUPI, MBS data, selected DNN, S-NSSAI of the HPLMN or subscribed SNPN, Serving PLMN ID(or PLMN ID and NID)) and may subscribe to notifications from UDR for the same data by Nudr_DM_subscribe. The MBS subscription data can also be retrieved along with the Session Management Subscription data, i.e. with additional input parameter for MBS subscription data in the Nudm_SDM services.

NOTE 2:	In this release, the roaming is not supported, i.e. HPLMN and Serving PLMN(or subscribed SNPN and serving SNPN) is same.

7.2.1.3	Multicast session join and session establishment procedure

The following steps are executed before the UE requests to join the MBS session:

-	The MBS Session may have has been configured in the 5GC (see clause 7.1.1 for details).

-	The UE registers in the PLMN or SNPN and establishes a PDU session.

-	The UE has known at least the MBS Session ID of a multicast group that the UE can join, e.g. via service announcement.

Figure 7.2.1.3-1: PDU Session modification for UE joining multicast session

- To join the multicast group, the UE sends a PDU Session Modification Request which additionally contains one or several MBS Session ID(s) and join request. The MBS Session ID(s) indicate the multicast group(s) that UE wants to join.

For example, if the UE has no appropriate PDU session for the MBS session ID(s) yet satisfying e.g. DNN and S-NSSAI etc., the UE may join the multicast MBS session by sending PDU Session Establishment Request for associated PDU session together with one or several MBS Session ID(s) and join request. In that case, before step 2, the network proceeds with establishment of the associated PDU session executing steps 4 to10 of PDU Session Establishment procedure as specified in TS 23.502 [6] clause 4.3.2.2.

2.	[Conditional] Based on the received MBS Session ID and join request, the SMF determines this is MBS Session join request. The SMF checks whether the user is authorized to use the required multicast service.

3.	[Conditional] If SMF has no information about MBS Session context for the indicated MBS Session ID(s), SMF discovers and selects an MB-SMF for the MBS Session via the NRF as described in clause 7.1.2. If no MB-SMF is assigned for the MBS session ID, the SMF may select an MB-SMF and request it configure the multicast session or the SMF may reject the join request.

NOTE 1:	Details how the SMF select an MB-SMF and request it configure the multicast session are left to SMF implementation.

4.	Nmbsmf_ MBSSession_ContextStatusSubscribe request indicates the SMF want to subscribe the MBS session context. For each MBS session in step 1, by using Nmbsmf_ MBSSession_ContextStatusSubscribe request (MBS Session ID) with the immediately reporting flag. SMF interacts with the MB-SMF to retrieve information about the indicated multicast session context information (multicast QoS flow information (e.g., QoS profile(s) for multicast MBS session), [start time], [session status indication (active/inactive)], [MBS session authorization information (MBS session open for any user)],, LL MC address]) and to subscribe to events notifications related to the multicast session.

If it is the first time for the MB-SMF to receive Nmbsmf_ MBSSession_ContextStatusSubscribe request of the indicated MBS Session from SMF, MB-SMF learns it is the first UE joining the multicast group. For multicast transport between MB-UPF and content provider, if it is the first UE joining the multicast group, and MB-UPF has not joined the multicast tree in the MBS configuration procedure, the MB-SMF requests the MB-UPF to join the multicast tree towards the AF/MBSF, otherwise MB-SMF will not send the request to the MB-UPF. The MB-SMF determines whether the user is authorized to join the multicast session as follows: The MB-SMF checks the user subscription data received from the UDM to determine whether the user is allowed to use any multicast service. If so, the MB-SMF checks the received indication whether the multicast session is open for any user. If the multicast session is not open to any user, the MB-SMF checks the user subscription data received from the UDM to determine whether the MBS session ID is included. If a UE joins prior to the start time of the multicast session, the SMF may accept the join request in step 8 and indicate to the UE the start time, or it may reject the join request with appropriate error cause and possible back-off time. If a UE joins while the multicast session is inactive, the SMF accepts the join request in step 8. If authorization check fails, the SMF indicates cause value in the PDU Session Modification Reject sent to the UE and proceeds with step 8.

NOTE 2:	The MB-SMF can answer the Nmbsmf_ MBSSession_ContextStatusSubscribe request either based on information received in the configuration procedures in Clause 7.1.1 or based on preconfigured information. The pre-configuration also includes information about the MBS session stored in the NRF. If the MB-SMF uses preconfigured information, the pre-configuration also include MB-UPF configuration.

7.	SMF responds to AMF through Nsmf_PDUSession_UpdateSMContext response (N2 SM information (PDU Session ID, MBS Session ID, [updated PDU Session information], [mapping information between unicast QoS flow(s) and multicast QoS flow (s)]), N1 SM container (PDU Session Modification Command)) to:

-	create an MBS session context for the indicated MBS session in the RAN, if it does not exist in the RAN already; and

-	inform the NG-RAN about the relation between the multicast context and the UE's PDU Session context by including the MBS session ID and the mapping between the multicast QoS flow(s) and associated QoS flow(s).

Based on operator policy, the SMF may prepare for 5GC individual MBS traffic delivery fall-back. The SMF maps the received QoS information of the multicast QoS Flow into PDU Session's unicast QoS Flow information, and includes the information of the QoS Flows and the mapping information about the QoS Flows in the SM information sent to RAN.

NOTE 3:	Detailed information included in N2 SM information will be aligned with by RAN WG3.

NOTE 4:	A PDU Session UP activation is not triggered by message 7 if it only includes information related to the multicast session and associated QoS flows and is received by an MBS capable NG RAN node.

Editor's Note: The implication of not triggering PDU Session UP activation in NG-RAN when SMF informs the NG-RAN of UE join requires RAN collaboration.

Editor's note:	Possible PCF interactions related to the multicast QoS flows are FFS.

8.	The N2 message, which includes the multicast session information and PDU session modification information is sent to the NG-RAN.

If the 5G MBS is not supported by NG-RAN, 5GC individual MBS traffic delivery may be used. Otherwise if the MBS is supported by NG-RAN, 5GC shared MBS traffic delivery is adopted.

If the NG-RAN supports 5G MBS, the NG-RAN uses the MBS Session ID to determine that the PDU Session identified by the PDU Session ID is associated with the indicated multicast session.

If the multicast QoS information is received and the NG-RAN supports MBS, the associated unicast QoS flow information is not used to allocate the radio resource and CN resource.

NOTE 5:	It is NG-RAN that decides whether radio resource is allocated or not, and it is NG-RAN/UPF that decides whether multicast transport or unicast transport is used between the NG-RAN/UPF and the MB-UPF.

9. [Conditional]	If shared tunnel has not been established for the MBS session towards the NG-RAN node, the procedures in clause 7.2.1.4 for the Establishment of shared delivery toward RAN node are executed. Step 9 is executed separately for each MBS session.

10.	The NG-RAN node performs AN specific signalling exchange with the UE to establish radio resource for the MBS session if not established yet. If the NG-RAN does not support MBS, radio resource are reconfigured for unicast transmission of the MBS data over the associated PDU session. As part of the AN specific signalling exchange, the N1 SM container (PDU Session Modification Command) is provided to the UE.

11.	The NG-RAN node sends the PDU session modification response.

If the MBS is not supported by NG-RAN, the accepted unicast QoS flow is included in the N2 SM response container. If the MBS is supported by NG-RAN, the N2 SM response container includes the accepted multicast QoS flow.

12.	The AMF invokes Nsmf_PDUSession_UpdateSMContext request ([N2 SM container]) to the SMF.

Per the accepted unicast QoS flow information, the SMF determines the delivery mode, i.e. whether 5GC individual MBS traffic delivery is used for multicast data transmission.

NOTE 6:	If the shared tunnel is used, no interaction with UPF is needed for the indicated MBS session

[Conditional] Step 13 is used for 5GC Individual MBS traffic delivery, if the related NG-RAN does not support multicast .If a shared tunnel between the UPF (PSA) and MB-UPF for 5GC individual MBS traffic delivery has not yet been established by the SMF for the multicast session, steps 13a to 13e are executed. Step 13f is executed irrespective of that.

13a.	The SMF contacts the UPF to request the creation of a tunnel and provides the MBS session ID. The UPF indicates to the SMF whether the tunnel for this MBS session is newly allocated (as there can be multiple SMFs interacting with the same UPF for the same MBS Session). If the UPF determines to use unicast transport over N19mb, the UPF allocates a DL N19mb Tunnel endpoint for the MBS session if the SMF request is the first one to allocate DL N19mb Tunnel endpoint for the MBS Session in the UPF. The UPF includes the DL Tunnel Info in the response to the SMF. The DL tunnel info includes the downlink tunnel ID and the UPF address. The UPF is now ready for receiving the MBS data from the MB-UPF and forwarding the data to the PDU session.

Editor's Notes: whether 13b to 13e can be skipped for multicast N10mb is FFS

13b.	If the UPF indicates the DL N19mb Tunnel is newly allocated, the SMF invokes Nmbsmf_MBSsession_ ContextUpdate request (MBS Session ID, [DL tunnel info]) towards MB-SMF that includes MBS Session ID and downlink tunnel info of UPF, for establishing the multicast session transport between MB-UPF and UPF.

13c.	If the DL tunnel info of the UPF is received, MB-SMF configures MB-UPF to transmit the multicast session data towards UPF using the possibly received downlink tunnel ID.

13d.	MB-SMF responds to SMF through Nmbsmf_MBSSession_ ContextUpdate response (MBS Session ID, [multicast DL tunnel info]). If the UPF DL tunnel info for unicast transport is not received by the MB-SMF, multicast transport between MB-UPF and UPF is to be used, and the SMF includes the downlink tunnel information with the transport multicast address for the multicast session.

13e.	For multicast transport between MB-UPF and UPF, SMF configures UPF to receive the multicast session data. If multicast transport over N19mb is used, the UPF joins the source specific multicast group of MB-UPF. The UPF forwards the data to the PDU session.

13f.	The MB-SMF configures the MB-UPF to forward the received multicast session data within the PDU session. (This step may be combined with step 13a or 13e).

14.	The SMF invokes Nsmf_PDUSession_UpdateSMContext response to the AMF.

15.	MB-UPF receives multicast PDUs, either directly from the content provider or via the MBSTF that can manipulate the data.

Steps 16 to 18 are for 5GC shared MBS traffic delivery:

16.	MB-UPF sends multicast PDUs in the N3mb tunnel associated to the multicast session to the NG-RAN. There is only one tunnel per multicast session and NG-RAN node, i.e., all the UEs which have joined the multicast session via the NG-RAN node share this tunnel for reception of the multicast session data.

17.	The NG-RAN selects PTM or PTP radio bearers to deliver the multicast PDUs to the UE(s) that have joined the multicast session.

18.	The NG-RAN transmits the multicast session data to the UE(s)  using the selected PTM or PTP radio bearer(s).

Steps 19 to 21 are for 5GC individual MBS traffic delivery:

19.	MB-UPF sends multicast PDUs in the N19mb tunnel associated to the multicast session to UPF. There is only one tunnel per multicast session and destination UPF, i.e., all associated PDU sessions share this tunnel.

20.	UPF forwards the multicast data towards the NG-RAN via unicast (i.e. in the N3 tunnel of the associated PDU Session).

21.	The NG-RAN forwards the multicast session data to the UE via unicast (i.e. over the radio bearer(s) corresponding to the mapped unicast QoS flow(s) of the associated PDU Session).

NOTE 7:	Details of the DL MBS data transmission are described in clause 6.7.

NOTE 8:	When the MBSF is involved in the multicast MBS session, the tunnel between MBSTF and MB-UPF has been established in the configuration procedure.

*******End of changes*******