3GPP TSG-SA2 Meeting #148E (e-meeting)	S2-2108522

# Elbonia, November 15 - 22, 2021                                                            (was S2-2107974)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0019 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Relay Discovery Additional Information message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In rev1: (1) In the SA2#146E meeting, it was agreed to support the Cell ID announcement procedure, in which the 5G ProSe Layer-3 UE-to-Network Relay announces the NCGI of the serving cell by sending Relay Discovery Additional Information message. This NCGI parameter along with other parameters included in Relay Discovery Additional Information message need to be clarified in cluase 5.8.3.2. (2) Since Relay TAI is used for N3IWF selection (instead of relay discovery) by Layer-3 Remote UE, it is proposed to include Relay TAI in Relay Discovery Additional Information message. (3) The descriptions about Relay Discovery Additional Information message in clauses 6.3.2.3.1, 6.3.2.3.2 and 6.5.1.3 need to be aligned. In rev2: It has been discussed that TAI of Layer-3 Relay (and potentially TAI in case of Layer-2 Relay) included in the RDAI message can be requested by Remote UE using a dedicated PC5-S message. It is proposed to extend cell ID annoucement procedure to accommodate more parameters, e.g. TAI of Layer-3 Relay. Source and Destination Layer-2 IDs for RDAI message need to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In rev1: (1) Add NCGI, Relay TAI and other parameters for Relay Discovery Additional Information message in clause 5.8.3.2. (2) Align the descriptions about Relay Discovery Additional Information message in clause 6.3.2.3.1, 6.3.2.3.2 and 6.5.1.3. In rev2: Extend cell ID annoucement procedure to accommodate more parameters, e.g. TAI of Layer-3 Relay. Source and Destination Layer-2 IDs for RDAI message are clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned descriptions between different clauses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.8.3.2, 6.3.2.3.1, 6.3.2.3.2, 6.5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** FIRST CHANGE ***************

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5G DDNMF	5G Direct Discovery Name Management Function

AS layer	Access Stratum layer

NCGI	NR Cell Global ID

PDUID	ProSe Discovery UE ID

PFI	PC5 QoS Flow Identifier

PQI	PC5 5QI

ProSe	Proximity based Services

RPAUID	Restricted ProSe Application User ID

RSC	Relay Service Code

RSD	Rule Selection Descriptor

TAI	Tracking Area Identity

****************** SECOND CHANGE ***************

5.8.3.2	Identifiers for 5G ProSe Layer-3 UE-to-Network Relay

For 5G ProSe Layer-3 UE-to-Network relay, a Relay Service Code in the Announcement Message is associated with a set of PDU session parameters (e.g. PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference). The Relay Service Code may also represent if the relay UE can provide secure N3IWF connection.

For 5G ProSe Layer-3 Remote UE discovering 5G ProSe Layer-3 UE-to-Network relay, the Relay Service Code in the Solicitation Message represents the PDU session parameters that a PDU session of the relay should be able to support. The Relay Service Code may also represent if the remote UE requires secure N3IWF connection.

The following parameters may be used in the Relay Discovery Additional Information message (for Model A) for 5G ProSe Layer-3 UE-to-Network Relay where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message, and the other parameters are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1.

-	Relay Service Code: the Relay Service Code associated with the message. The Relay Service Code is used to identify the security parameters needed by the receiving UE to process the discovery message.

-	Announcer Info: provides information about the announcing user.

-	NCGI: indicates the NCGI of the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay. This parameter is used in Cell ID announcement procedure as described in clause 6.5.1.3.

-	TAI: indicates the Tracking Area Identity of the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay. This parameter may be used by 5G ProSe Layer-3 Remote UE to select a N3IWF.

****************** THIRD CHANGE ***************

6.3.2.3.1	General

5G ProSe UE-to-Network Relay Discovery is applicable to both 5G ProSe Layer-3 and Layer-2 UE-to-Network relay discovery for public safety use and commercial services. To perform 5G ProSe UE-to-Network Relay Discovery, the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay are pre-configured or provisioned with the related information as described in clause 5.1.

In 5G ProSe UE-to-Network Relay Discovery, the UEs use pre-configured or provisioned information for the relay discovery procedures as defined in clause 5.1.4.1.

The Relay Service Code (RSC) is used in the 5G ProSe UE-to-Network Relay discovery, to indicate the connectivity service the 5G ProSe UE-to-Network Relay provides to the 5G ProSe Remote UE. The RSCs are configured on the 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE as defined in clause 5.1.4. The 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE are aware of whether a RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service based the policy as specified in clause 5.1.4. A 5G ProSe UE-to-Network Relay supporting multiple RSCs can advertise the RSCs using multiple discovery messages, with one RSC per discovery message.

Additional information (e.g. NCGI of the serving cell) not directly used for discovery can also be advertised using the PC5-D protocol stack in single or separate discovery messages of type "Relay Discovery Additional Information".

Both Model A and Model B discovery are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

For Relay Discovery Additional Information, only Model A discovery is used.

****************** FOURTH CHANGE ***************

6.3.2.3.2	Procedure for 5G ProSe UE-to-Network Relay Discovery with Model A

Depicted in Figure 6.3.2.3.2-1 is the procedure for 5G ProSe UE-to-Network Discovery with Model A.

Figure 6.3.2.3.2-1: 5G ProSe UE-to-Network Relay Discovery with Model A

1.	The 5G ProSe UE-to-Network Relay sends a UE-to-Network Relay Discovery Announcement message. The UE-to-Network Relay Discovery Announcement message contains the Type of Discovery Message, Announcer Info and RSC, and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3.

For 5G ProSe Layer-3 UE-to-Network Relay, the 5G ProSe Layer-3 UE-to-Network Relay shall only include a RSC in the UE-to-Network Relay Discovery Announcement when the S-NSSAI associated with that RSC belongs to the Allowed NSSAI of the UE-to-Network Relay.

The 5G ProSe Remote UE (1 to 3) determines the Destination Layer-2 ID for signalling reception. The Destination Layer-2 ID is configured with the UE(s) as specified in clause 5.1.4.1.

5G ProSe Remote UE (1 to 3) monitors announcement messages with the 5G ProSe UE-to-Network RSC corresponding to the desired services.

Optionally, the 5G ProSe UE-to-Network Relay may also send Relay Discovery Additional Information messages. The parameters contained in this message and the Source Layer-2 ID and Destination Layer-2 ID used for sending and receiving the message are described in clause 5.8.3.2.

The 5G ProSe Remote UE selects the 5G ProSe UE-to-Network Relay based on the information received in step 1.

NOTE:	Access Stratum layer information used for 5G ProSe UE-to-Network Relay selection is specified in RAN specifications.

****************** FIFTH CHANGE (almost all changes are new in rev2) ***************

6.5.1.3	Additional parameters announcement procedure

Additional parameters announcement procedure outlined in figure 6.5.1.3-1 is used by a 5G ProSe Remote UE to request a 5G ProSe UE-to-Network Relay to announce some additional parameters, e.g. it allows an authorized 5G ProSe Layer-3 remote UE, based on its application specific requirements outside the scope of the present document, to request a 5G ProSe Layer-3 UE-to-Network Relay to announce the NR Cell Global ID (NCGI) of the Cell serving the 5G ProSe Layer-3 UE-to-Network Relay, or it allows a 5G ProSe Layer-3 Remote UE to request a 5G ProSe Layer-3 UE-to-Network Relay to announce the Tracking Area Identity (TAI) of the the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay.

Figure 6.5.1.3-1: Additional parameters announcement procedure

1)	5G ProSe Remote UE has discovered a 5G ProSe UE-to-Network Relay.

An application on 5G ProSe Layer-3 Remote UE may require serving cell NCGI reporting even when it is accessing via 5G ProSe Layer-3 UE-to-Network Relay (i.e. the application benefits from this value even if it is the value of the cell serving of the 5G Layer-3 ProSe UE-to-Network Relay), or 5G ProSe Layer-3 Remot UE may require TAI of the the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay for N3IWF selection.

2)	The 5G ProSe Remote UE sends to the 5G ProSe UE-to-Network Relay an Additional Parameters Announcement Request to obtain some additional parameters.

The 5G ProSe Layer-3 Remote UE may request the NCGI of the 5G ProSe Layer-3 UE-to-Network Relay's serving cell and/or TAI of the the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay.

3)	The 5G ProSe UE-to-Network Relay acknowledges receipt of the request in step 2 with an Additional Parameters Announcement Response (Additional Parameters_Announcement_Request_Refresh Timer). The Additional Parameters_Announcement_Request_Refresh Timer (configurable in the 5G ProSe UE-to-Network Relay), is provided to the 5G ProSe Remote UE so that when this timer expires the 5G ProSe Remote UE repeats the Additional Parameters Announcement Request procedure if it still needs obtain the additional parameters. If the 5G ProSe Remote UE does not initiate new Additionl Parameters Announcement Request procedure when this Additional Parameters_Announcement_Request_Refresh Timer expires and no other UE request additional parameters announcement before the Additional Parameters_Announcement_Request_Refresh timer expires in the 5G ProSe UE-to-Network relay, then the relay shall stop announcing the addtional parameters.

4)	The 5G ProSe UE-to-Network Relay announces the additional parameters by sending Relay Discovery Additional Information Message as defined in clause 5.8.3.2. This is repeated periodically with a configurable frequency (normally Higher than the one related to the Additional Parameters_Announcement_Request_Refresh Timer) until there is no UE requesting to announce the additional parameters as determined by the Additional Parameters_Announcement_Request_Refresh Timer running in the 5G ProSe UE-to-Network Relay.

The 5G ProSe Layer-3 UE-to-Network Relay may announce the NCGI of the 5G ProSe Layer-3 UE-to-Network Relay's serving cell and/or TAI of the the serving cell of the 5G ProSe Layer-3 UE-to-Network Relay based the request in step 2.

5)	The 5G ProSe UE-to-Network Relay detects new additional parameters.

The 5G ProSe Layer-3 UE-to-Network Relay may detect new NCGI of a new serving cell it is happening to be camping on, or new TAI as it is moving to a new tracking area.

6)	Detection of additional parameters in Step 5 triggers the 5G ProSe UE-to-Network Relay to announce the additional parameters by sending a Relay Discovery Additional Information Message immediately and to repeat it periodically with a configurable frequency as in step 4 until there are no UEs requesting to announce the additional parameters, i.e. until the Additional Parameters_Announcement_Request_Refresh Timer expires in the 5G ProSe UE-to-Network Relay.

****************** END OF CHANGES ***************