3GPP TSG-SA WG2 Meeting #148E (e-meeting)	S2-2108564

# November 15 – November 22, 2021, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR |  | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for UUAA-SM procedure when UUAA-MM performed |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If UUAA-MM is performed, Figure 5.2.3.2-1 of TS 23.256 specifies at step 1, “If a UUAA has been performed at Registration there is no need for the USS to perform UUAA at PDU Session establishment and steps 1 to 5 is not performed.” In this case, if UUAA-MM performed, USS have no reference to trigger step 6 to subscribe PDU session status event. So we can consider that skipping only step 3. Although USS know that the UAV has been Authenticated already, AMF can initiate re-authentication as specified in Clause 5.2.2.1, so the USS need to differentiate that whether this request is for re-authentication request from AMF or not. For this, it is needed to inform to the USS that the authentication "request is not for authentication". USS can decide to skip step3 based on this indication and maintain authentication correlation ID for UUAA re-authentication with AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that UUAA-SM procedure when UUAA-MM performed. - Introduce an indication that "request is not for authentication". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | USS cannot subscribe PDU session status event if UUAA-MM is performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.2.3.2	USS UAV Authorization/Authentication (UUAA) during the PDU Session Establishment

The USS UAV Authorization/Authentication (UUAA) is triggered by the SMF during the PDU Session Establishment, specified in TS 23.502 [3], clause 4.3.2.2 and additionally based on the SM subscription data obtained from UDM, and the Service Level Device Identity provided by the UE in the PDU Session establishment request.

Figure 5.2.3.2 -1: UUAA during PDU Session Establishment

The procedure assumes that the UE/UAV has already registered on the AMF.

0.	Steps 1 - 5 as in TS 23.502 [3] figure 4.3.2.2.1-1.

The SMF determines that it needs to invoke UAS NF/NEF service operation for UUAA Authentication/Authorization of the PDU session establishment request based on the provided DNN/S-NSSAI.

The UAV includes the Service Level Device Identity (e.g. the CAA-Level UAV ID of the UVA) and may include the Authentication Server Address (i.e. the USS address) and optionally Authentication Data (i.e. the UUAA Aviation Payload) in the PDU Session Establishment request.

The SMF identifies the UAS NF/NEF based on local configuration or using UE provided identity e.g. USS address.

1.	The SMF invokes Nnef_Authentication_Authenticate service operation, including the Service Level Device Identity (that contains the CAA-Level UAV ID of the UAV), DNN, S-NSSAI, and may include the Authentication Server Address (i.e. the USS address) and the Authentication Data (i.e. the UUAA Aviation Payload) if it was provided by the UE, GPSI, optionally UAV location, PEI if available, and the UE IP Address if available. The UAV location is the User Location Information provided by the AMF (e.g. Cell ID). The UAS NF/NEF selects a USS based on either the Service Level Device Identity (i.e. CAA-Level UAV ID of the UAV) or the Authentication Server address (i.e. USS address) as described in clause 4.4.2.

2.	From UAS NF/NEF to USS: Naf_Authentication_Authenticate_service operation forwarding the authentication request received information from the SMF. UAS NF may translate the Cell ID received as part of UAV location in the Nnef_Authentication_Authenticate request at step 1 into a corresponding geographic area and/or may further obtain the UE location information using Location Service Procedures as defined in TS 23.273 [8] and include them in the Naf_Authentication_Authenticate message towards the USS e.g. to support geo-caging functionality.

If a UUAA has been performed at Registration and the authentication request is received from the SMF, UAS NF/NEF includes an indication that "request is not for authentication" in the Naf_Authentication_Authenticate message.

3.	[Conditional] Multiple round-trip messages as required by the authentication method used by USS. N33_Authentication_Authenticate response messages from USS shall include GPSI and may include a authentication message based on authentication method used that is forwarded transparently to UE over NAS MM transport messages. The authentication message in step3e may contain UUAA Aviation Payload required by the USS if it was not provided by the UE before.

If a UUAA has been performed at Registration there is no need for the USS to perform UUAA at PDU Session establishment. Based on the indication that "request is not for authentication", the USS can decide not to perform UUAA, thus step 3 is not performed.

4.	From USS to NEF: Naf_Authentication_Authenticate response.

The USS sends Naf_Authentication_Authenticate response to the UAS NF/NEF with the Authentication/Authorization result containing the UUAA result, optionally a Service Level Device Identity containing the new CAA-Level UAV ID, requested policy information, an Authorization Data (i.e. the UUAA Authorization Payload). The requested policy information from USS may contain a DN Authorization Profile Index and/or a DN authorized Session AMBR.

NOTE 1:	The USS stores a mapping between CAA-Level UAV ID and the External Identifier (i.e. GPSI as defined in clause 4.5.3). The External Identifier (GPSI) and/or UAV IP Address can be used at a later point by the USS for accessing various services exposed by 3GPP network e.g. location information retrieval, monitoring event configuration, requesting dedicated policies for e.g. C2, etc.

The External Identifier and/or UAV IP Address can be used at a later point by the USS for requesting dedicated policies for e.g. C2, etc.

5.	The UAS NF/NEF confirms the successful Authentication/Authorization of the PDU Session. The UAS NF/NEF stores the UUAA result together with the GPSI. UAS NF/NEF forwards the Authentication/Authorization result, a Service Level Device Identity containing the new CAA-Level UAV ID, if received from the USS, and the Authorization Data (i.e. the UUAA Authorization Payload), if received from the USS, to the SMF. If the authentication/authorization is successful, the SMF shall subscribe for notifications from UAS NF/NEF which may be used to trigger re-authentication, update authorization data or revoke authorization of the UAV, upon receipt of such request from the USS.

6.	If the authentication/authorization is successful, the USS shall subscribe to the PDU Session Status Event as described in steps 1-5 in Figure 4.15.3.2.3-1 of TS 23.502 [3]. This step can be executed in parallel to step 4. The UAS NF/NEF shall use the DNN, S-NSSAI received from the SMF in step 1 to subscribe to the PDU Session Status Event notification.

7.	The PDU Session establishment continues with steps 7 to 21 and completes. In the step 7b in Figure 4.3.2.2.1-1of TS 23.502 [3], if the SMF receives the DN Authorization Profile Index from the UAS NF/NEF, it sends the DN Authorization Profile Index to retrieve the PDU Session related policy information (described in clause 6.4 of TS 23.503 [9]) and the PCC rule(s) (described in clause 6.3 of TS 23.503 [9]) from the PCF. If the SMF receives the DN authorized Session AMBR in from the UAS NF/NEF, it sends the DN authorized Session AMBR within the Session AMBR to the PCF to retrieve the authorized Session AMBR (described in clause 6.4 of TS 23.503 [9]).

The SMF transfers the Authentication/Authorization result, the Service Level Device Identity containing the new CAA-Level UAV ID and the Authorization Data (i.e. the UUAA Authorization Payload) to the UAV as in steps 11, 12 and 13 in figure 4.3.2.2.1-1 of TS 23.502 [3].

8.	If the USS in step 6 subscribed to the PDU Session Status Event the SMF will, as described in steps 6-7 in Figure 4.15.3.2.3-1 of TS 23.502 [3], detect when the PDU Session is established, and send the PDU Session Establishment event report to the UAS NF/NEF by means of Nsmf_EventExposure_Notify message, including GPSI and the UE IP Address. Then, the UAS-NF/NEF forwards the event message to the USS.

NOTE 2:	If C2 information reference is available from USS during the initial PDU Session Establishment procedure the SMF can interact with the PCF to set up a predefined PCC rule(s) profile for the C2 communication.

* * * * End of Changes * * * *