3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108572

# Elbonia, November 15 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3395 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | clarification for SNPN access using CH credentials and NSAC for onboarding slice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Clarify the Network Slice Admission Control for S-NSSAI used for onboarding based on the reply LS. 2. SNPN should terminate the PDU session within SNPN itself in the case of access to SNPN using credentials from CH. In this case, the SNPN AMF should disregard the bullet f) of clause 6.3.2 and always select the SMF in SNPN for the PDU session |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Calrify that Network Slice Admission Control maybe supported for S-NSSAI used for onboarding based on operator policies in the onbarding network. - Clarify that the AMF always selects the SMF in the SNPN regardless of Subscription information from UDM, i.e. bullet f) of clause 6.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Not clear how SNPN AMF treats the Subscription information from UDM when selecting the SMF in case of UE access using credentials from CH - Relationship for Network Slice Admission Control and S-NSSAI used for onboarding is unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.9.1, 5.30.2.10.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.30.2.9.1	General

SNPNs may support UE access using credentials owned by a Credentials Holder separate from the SNPN. In this case the Session Management procedures (i.e. PDU Sessions) terminate in an SMF in the SNPN.

NOTE:	During SMF discovery and selection, subscription information from UDM, i.e. bullet f) of clause 6.3.2 is not applicable.

When an SNPN supports UE access using credentials assigned by a Credentials Holder separate from the SNPN, it is assumed that is supported is homogeneously within the whole SNPN.

Credentials Holder using AAA Server for primary authentication and authorization is described in clause 5.30.2.9.2 and Credentials Holder using AUSF and UDM for primary authentication and authorization is described in clause 5.30.2.9.3.

# * * * * Second change * * * *

5.30.2.10.4	Remote Provisioning of UEs in Onboarding Network

5.30.2.10.4.1	General

Editor's note:	This clause should introduce the following clauses.

The support of Network Slice Admission Control as described in clause 5.15.11 for S-NSSAI used for onboarding is optional and subject to Onboarding Network operator policies.

# * * * * End of changes * * * *