SA WG2 Meeting #S2-148E	S2-2108575

15 - 22 November, 2021, Electronic meeting

Source:	Saankhya Labs, Ligado Networks, ONE Media 3.0, Fraunhofer IIS, CEWiT, Tejas Networks, IIT Bombay, IIT Kanpur, IIT Madras, IIT Hyderabad, IIT Kharagpur

Title:	Study on Interworking of Non-3GPP Multicast and Broadcast Networks with 3GPP 5G System

Document for:	Approval

Agenda Item:	9.1.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Interworking of Non-3GPP Multicast and Broadcast Networks with 3GPP 5G System

Acronym:

Unique identifier:

Potential target Release:	Rel-18

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X | X | X |  |
| No |  |  |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

Dependency on non-3GPP (draft) specification:

# 3	Justification

The proliferation of mobile broadband networks is increasingly making the devices like smartphones as the preferred choice for content consumption. However, with the growth in content consumption, especially video consumption, via cellular networks, the load on the available cellular spectrum has also increased, leading to congestion in cellular networks.

The non-3GPP satellite networks (non-3GPP NTN) and non-3GPP Digital Terrestrial Broadcasting Technology networks (non-3GPP DTT) might help in reducing the load on the cellular spectrum/network if a portion of the content delivery (video data) is offloaded onto them. This may efficiently be done through interworking or convergence of non-3GPP NTN and DTT networks with 5GS.

Advantages of using Non-3GPP Satellite Broadcasting Technologies for Multicast/Broadcast delivery in 5GS

- In 3GPP, the usage of NTN is already being considered as a means of providing coverage enhancement for terrestrial networks. They are being proposed to be used both for access as well as backhaul connectivity.

- Services such as Firmware-Over-The-Air (FOTA) might be delivered very efficiently via satellite broadcast due to the pervasive coverage provided by them. The satellite-based delivery may be cost-effective too. Apparently, there is no plan in 3GPP as yet to issue standards for satellite multicast/broadcast for services like FOTA until Release 3GPP 18.

- Non-3GPP satellite multicast/broadcast systems (both open-standard and proprietary) have existed for some time, and it may not be necessary to wait for a 3GPP satellite broadcast system to be standardized in NTN before other 5G 3GPP systems can roam to them. 3GPP can play an enabling role in the process (of standardization) by standardizing the roaming interfaces while remaining agnostic towards the air interface.

Advantages of using Non-3GPP Digital Terrestrial Broadcasting Technologies

- They also provide support for large coverage areas, which might be helpful for service delivery in rural areas and sparsely populated areas.

- Most Non-3GPP DTTs provide high spectral efficiency supporting higher-order modulation and coding schemes, e.g., Advanced Television Systems Committee 3.0 (ATSC3) standard supports 1024-QAM.

- Improved mobility managing due to support for large coverage areas and SFNs

Convergence of Cellular and Non-3GPP Digital Terrestrial Broadcast Network /Non-3GPP Satellite Broadcasting Technologies would

- Reduce load on Cellular Network through efficient offloading

- Enable creation of ‘best connected’ hybrid broadcast network. Higher throughput payload could be broadcast over terrestrial networks, and lower throughput, but critical payloads broadcast over the satellite network. The Converged Core architecture of the 5G system can be used for simpler dynamic interworking with non-3GPP NTN/DTT networks just as it has been achieved with Wi-Fi and Wireline access networks (via functions like N3IWF and W-AGF). Extending the interworking to non-3GPP NTN/DTT is likely to unleash benefits through newer services which have not been realized so far, similar to the converged services such as ATSSS or multiple access support achieved with Wi-Fi and Wireline access in the unicast domain.

- Enhance the device ecosystems of both cellular and broadcast networks by leveraging the scales of cellular devices

- Improve resource utilization across non-3GPP NTN/DTT and 3GPP 5G NG-RAN

- Open up new business opportunities through the interfacing of 5GS with the legacy and new broadcasting networks. For example, ATSC3 is deployed in the USA, Korea & DVB-T2 is deployed in 144 countries worldwide. There can be improved monetization of these networks through convergence with 5GS.

- The 5G system architecture has already been defined with a converged core network integrating non-3GPP access networks such as WLANs and wireline access for unicast communication. Therefore, it may be appropriate to extend this capability to wide-area multicast/broadcast communication, including satellite networks, and integrate the non-3GPP multicast and broadcast networks within the 5G system.

The 5GC Integration with non-3GPP DTT/NTN for Broadcast will help in providing improved services via the following representative scenarios.

  - Efficient offload of several unicast sessions in 3GPP NG-RAN to Broadcast session in non-3GPP DTT/NTN network based on the “best connected” hybrid network.

  - Enables Broadcast Capacity on-demand update as per the traffic needs and Flexible capacity allocation by making use of non-3GPP DTT/NTN network.

  - Enables on-demand Coverage improvement for Broadcast mode delivery of OTT content/TV services by making use of non-3GPP DTT/NTN network

  - They are ensuring Coverage (and service continuity) of Broadcast Channels when a user moves outside the coverage area of the 3GPP NR/MBS network.

In all the above scenarios, seamlessly managing the sessions between 3GPP and non-3GPP networks can be done with a 5GC Integration.

The following figure provides a high-level view of the interworking architecture as envisioned by the proponents. It proposes to utilize the concept of converged core in the 5G system. The non-3GPP NTN and the non-3GPP DTT networks can use a new interworking function (non-3GPP Broadcast IWF) and the N2 and N3mb interfaces to interwork with the 5G core. The N2 and the N3mb interfaces used here are expected to be similar to the one used between NG-RAN and the 5G Core for multicast/broadcast service delivery, as far as possible. A UE has the capability to receive content via NG-RAN as well as non-3GPP NTN/DTT networks.

It should be noted that it is only an early view of the proposed architecture, and the study phase is expected to come up with the final interworking architecture.

# 4	Objective

This study item aims to investigate the interworking of non-3GPP satellite networks and non-3GPP Digital Terrestrial Broadcasting Technologies with 5GS. Specifically, the study item would undertake the following (work) tasks:

- Identification of “key technical issues” for interworking between non-3GPP NTN/DTT and 5GS and their resolution

  - In order to utilize the non-3GPP NTN/DTT for multicast/broadcast delivery, a multicast broadcast session in a 5G system needs to be created. WT_#1 would study if it were possible to support unified Multicast/Broadcast (MB) Session Management across 3GPP NG-RAN and non-3GPP NTN/DTT networks? Would it be possible to switch the delivery of an MB session from NG-RAN based delivery method (i.e., shared and/or individual MBS traffic delivery via NG-RAN) to shared MBS traffic delivery via non-3GPP NTN/DTT network and vice versa?

  - How does the 5GC determine if the data to an individual UE can be delivered via the non-3GPP NTN/DTT network?

    - Does the UE report the non-3GPP NTN/DTT networks to the 5GC? or

    - Does the UE send the measurement reports for the non-3GPP NTN/DTT to 5G RAN? or

    - It may also be based on UE’s location.

  - WT_#1 would also study, how the mobility of a UE impacts the MBS data delivery to the UE?

  - WT_#1 would also study if it is possible to support unified Multicast/Broadcast session management across non-3GPP access (e.g., WLAN) and non-3GPP NTN/DTT networks? i.e., if it is possible to use the non-3GPP NTN/DTT network for MBS data delivery for those UEs, which have connectivity to 5GC via non-3GPP access (e.g., WLAN) only and do not have connectivity via 3GPP access.

    - It may be possible to establish per-UE PDU session for MBS data delivery via non-3GPP access network and the MBS delivery can switch between individual per-UE PDU session-based delivery via non-3GPP access network and shared delivery via non-3GPP NTN/DTT network.

  - WT_#1 would also study how to make the 5GS interwork with non-3GPP NTN/DTT technologies for the MBS data delivery? Can the existing 5G Converged Core be used to support both 3GPP NG-RAN and non-3GPP NTN/DTT Network similarly? Do we need to define a Non-3GPP Broadcast Interworking Function for interworking between the 5GC and non-3GPP NTN/DTT networks, similar to those defined for other non-3GPP access like WLAN? Are any changes expected in the existing N2 and N3mb interfaces for integrating non-3GPP NTN and non-3GPP DTT network with 5G Core, especially since non-3GPP NTN/DTT network needs to be selected for broadcast data distribution instead of NG-RAN? Will there be any impact on functions such as AF, MBSF and MBSTF, AMF, MB-SMF, MB-UPF, PCF, NEF to integrate non-3GPP NTN/DTT network with 5GS?

  - Will there be a need for authorization to deliver content via non-3GPP NTN/DTT networks to UEs?

- Evaluation of the proposed resolution mechanisms

- Architectural modifications in the 5G System

  - Modifications required in the Core Network

  - High-Level Architecture of the Non-3GPP Broadcast Interworking Function

  - Modifications required in the Access Network

  - Modifications required in the UICC APPs and ME

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 4 |  | May be | WT#1 is self-contained |
| WT#2 | 2 |  | May be | WT#2 is dependent on WT#1 |
| WT#3 | 2 |  | May be | WT#3 is dependent on WT#1 and WT#2 |
| WT#3.1 | 0.5 |  |  |  |
| WT#3.2 | 0.5 |  |  |  |
| WT#3.3 | 0.5 |  |  |  |
| WT#3.4 | 0.5 |  |  |  |

Total TU estimates for the study phase: 8

Total TU estimates for the normative phase: 0

Total TU estimates: N + M = 8

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur | Remarks |
| External TR | 23.xyz | Study on interworking of non-3GPP multicast and broadcast networks with the 3GPP 5GS | TBD | TBD | Saha, Anindya, Saankhya Labs, anindya@saankhyalabs.com Jha, Pranav, IIT Bombay, pranavjha@ee.iitb.ac.in | Tasks for the Secondary Rapporteur is TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| NA |  |  |  |

# 6	Work item Rapporteur(s)

Saha, Anindya, Saankhya Labs, anindya@saankhyalabs.com

Jha, Pranav, IIT Bombay, pranavjha@ee.iitb.ac.in: Tasks for the secondary Rapporteur is <TBD>

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None Identified Yet

# 9	Supporting Individual Members

Supporting IM name

Saankhya Labs

Ligado Networks

ONE Media 3.0

Fraunhofer IIS

CEWiT

Tejas Networks

IIT Bombay

IIT Kanpur

IIT Madras

IIT Hyderabad

IIT Kharagpur