3GPP TSG-SA WG2 Meeting #148-e	S2-2108579

# Electronic meeting,15 – 22 November 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0052 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Policy Container with ProSe Policy Provisioning Request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2021-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | Per clause 6.1.2.2.2 of TS 23.503, UE Policy Container with V2X/ProSe Policy Provisioning Request can be included in registration procedure, see below 6.1.2.2.2 Distribution of the policies to UE … At Initial Registration or the Registration to 5GS when the UE moves from EPS to 5GS: … - UE may indicate the V2X Policy Provisioning Request in the UE Policy Container. If this indication is received, the PCF includes V2XP in the UE policy information as defined in clause 6.2.2 of TS 23.287 [28]. CT1 has questioned in the incoming LS (S2-2108284) both the PSI in the UE state indication and the UE policy provisioning request in the registration request. The PCF can deliver the V2X policy based on the PSI and the V2X capability forwarded from the AMF to the UE. In the clause 4.2.2.2.1.2 and clause 4.2.2.2.1.3 of TS 29.525, It is also specified that the PCF can deliver the V2X and ProSe policy based on the ProSe and V2X capability forwarded by the PCF. Additionally the V2X and the ProSe capability forwarded by the AMF to PCF is required to determine whether to send the updated V2X/ProSe policy to the UE if the service specific information from UDR has been updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description of the ProSe Policy provisioning from Registration procedure is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PCF may require to compare the PSI and the UE provisioning request, and then determine whether to forward the V2X/ProSe policy to the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

4.3.1	UE

Any 5G ProSe-enabled UE may support the following functions:

-	Exchange of information for 5G ProSe Direct Discovery between 5G ProSe-enabled UE and the 5G DDNMF over PC3a reference point.

-	Procedures for 5G ProSe Direct Discovery of other 5G ProSe-enabled UEs over PC5 reference point.

-	Procedures for 5G ProSe Direct Communication over PC5 reference point, including Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication.

-	Procedures to act as a 5G ProSe Layer-2 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-3 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-2 Remote UE.

-	Procedures to act as a 5G ProSe Layer-3 Remote UE.

-	Indicating 5G ProSe Policy Provisioning Request in UE Policy Container UE triggered 5G ProSe Policy provisioning procedure, which may request one or multiple types of policies/parameters as listed below:

-	Policy/parameters for 5G ProSe Direct Discovery;

-	Policy/parameters for 5G ProSe Direct Communication;

-	Policy/parameters for 5G ProSe Layer-2 Remote UE;

-	Policy/parameters for 5G ProSe Layer-3 Remote UE;

-	Policy/parameters for 5G ProSe Layer-2 UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay.

-	Receiving the 5G ProSe Policy from 5GC over N1 reference point.

-	Configuration of parameters for 5G ProSe Direct Discovery and 5G ProSe Direct Communication (e.g., including IP addresses, ProSe Layer-2 Group IDs, see clause 5.1). These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over PC1 reference point from the ProSe Application Server.

-	Reporting the following capabilities to 5GC over the N1 reference point:

-	5G ProSe Capability.

* * * * Next Change * * * *

6.2.2	PCF based Service Authorization and Provisioning to UE

For PCF based Service Authorization and Provisioning to UE, the Registration procedures as defined in clause 4.2.2.2 of TS 23.502 [5], UE Policy Association Establishment procedure as defined in clause 4.16.11 of TS 23.502 [5] and UE Policy Association Modification procedure as defined in clause 4.16.12 of TS 23.502 [5] apply with the following additions:

-	If the UE indicates 5G ProSe Capability in the Registration Request message and if the UE is authorized to use 5G ProSe service based on subscription data, the AMF selects the PCF which supports 5G ProSe Policy/Parameter provisioning as described in clause 6.2.3 and establishes a UE policy association with the PCF for 5G ProSe Policy/Parameter delivery.

-	If the UE includes the the list of PSIs for 5G ProSe policy/parameters in the UE Policy Container in the Registration Request message, the PCF determines whether to provision 5G ProSe Policy/parameters to the UE, as specified in clause 6.1.2.2.2 of TS 23.503 [9], and the PCF provides the 5G ProSe Policy/parameters (see clause 5.1.2.1, clause 5.1.3.1 and clause 5.1.4.1) to the UE by using the procedure as defined in clause 4.2.4.3 "UE Configuration Update procedure for transparent UE Policy Delivery" in TS 23.502 [5].

The PCF may update the 5G ProSe Policy/parameters to the UE in following conditions:

-	UE Mobility, e.g. UE moves from one PLMN to another PLMN. This is achieved by using the procedure of UE Policy Association Modification initiated by the AMF, as defined in clause 4.16.12.1 of TS 23.502 [5].

-	When there is a subscription change in the list of PLMNs where the UE is authorized to perform 5G ProSe services. This is achieved by using UE Policy Association Modification initiated by the PCF procedure as defined in clause 4.16.12.2 of TS 23.502 [5].

-	When there is a change of service specific parameter (including path selection policy) as described in clause 6.2.5 (performing the procedure in clause 4.15.6.7 of TS 23.502 [5]).

-	When the timer associated with some Policy/parameter expires.

-	When the UE determines that the ProSe Policy/parameter(s) is invalid and performs UE triggered Policy Provisioning procedure to the PCF.

If the serving PLMN is removed from the list of PLMNs in the service authorization parameters, the service authorization is revoked in the UE.

When the UE is roaming, the change of subscription resulting in updates of the service authorization parameters are transferred to the UE by H-PCF via V-PCF.

The UE may perform UE triggered Policy Provisioning procedure to the PCF as specified in clause 6.2.4 when the UE determines the 5G ProSe Policy/Parameter is invalid (e.g. Policy/Parameter is outdated, missing or invalid). The PCF determines the 5G ProSe Policy/Parameters for the UE based on UE requested policies and authorized 5G ProSe capabilities.

When the UE disables a ProSe capability, the PCF may stop updating the corresponding ProSe Policy/parameter(s) and when the UE enables a ProSe capability the PCF may need to provide or update the corresponding ProSe Policy/parameter(s).

When a 5G ProSe Layer-3 Remote UE is accessing to 5GC via a 5G ProSe Layer-3 UE-to-Network Relay without involving N3IWF, the PCF based provisioning and update of 5G ProSe Policy/parameters to the 5G ProSe Layer-3 Remote UE are not supported.

* * * * End of Changes * * * *