3GPP TSG-SA2 Meeting #148-e	S2-2108600

Nov 15 - 22, 2021, Electronic

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0059 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS session update related procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - MBS Session Update procedure needs corrections on NF services used, clause references for multicast session update procedures, etc. - In current MBS Session Update with PCC procedure, there are different handlings & NF interactions for Scenario A: updates of MBS service area and/or MBS session activation/deactivation and Scenario B: other updates of the service description and QoS related updates. However, considering: a) updating QoS or other servie description may happen during MBS session activation or MBS service area updates; b) updates of MBS service area and/or MBS session activation may subject to operator policy, e.g. if the MBS session is (temporarily) forbiddened in certain area or certain time period; it is suggested to apply the same handling to Scenario A as to Scenario B. That is, for both scenarios, the interactions of NEF/MBSF->PCF->MB-SMF for MBS related PCC control apply. This can also simply the logic functionality of the relavant NFs (e.g. NEF/MBSF). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For both MBS Session Update with PCC and without PCC procedures: Correct the NF services used, clause references for multicast session update procedures, etc. - For MBS Session Update with PCC procedure: merge the handling for "updates of MBS service area and/or MBS session activation/deactivation" with that for "updates of the service description and QoS related updates", i.e. applying same interactions of NEF/MBSF->PCF->MB-SMF for MBS related PCC control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned specification. Possible insufficient or complex logic functionality for MBS related PCC control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6, 7.1.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.1.1.6	MBS Session Update without PCC

This procedure is used by the AF to update the MB service area and/or update QoS of an MBS Session. Updating QoS of an MBS Session may lead to addition of new MBS QoS Flow(s). The procedure applies to both multicast and broadcast communications unless otherwise stated.

Figure 7.1.1.6-1: Update of MBS Session without PCC

1.	AF of content provider initiates MBS Session Update to a NEF/MBSF, e.g. to update MBS service area and/or update service requirement, or to activate or deactivate an MBS session. AF may provide updated information for an MBS session (identified by MBS session ID) by invoking the Nnef_MBSSession_Update request ([MBS Session ID], MBS information, AF Identifier). MBS information may include service requirements, MBS service area information and media information. The service requirements adjustment may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s).

2.	NEF checks authorization of AF.

3.	NEF/MBSF invokes the Nmbsmf_MBSSession_Update request ([MBS Session ID], MBS information, AF Identifier) to the MB-SMF.

4.	The MB-SMF derives any updated QoS parameters based on the MBS information and local policy, if needed.

5-6.MB-SMF may need to update MB-UPF, e.g. if new MBS QoS Flow is to be created, or existing MBS QoS Flow is to be deleted.

7.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.3. For multicast communication, the MB-SMF continues the procedure towards the SMF, AMF and NG-RAN as specified in clause 7.2.5 (for service activation/deactivation) or clause 7.2.6 (for QoS updates and/or MBS service area updates).

8.	If the MBS service area is being updated, the MB-SMF stores the updated MBS service area in its profile at the NRF by invoking the Nnrf_NFManagement_NFUpdate request (MB-SMF ID, MBS Session ID, MBS Service Area).

9.	MB-SMF sends the Nmbsmf_MBSSession_Update response to the NEF/MBSF.

10.	NEF/MBSF sends the Nnef_MBSSession_Update response to the AF.

#### 7.1.1.7	MBS session configuration Update with PCC

Figure 7.1.1.7-1: MBS session configuration update with PCC

1-2.	Same as in Figure 7.1.1.6-1.

3.	NEF/MBSF provides the input received from the AF, i.e. updated information for an MBS session, to the PCF by sending Npcf_MBSPolicyAuthorization_Update Request message (MBS Session ID,MBS information).

4.	Based on the Npcf_MBSPolicyAuthorization_Update Request received in step 7, the PCF may provide updated policy rules to the MB-SMF by invoking Npcf_MBSPolicyControl_UpdateNotify request message including the updated policy information about the MBS Session.

5-6.	Same as steps 5-6 in Figure 7.1.1.6-1.

7.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.3. For multicast communication, the MB-SMF continues the procedure towards the SMF, AMF and NG-RAN as specified in clause 7.2.5 (for service activation/deactivation) or clause 7.2.6 (for QoS updates and/or MBS service area updates).

8.	If the MBS service area is being updated, the MB-SMF stores the updated MBS service area in its profile at the NRF by invoking the Nnrf_NFManagement_NFUpdate request (MB-SMF ID, MBS Session ID, MBS Service Area).

9.	The PCF sends the Npcf_MBSPolicyAuthorization_Update response to the NEF/MBSF.

10.	Same as step 10 in Figure 7.1.1.6-1.

# * * * * End of changes * * * *