3GPP TSG-SA WG2 Meeting #148E e-meeting	S2-2108623

# Elbonia, November 15 – 22, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0038 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on UUAA context during revocation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As defined in TS 23.256, the successful UUAA result is stored in AMF or SMF and the corresponding UAV UE’s UUAA context is stored in UAS NF after the successful UUAA-MM or UUAA-SM procedure. The UUAA result and the UUAA context shall be removed accordingly when the UUAA Revocation is initiated by the USS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | During the UUAA Revocation procedure, the UUAA result and the UUAA context in the network are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UUAA status mismatching is occurred between the core network and the USS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change *****

5.2.7	UUAA Revocation by USS/UTM

Figure 5.2.7.1-1: Procedure for UAV authorization revocation by USS.

UAS NF stores the UAV UE’s UUAA context after successful UUAA procedure as explained in clause 5.2.2.2 for UUAA-MM and in clause 5.2.3.2 for UUAA-SM procedure.

1.	The USS sends Naf_Auth_notification to UAS NF for authorization revocation. The USS includes GPSI, CAA-Level UAV ID, cause of revocation, PDU Session IP address if available in the authorization revocation request.

2.	UAS NF retrieves the UAV UE's stored UUAA context. From the stored UUAA context the UAS NF determines the target AMF or SMF for sending the notification.

3a or 3b.	The UAS NF sends Nnef_Auth_Notification request to notify the target NF, i.e. either the AMF or the SMF that the UAV is not authorized anymore. The target NF shall remove the successful UUAA result and response to the UAS NF.

4.	The UAS NF shall remove the UAV UE’s UUAA context. The UAS NF responds back to the USS indicating that authorization revocation request has been successfully initiated.

5a.	If UAS NF has subscribed to AMF for the Mobility Event Exposure with Event ID = Reachability Filter before, UAS NF unsubscribes to AMF for the mobility event notification by sending Namf_EventExposure_Unsubscribe request with Subscription Correlation ID.

5b.	The AMF acknowledges the un-subscription request from 5a by sending Namf_EventExposure_Unsubscribe response.

6.	If UE is in CM_Idle state, the target NF (i.e. either the AMF or the SMF) initiates the Network Triggered Service Request procedures as described in clause 4.2.3.3 of TS 23.502 [3].

7a.	If the target NF is AMF, the AMF initiates UCU procedure to inform the UE that UUA is revoked. The AMF shall also initiate the release of PDU Sessions related to UAS services.

7b. If the target NF is AMF, based on network policy the AMF may start network initiated de-registration process as described in clause 4.2.2.3.3 of TS 23.502 [3].

7c.	If the target NF is SMF, the SMF starts network initiated PDU session release process as described in clause 4.3.4 of TS 23.502 [3] to release the associated PDU session.

*** End of changes ***