3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108674

# Elbonia, November 15 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0176 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UE triggered Policy provisioning Procedure for V2X |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. The V2X Policy Provisioning Request is included in the UE Policy Container, which is described in clause 6.2.2 of TS23.287 as followings: - If the UE supports V2X communication and it does not have valid V2X policy/parameters, the UE includes the UE Policy Container with indicating the V2X Policy Provisioning Request during registration procedure. - If the UE indicates the V2X Policy Provisioning Request in the UE Policy Container, the PCF determines whether to provision V2X Policy/parameters for V2X communication over PC5 reference point and/or V2X communication over Uu reference point to the UE, as specified in clause 6.1.2.2.2 of TS 23.503 [16], and the PCF provides the V2X Policy/parameters to the UE by using the procedure as defined in clause 4.2.4.3 of TS 23.502 [7]. And CT1 also defines the V2X Policy Provisioning Request as one type message of UE Policy Container: D.6.1 UE policy delivery service message type Table D.6.1.1: UE policy delivery service message type Bits. 8; 7; 6; 5; 4; 3; 2; 1. 0; 0; 0; 0; 0; 0; 0; 0; Reserved. 0; 0; 0; 0; 0; 0; 0; 1; MANAGE UE POLICY COMMAND message. 0; 0; 0; 0; 0; 0; 1; 0; MANAGE UE POLICY COMPLETE message. 0; 0; 0; 0; 0; 0; 1; 1; MANAGE UE POLICY COMMAND REJECT message. 0; 0; 0; 0; 0; 1; 0; 0; UE STATE INDICATION message. 0; 0; 0; 0; 0; 1; 0; 1; UE POLICY PROVISIONING REQUEST message (see NOTE). 0; 0; 0; 0; 0; 1; 1; 0; UE POLICY PROVISIONING REJECT message (see NOTE). All other values are reserved. NOTE: Coding and usage of UE POLICY PROVISIONING REQUEST message and UE POLICY PROVISIONING REJECT message are specified in 3GPP TS 24.587 [19B].. But the current description in 6.2.4 is not correct, i.e. UE Policy provisioning request including the UE Policy Container. 2. The “UE Policy Container” is encapsulated in Payload Container IE carried in e.g. UL NAS TRANSPORT message and AMF forwards it to the PCF transparently, so the AMF does not know the UE policy need to be updated, i.e. trigger the Npcf_UEPolicyControl_Update request. In the other hand, the Policy Control Request Trigger of UE Policy Association is configured in AMF by PCF when the UE Policy Association is created, and only when the trigger condition has been met, the AMF initiates the Npcf_UEPolicyControl_Update request to the PCF. So the current description in 6.2.4 is not correct, i.e. the AMF sends the Npcf_UEPolicyControl_Update request to the PCF including the UE Policy Container received from UE. 6.2.4 Procedure for UE triggered V2X Policy provisioning The UE triggered Policy Provisioning procedure is initiated by the UE to request V2X Policy/Parameter from the PCF when UE determines the V2X Policy/Parameter is invalid in the following cases: - if the validity timer indicated in the V2X Policy/Parameter expires; - if there are no valid parameters, e.g. for the V2X service type a UE wants to use, for current area, or due to abnormal situation. Figure 6.2.4-1: UE triggered V2X Policy provisioning procedure 1. The UE sends the UE Policy provisioning request including the UE Policy Container (V2X Policy Provisioning Request) to the AMF. 2. The AMF sends the Npcf_UEPolicyControl_Update request to the PCF including the UE Policy Container received from UE. 3. The UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [7] is triggered. |
| Summary of change: |  |  | 1. The UE sends the UE Policy Container including the UE Policy provisioning request to the AMF. 2. The AMF sends the Namf_Communication_N1MessageNotify to the PCF including the UE Policy Container received from UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | The description is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 6.2.4 |  |  |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.4	Procedure for UE triggered V2X Policy provisioning

The UE triggered Policy Provisioning procedure is initiated by the UE to request V2X Policy/Parameter from the PCF when UE determines the V2X Policy/Parameter is invalid in the following cases:

-	if the validity timer indicated in the V2X Policy/Parameter expires;

-	if there are no valid parameters, e.g. for the V2X service type a UE wants to use, for current area, or due to abnormal situation.

Figure 6.2.4-1: UE triggered V2X Policy provisioning procedure

1.	The UE sends the UE Policy Container (V2X Policy Provisioning Request) via UL NAS TRANSPORT message to the AMF.

2.	The AMF sends the Namf_Communication_N1MessageNotify to the PCF including the UE Policy Container received from UE.

3.	The UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [7] is triggered.

# * * * * End of changes * * * *