3GPP TSG-SA WG2 Meeting #148E	S2-2108681

15 – 22 November 2021, e-Meeting	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3413 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple TACs handling for NR satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the LS S2-2103770, RAN2 asked feasibility from NAS layer for the use of “option 2: AS indicates all received TAC(s) for one PLMN to NAS layer”. Based on the reply LS, SA2 and CT1 have already agreed that option is feasible. SA2 thinks there are some other aspects related to supporting multiple TACs, such as how Service Area Restrictions and Forbidden Areas would be supported. For the assumotion that a cell may broadcast one or more TACs per PLMN, UE cannot know its current location through SI so that UE cannot know if it is in a Service Restrictions Area or Forbidden Area. AMF may receive more than one TAI in the ULI, therefore, how to handle the mutilple TACs and how to support SRA and FA are still not addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - When receives multiple TAIs in the ULI, AMF proceeds with the procedure if one of the multiple TACs is not in non-allowed area or Forbidden Area, and initiates LCS procedure to determine the UE location and the TA the UE currently located. - the NG-RAN may determine the TA which the UE is currently located when the target UE is in CM-CONNECTED state and provide that TAI to AMF as part of ULI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multiple TACs cannot be supported for NR satellite access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4.11.X 	Tracking Area handling for NR satellite access

A radio cell for NR satellite access may indicate support for one or more TACs for each PLMN. If AMF, based on the ULI, receives multiple TAIs during Registration procedure or UE triggered Service Request procedure in CM-IDLE, and cannot be aware of the TA which the UE is currently located to make a decision, AMF proceeds with the procedure if one of the multiple TACs is not in non-allowed area or Forbidden Area, and initiates LCS procedure as specified in clause 6.10.1 of TS 23.273 [87] to determine the UE location and the TA which the UE is currently located. The AMF should reject any NAS request if the UE is not allowed to operate in the present TA which the UE is currently located. Or the AMF may include that TA to the RA if the UE is allowed to operate in the present TA and the TA is not in the RA.

Editor’s note: Whether UE needs to know the TA it is currently located is FFS.

In case multiple TACs are broadcasted for a PLMN in the serving radio cell for a UE, the NG-RAN may determine the TA which the UE is currently located when the target UE is in CM-CONNECTED state as specified in TS 38.300 [27] and provide that TAI to AMF as part of ULI.

# * * * * End of changes * * * *