SA WG2 Meeting #S2-148E	S2-2108697

15 - 22 November, 2021, Electronic meeting

Title:	Reply LS on UE location aspects in NTN

Response to:	S2-2108279 / C1-216250

Release:	Release 17

Work Item:	5GSAT_ARCH

Source:	SA2

To:	CT1, RAN2, RAN3

Cc:

Contact Person:

Name:	Jingran Chen

E-mail Address:	chenjingran@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2108681, S2-2108693

1. Overall Description:

In case multiple TACs are broadcasted for a PLMN in the serving radio cell for a UE , SA2 has reached the following agreement:

- For NR satellite access, during the Registration procedure or UE triggered Service Request procedure in CM-IDLE, NG-RAN will report all broadcast TACs to AMF as part of ULI.

- The NG-RAN may determine the TAI the UE is currently located when the target UE is in CM-CONNECTED state and provide that TAI to AMF as part of ULI. The ULI contains the TAI for the TA in which the UE is physically located, no matter whether the TAC is broadcasted in the serving radio cell or not. NG-RAN determines the TAC based on its available knowledge of the UE location.

2. Actions:

To CT1, RAN2, RAN3 group.

ACTION: 	SA2 asks CT1, RAN2 and RAN3 to take the above into account and to provide any feedback including comments or questions.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#149-e			14 - 25 February 2022			Electronic Meeting