3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108726

# Elbonia, November 11 – 15, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3280 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.502 Clarification on NF discovery by NRF in Onboarding or external authentication scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The identificaiton of the CH with AAA server or the identification of the DCS with AAA server has been added to the NF profile or NRF related service request as specified in TS 23.501. Therefore, the corresponding procedure for NF/NF service discovery by NF service consumer in the same SNPN shall be added accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe the NF/NF service discovery by NF service consumer in the same SNPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NF/NF service discovery by NF service consumer in the same SNPN is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.4a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.17.4a	NF/NF service discovery by NF service consumer in the same SNPN

Figure 4.17.4a-1: NF/NF service discovery in the same SNPN

The NF/NF service discovery by NF service consumer in the same SNPN is specified as clause 4.17.4 with the following additions:

-	If the target NF is AUSF or NSSAAF and the Nnrf_NFDiscovery_Request includes a Home Network Identifier in the form of a realm which is the identification of the CH with AAA server or the identification of the DCS with AAA server, the NRF selects the AUSF or NSSAAF in the same SNPN based on the NF profile as specified in clause 6.2.6.2 in TS 23.501 [2].

-	If the target NF is UDM and the Nnrf_NFDiscovery_Request includes a Home Network Identifier in the form of a realm which is the identification of the CH with AAA server, the NRF selects the UDM in the same SNPN based on the NF profile as specified in clause 6.2.6.2 in TS 23.501 [2].

# * * * * End of changes * * * *