3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108740

# Elbonia, November 15th – 19th, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0686 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS parameter handling for TSC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The recent update of the functionality description for AF session with required QoS resulted in some unclear or misplaced statements which are also not fully aligned with statements in 23.501. It is therefore suggested to update the description of the functionality with the goal to fully describe the PCF mapping of the AF provided QoS parameters to PCC rule QoS parameters in this specification. There is also the need to enable the PCF to reject an AF request with individual QoS parameters to address situations in which the QoS requirements (represented by the AF provided QoS parameters) cannot be fulfilled due to operator policies or limitations of the deployed network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is therefore proposed to update and restructure the description by: - Clarifying that either a QoS reference or a set of individual QoS parameters is provided by the AF (including the Alternative Service Requirements). - The Requested 5GS Error Ratio is added as another QoS parameter that can be provided by the AF (as this is an important input for the 5QI selection). - The mapping of AF provided QoS parameters to PCC rule QoS parameters is described explicitly. - The principle for the selection of the 5QI is described explicitly. - The possibility for a rejection of the AF request is added so that the PCF can respond appropriately if the QoS requirements (represented by the AF provided QoS parameters) cannot be fulfilled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The details of the description for PCF mapping of AF provided QoS parameters to PCC rule QoS parameters is inconsistent and incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.3.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System Architecture for the 5G System".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6]	3GPP TS 23.179: "Functional architecture and information flows to support mission-critical communication service; Stage 2".

[7]	Void.

[8]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[9]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[10]	3GPP TS 23.161: "Network-Based IP Flow Mobility (NBIFOM); Stage 2".

[11]	3GPP TS 23.261: "IP flow mobility and seamless Wireless Local Area Network (WLAN) offload; Stage 2".

[12]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[13]	3GPP TS 29.507: "Access and Mobility Policy Control Service; Stage 3".

[14]	Void.

[15]	3GPP TS 22.011: "Service Accessibility".

[16]	3GPP TS 23.221: "Architectural requirements".

[17]	3GPP TS 29.551: "5G System; Packet Flow Description Management Service; Stage 3".

[18]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[19]	3GPP TS 24.526: "UE Equipment (UE) policies for 5G System (5GS); Stage 3".

[20]	3GPP TS 32.291: "Charging management; 5G system, Charging service; stage 3".

[21]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; Stage 2".

[22]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[24]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[25]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[26]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[27]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[28]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[29]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[30]	3GPP TS 24.237: "IP Multimedia (IM) Core Network (CN) subsystem IP Multimedia Subsystem (IMS) Service Continuity; Stage 3".

[31]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[32]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[33]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[34]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[35]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[xx]	3GPP TS 29.514: "Policy Authorization Service; Stage 3".

# * * * * Second change * * * *

#### 6.1.3.22	AF session with required QoS

The AF may request that a data session to a UE is set up with a specific QoS (e.g. low latency or jitter) and priority handling. The AF can request the network to provide QoS for the AF session based on the service requirements with the help of a QoS reference parameter which refers to pre-defined QoS information (which also includes the bandwidth). Instead of using the QoS reference, the AF may provide individual QoS parameters associated to the Flow Description to request the network to provide QoS for the AF session.

a)	When the PCF receives a QoS reference but no individual QoS parameters:

-	When the PCF authorizes the service information from the AF, it derives the QoS parameters of the PCC rule based on the service information and the indicated QoS reference parameter.

NOTE 1:	A SLA has to be in place between the operator and the ASP defining the possible QoS levels and their charging rates. For each of the possible pre-defined QoS information sets, the PCF needs to be configured with the corresponding QoS parameters and their values as well as the appropriate Charging key (or receive this information from the UDR).

-	The AF may change the QoS by providing a different QoS reference parameter while the AF session is ongoing. If this happens, the PCF shall update the related QoS parameter sets in the PCC rule accordingly.

b)	When the PCF receives individual QoS parameters but no QoS reference:

-	The PCF may receive one or more of the following individual QoS parameters: bandwidth, priority, Maximum Burst Size, Requested 5GS Delay and Requested 5GS Error Ratio.

NOTE 2:	Different combinations of individual QoS parameters with specific parameter names exist and they are described in TS 23.501 [2] (for Time Sensitive Communication), in clause 6.1.3.23 (for integration with Time Sensitive Networking) and in TS 29.514 [xx].

-	When the PCF authorizes the service information from the AF, it derives the QoS parameters of the PCC rule based on the service information and the individual QoS parameters received from the AF:

-	The PCF derives the GBR and MBR from the bandwidth, if provided by the AF, or otherwise according to operator policy.

-	The PCF sets the ARP according to operator policy (service information provided by the AF can be taken into account).

-	The PCF derives the PDB from the Requested 5GS Delay, if provided by the AF, or otherwise according to operator policy.

NOTE 2:	The TSCTSF can adjust the Requested 5GS Delay according to the UE-DS-TT residence time (if available) as described in TS 23.501 [2].

-	The PCF derives the PER from the Requested 5GS Error Ratio, if provided by the AF, or otherwise according to operator policy.

-	The PCF derives the Priority Level from the priority, if provided by the AF, or otherwise according to operator policy.

-	The PCF derives the MDBV from the Maximum Burst Size, if provided by the AF, or otherwise according to operator policy.

-	The PCF checks if there is any standardized, pre-configured or dynamically assigned 5QI available which matches the derived PDB, and PER. If such 5QI(s) are found, the PCF selects one of them based on operator policy (the selection can take the derived MDBV and the derived Priority Level into account). If the selected 5QI does not match the derived MDBV, the PCF shall set the MDBV parameter in the PCC rule according to the derived MDBV. If no matching standardized, pre-configured or dynamically assigned 5QI is found, the PCF shall generate a new dynamically assigned 5QI based on the derived PDB, PER, MDBV and Priority Level. If necessary, the PCF can either adjust the GBR and MBR values according to the Averaging Window associated with the selected 5QI or set the Averaging Window parameter in the PCC rule according to the duration relevant for the AF session.

-	The AF may change the QoS by providing different values for the individual QoS parameters while the AF session is ongoing. If this happens, the PCF shall update the related QoS parameter sets in the PCC rule accordingly.

-	The PCF may reject the individual QoS parameters received from the AF based on operator policy or impossibility to support the requested values of the individual QoS parameters. If this happens, the PCF may provide one or more combinations of individual QoS parameters that can be supported in the response to the AF.

The PCF generates a PCC Rule with service data flow filter (including IP Packet Filter set as in clause 5.7.6.2 of TS 23.501 [2]) or Ethernet Packet Filter set as in clause 5.7.6.3 of TS 23.501 [2]) derived from the Flow Descriptions provided by the AF, and the derived PCC rule QoS parameters like 5QI, ARP, GBR and MBR (see clause 6.3.1 for all possible PCC rule QoS parameters).

If the PCF gets informed about Policy Control Request Triggers relevant for the AF session, the PCF shall inform the AF about it as defined in clause 6.1.3.18.

If an AF session can adjust to different QoS parameter combinations, the AF may provide Alternative Service Requirements containing one or more QoS reference parameters or sets of individual QoS parameters (i.e. bandwidth, Requested 5GS Delay, Requested 5GS Error Ratio) in a prioritized order (which indicates the preference of the QoS requirements with which the service can operate). If so, the AF shall also subscribe to receive notifications from the PCF for successful resource allocation and when the QoS targets can no longer (or can again) be fulfilled as described in clause 6.1.3.18.

When the PCF authorizes the service information from the AF and generates a PCC rule, it shall also derive Alternative QoS parameter sets for this PCC rule based on the QoS reference parameters or the sets of individual QoS parameters in the Alternative Service Requirements. If the AF provided individual QoS parameters in the Alternative Service Requirements, the PCF shall derive the Alternative QoS parameter sets based on the bandwidth, Requested 5GS Delay and Requested 5GS Error Ratio in the respective Alternative Service Requirement.

The PCF shall enable QoS Notification Control and include the derived Alternative QoS parameter sets (in the same prioritized order indicated by the AF) in the PCC rule sent to the SMF. When the PCF notifies the AF that QoS targets can no longer be fulfilled, the PCF shall include the QoS reference parameter or the set of individual QoS parameters corresponding to the Alternative QoS parameter set referenced by the SMF or an indication that the lowest priority QoS reference of the Alternative Service Requirements cannot be fulfilled (as described in clause 6.1.3.18).

NOTE 2:	The AF behaviour is out of the scope of this TS but can include adaptation to the change of QoS (e.g. rate adaptation) as well as application layer signalling with the UE.

The AF may change the Alternative Service Requirements while the AF session is ongoing. If this happens, the PCF shall update the Alternative QoS parameter sets in the PCC rule accordingly.

The AF may indicate to the PCF that the UE does not need to be informed about changes related to Alternative QoS Profiles. With this indication received from the AF, the PCF decides whether to disable the notifications to the UE when changes related to the Alternative QoS Profiles occur and sets the Disable UE notifications at changes related to Alternative QoS Profiles parameter in the PCC rule accordingly.

# * * * * End of changes * * * *