SA WG2 Meeting #S2-148E	S2-2108774

15 - 22 November, 2021, Electronic meeting	(revision of S2-2107379r01)

# Source:	Nokia, Nokia Shanghai Bell

# Title:	New SID on 5G support for Femto

# Document for:	Approval

# Agenda Item:	9.1.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on 5G support for Femto

Acronym:	FS_5GSF

Unique identifier:	tbd

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_Resident | SA1 | 880040 | Study of Enhancements for Residential 5G |
| FS_PIN | SA1 | 880041 | Study on Personal IoT Networks |
| PIRates | SA1 | 930022 | Personal IoT and Residential networks Service Requirements |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 830042 | Vertical_LAN | CAG introduction |
| 900015 | Enhanced support of Non-Public Networks | CAG enhancements |

Dependency on non-3GPP (draft) specification:

# 3	Justification

SA1 is specifying normative requirements for Premises Radio Access Station (PRAS) as part of the PIRates WID. This study item aims to focus on enabling access control for “5G Femto”. 5G Femto or Home Base Stations (HBS) in this context refers to a 5G related concept or architecture like the LTE concept and architecture for Home eNodeB(s) with e.g., NR capable base stations deployed at home or at enterprise premises.

NOTE:	A Rel-18 WID was proposed in RAN WG4 that aims to specify necessary features to support NR deployments for Home Base Stations (HBS). RAN WG4 Rel-18 package has been discussed and is planned to be approved in March 2022.

Following are the justifications for the study objectives:

1) 5G Femto offers a cost effective way to improve 5G indoor coverage and enables better support for Enterprise mobility.

2) 80% of mobile sessions are indoor and inside coverage with 5G mid and high-bands is limited. Need for a solution that enables end user plug and play, allowing customized access control (i.e., to specify access control using CAG concept, updating CAG list on the UE).

3) High bandwidth and throughput with 5G needed at home and at campus locations for new immersive applications such as AR/VR/MR gaming, e-sports, UHD 8K video, telepresence and remote meeting, HD surveillance.

Access control for 5G Femto can leverage the CAG concept defined for NPN. Close cooperation with RAN groups when defining the 5G Femto architecture is required.

# 4	Objective

Following aspects will be studied:

WT#1: Study how to support 5G Femto deployments:

WT#1.1 Study how to define the necessary overall architecture and required functionalities for 5G Femto deployments.

WT#1.2: Study how to define the access control mechanism to allow for 5G Femto deployments by using the CAG concept.

NOTE:	RAN internal (architecture) aspects need to be studied in RAN WGs, e.g., RAN WG3 and RAN WG4. Cooperation with RAN WGs is required, for example when enhancing interfaces between RAN and Core.

WT#2: Study how to enable provisioning of subscribers allowed to access 5G Femto (such as using a function like CSG Subscriber Server (CSS) as specified for EPS), e.g., within a CAG, and how to manage 5G Femto access control by the CAG owner or an authorized administrator.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or is depended on completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1.1 | 2 | 1 | Yes | self-contained |
| WT#1.2 | 1 | 1 | No | Depends on outcome of WT#1.1 |
| WT#2 | 1.5 | 1 | Maybe | Depends on outcome of WT#1.1 |

Total TU estimates for the study phase: 4.5

Total TU estimates for the normative phase: 3

Total TU estimates: 4.5 + 3 = 7.5

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | TR 23.xxx | Study on support for 5G Femto | SA#96 June 2022 | SA#97 September 2022 | Nokia |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Rainer Liebhart, Nokia, rainer dot liebhart at nokia dot com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

RAN impacts covered by RAN WGs.

Security aspects covered by SA3.

OAM and charging aspects covered by SA5.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

Verizon

BT

KPN

Matrixx