3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108803

# Elbonia, Nov 15-19, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0056 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on subscription information to 5G ProSe |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In current text of clause 5.7, it reads like the subscription information listed are all obtained from the UDM. However, PC5 QoS parmaters are provided by the UDR to the PCF, and for the allowed PLMNs, both UDM and PCF should contains this information (provided to DNNMF and UE, respectively). Thus, it needs revisoins to clarify this. - The Service-based interfaces in clause 4.2.6 does not contain Naf service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that PC5 QoS parameters are provided by the PCF and the list of the PLMNs authorized for 5G ProSe services are contained both in UDM and PCF. - Add Naf service to clause 4.2.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding of how subscription information to 5G ProSe are provided and wrong UDM implementation. Incomplete Service-based interfaces list in TS 23.256. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.6, 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.2.6	Service-based interfaces

N5g-ddnmf:	Services provided by 5G DDNMF to manage inter-PLMN 5G ProSe Direct Discovery operations.

In addition to the relevant services defined in TS 23.501 [4] for the following service-based interfaces, in the case of ProSe Service, the services can be provided by corresponding NF are as follows:

Nudm:	Services provided by UDM are used to get 5G ProSe Service related subscription information to the AMF during Initial Registration procedure or UE Configuration Update (UCU) procedure to inform the AMF the subscription information has changed, and to provide ProSe Service related subscription information to 5G DDNMF for the authorisation of 5G ProSe Direct Discovery requests. The subscription information is described in TS 23.502 [5].

Npcf:	Services provided by H-PCF are used to provide 5G ProSe Service related parameters to V-PCF for the UE and NG-RAN in the roaming case, and to enable the 5G DDNMF to get a PDUID or be notified of PDUID change.

Nudr:	Services provided by UDR are used to notify the PCF and the UDM of the update of the 5G ProSe Service related information as described in TS 23.502 [5].

Nnef:	Services provided by NEF are used by the ProSe Application Server to update 5G ProSe Service related information of 5GC.

Namf:	Services provided by AMF are consumed by PCF to provide the 5G ProSe Service related parameters for the UE and the NG-RAN to the AMF, and to enable the AMF create or update the UE context related to 5G ProSe service.

Nnrf:	Services provided by NRF are used to discover the PCF that supports 5G ProSe service and for 5G DDNMF in HPLMN to discover other 5G DDNMFs in VPLMN or local PLMN.

Naf:	Services provided by AF are consumed by the DDNMF to request authorization for Discovery Request. The AF may update the authorization information to revoke the Restricted ProSe Direct Discovery permission.

# * * * * Second change * * * *

## 5.7	Subscription to 5G ProSe

The subscription information in the UDM contains information to give the user permission to use 5G ProSe.

At any time, the operator can amend or remove the ProSe UE subscription rights from subscription information in the UDM, or to revoke the user's permission to use 5G ProSe.

The following subscription information is defined for 5G ProSe:

-	subscription for open 5G ProSe Direct Discovery for NR PC5:

-	open 5G ProSe Direct Discovery Model A.

-	subscription for restricted 5G ProSe Direct Discovery for NR PC5:

-	restricted 5G ProSe Direct Discovery Model A;

-	restricted 5G ProSe Direct Discovery Model A with application-controlled extension;

-	restricted 5G ProSe Direct Discovery Model A with "on demand" announcing;

-	restricted 5G ProSe Direct Discovery Model B.

-	subscription for Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication for NR PC5.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-2 UE-to-Network Relay.

-	subscription for 5G ProSe UE acting as 5G ProSe Layer-3 UE-to-Network Relay.

-	subscription for 5G ProSe Layer-2 Remote UE access via 5G ProSe Layer-2 UE-to-Network Relay

-	subscription for 5G ProSe Layer-3 Remote UE access via 5G ProSe Layer-3 UE-to-Network Relay

-	UE-PC5-AMBR for NR PC5.

-	PC5 QoS parameters as defined in clause 5.6.1 used by NG-RAN.

-	the list of the PLMNs authorized for 5G ProSe services, including:

-	the list of the PLMNs where the UE is authorised for open 5G Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted 5G ProSe Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted 5G ProSe Direct Discovery Model B, i.e. to perform Discoverer operation or Discoveree operation or both.

-	the list of the PLMNs where the UE is authorised to perform Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication for NR PC5.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-2 UE-to-Network Relay.

-	the list of the PLMNs where the UE is authorised to act as a 5G ProSe Layer-3 UE-to-Network Relay.

The PC5 QoS parameters are provided by the UDR to PCF and the list of the PLMNs authorized for 5G ProSe services are provided by the UDR to both UDM and PCF. Other subscription information is provided by the UDR to UDM.

# * * * * End of changes * * * *