SA WG2 Meeting #S2-148E	S2-2108883

15 November – 19 November 2021, Electronic, Elbonia	(revision of S2-2107740)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0040 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to the Removal of MBS session configuration with PCC and related service operation. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is EN on the Npcf_MBSPolicyControl_Delete service operation, i.e. whether it is needed. In the 23.502 clause 4.16.6, for the PCF initiated SM Policy Termination (case A), the PCF send the Npcf_SMPolicyControl_UpdateNotify to SMF. And the SMF invoke the Npcf_SMPolicyControl_Delete service operation to request the deletion of the SM Policy Association with the PCF. This CR proposes to keep alignment with 416.6 for SM policy association termination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the EN in the clause 9.2.2.4. Modify the Removal fo MBS session configuration procedure, the MB-SMF sends the Npcf_MBSPolicyControl_Delete to request the deletion of the SM Policy Association. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN is not resolved. The SM Policy Association termination is different for unicast PDU session and MBS session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5, 9.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 7.1.1.5	Removal of MBS session configuration with PCC

This procedure is used by the AF to release the MBS Session. This procedure may also include TMGI de-allocation. The procedures apply to both multicast and broadcast communications unless otherwise stated. This procedure releases the reserved resources in both 5GC and NG-RAN.

Figure 7.1.1.5-1: Removal of configuration for MBS Session with PCC

1-3	Same as in Figure 7.1.1.4-1

4.	The NEF/MBSF sends an NMBSPolicyAuthorization_Delete Request to the PCF that handles the Policy of the MBS Session.

5.	The PCF sends an Npcf_MBSPolicyControl_UpdateNotify Request to MB-SMF to release the MBS Policy Control Association. The MB-SMF acknowledges the request.

6-9.	Same as steps 5-8 in Figure 7.1.1.4-1

10.	The MB-SMF sends the Npcf_MBSPolicyControl_Delete Request to request the deletion of the SM Policy Association with  the PCF.

11.	The PCF de-registers at the BSF that it handles the MBS session.

12.	The PCF sends an NpcfMBSPolicyAuthorization_Delete Response to the NEF/MBSF.

13-17.	Same as steps 11-15 in Figure 7.1.1.4-1.

Next CHANGE

#### 9.2.2.4	Npcf_MBSPolicyControl_Delete service operation

Service operation name: Npcf_MBSPolicyControl_Delete

Description: The NF Service Consumer (i.e. MB-SMF) can request the deletion of the MBS Policy Association and of the associated resources.

Inputs, Required: MBS Policy Association ID.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

END OF CHANGES