3GPP TSG-WG SA2 Meeting #148E e-meeting 	S2-2108890

November 15 – 19, 2021

Source:	Cisco Systems

Title:	Mandatory SSC Mode Support for UE

Document for:	Discussion

Agenda Item:	4.1

Work Item / Release:	5GS_Ph1_UEConTest

Abstract: Discusses this topic in light of the LS sent by CT6 (S2-2108269).

# 1. Introduction

SA2 received an LS from CT6 (S2-2108269) asking some questions regarding the mandatory or optional nature of SSC modes for the UE.

# 2. Discussion

Here are the relevant snippets of the LS, with the main discussion point highlighted in yellow:

== Snip start

.... It’s our understanding that supporting SSC mode 1 is mandatory, while the support of SSC modes 2 and 3 is optional. The support of SSC modes 2 and 3 is dependent on the type of device, and can be defined by the device manufacturer. However, for CT6 the existing definition in 3GPP TS 23.501 seems to be not very clear.

...

Although the mechanism to handle ‘unsupported’ SSC modes is defined in 3GPP TS 24.526, clause 4.2.2 (CT1 Rel-16, C1-196422), there is no clear indication  what SSC modes can be treated as optional for Rel 15 and Rel 16 devices.

==Snip end

The questions from CT6 should be taken from the perspective of requirements for UE to process signalling from network (NAS and URSP) that have SSC Mode information.

Currently, Stage 2 specifications, 23.501, 23.502 and 23.503 do not specify which SSC Modes are mandatory for the UE. Also, Stage 3 specification for NAS in 24.501 provides the following indication:

1. 	SSC Mode 1: UE shall support this. The main reason for this is that SSC Mode is MANDATORY sent for PDU Session Establishment accept. Emergency calls are necessarily setup as SSC Mode 1. What is not provided in the PDU Session establishment accept message is what happens if SSC Mode, eg SMF puts the SSC mode 3 in PDU Session Establishment accept message.

2.	SSC Mode 2: UE should support this. The terminology used for PDU Session Release with re-establish is that the UE should re-establish a new PDU session. The "should" is not that the UE does not process the cause #39, which can be used also for load-balancing between SMFs and not just for SSC Mode 2, but possibly because UE does not need to re-establish the PDU connection.

3.	SSC Mode 3: This is clearly stated as optional. The UE "can" provide the lifetime to the application.

The main question is for URSP rules that have SSC Mode associated with the Route Selection Descriptor (RSD). Neither Stage 2 nor Stage 3 TS 24.526 specifies whether the UE is MANDATED to:

A. Process RSD that has SSC Mode set to 1.

B. Process RSD that has SSC Mode set to 2.

C. Process RSD that has SSC Mode set to 3.

Clearly, a UE should process RSD that has SSC Mode set to 1. If UEs do not support this, then the very use of RSD becomes questionable.

Clearly, the processing of SSC Mode set to 3 is optional and based on whether the UE supports SSC Mode 3.

The main discussion is whether the UE shall process RSDs that have SSC Mode set to 2?

-	The use of SSC Mode 2, i.e. re-anchoring the IP anchor to a point closer to the current location of the UE, is the simplest MEC mechanism that an operator can deploy. The use of schemes such as UL-CL and IPv6 multi-homing are much more complicated and require intricate interactions with DNS etc along with complicated support for charging and LI. Hence, it is important for UEs to support SSC Mode 2.

While one can argue that an operator can always do a "PDU disconnect with reconnect" even for SSC Mode 1 PDU session, enabling the separation of applications into those that require IP@ continuity, i.e. SSC Mode 1, versus those that can benefit with re-anchoring (SSC Mode 2) provides an essential tool for operator to deploy SSC Mode 2. If UEs do not support SSC Mode 2, then essentially all applications will need to be re-anchored and the 5G System will fail to provide the simplest form of MEC for operators.

Proposal: UEs shall support SSC Mode 1 and 2.

Based on this CT1 should change the Stage 3 text in 24.526 from:

ii)	the selected route selection descriptor includes a PDU session type or an SSC mode which is not supported by the UE, the UE shall proceed to step 4);

to:

ii)	the selected route selection descriptor includes a PDU session type or an SSC mode set to 3 which is not supported by the UE, the UE shall proceed to step 4);

# 3. Conclusion and proposal(s)

1. Update TS 23.501 to state that UEs shall support SSC Mode 1 and SSC Mode 2.

- 	UE shall be able to request SSC Mode 1 and SSC Mode 2 in PDU Session Establishment request.

-	UE shall be able to accept SSC Mode 1 and SSC Mode 2 in PDU Session Establishment accept.

-	UE shall be able process SSC Mode values set to 1 and 2 in the RSD of URSP.

The support of SSC Mode 3 is optional for the UE.

- 	UE may request SSC Mode 3 in PDU Session Establishment request.

-	The SMF shall only provide SSC Mode 3 in PDU Session Establishment accept, if the UE requested SSC Mode 3 in the PDU Session Establishment Request.

-	UE may be able to process RSD in URSP with SSC Mode values set to 3, if the UE supports SSC Mode 3.

CR attached in S2-210xxxx.

# Annex: 24.501 excepts about SSC Mode processing on UE