SA WG2 Meeting #148e	S2-2108924

# November 15th – 22nd; Elbonia                         	    		 	 			(revision of S2-2107654r06)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2329 | rev | 5 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of the mapping from IP addressing information provided to an AF to the user identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Relates with incoming LS in S2-2106990 = S6-211082 An AF may only be able to identify the UE target of an AF request for Data Provisioning or for Event Exposure about a UE by providing UE addressing information (e.g. IP address and port information) to identify this UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An AF may request (via the NEF) information exposure or parameter provisioning targetting an individual UE, identifying the target UE by providing UE addressing information. In this case the 5GC (NEF) first needs to retrieve a Permanent Identifier of the UE Then the NEF Event exposure or the parameter provisioning may apply as defined in R16. Nnef_EventExposure_Subscribe, Nnef_Location_LocationUpdateNotify and Nnef_ParameterProvision operations are modified accordingly. The NEF may deliver back to the AF an UE external Identifier for an Application. This UE external Identifier for an Application may later on be used by the AF to issue further requests about the same UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An AF cannot identify the UE target of an AF request for Data Provisioning or for Event Exposure by providing UE addressing information to identify this UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.3.2.X (new) ; 4.15.6.X (new) ; 5.2.6.2.2 ; 5.2.6.4; 5.2.13.2.4 ; 4.15.Z (new); 5.2.6.1 ; 5.2.6.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 2385 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Aligned wording towards “AF specific UE Identifer” (aligning with the .501 CR) This CR version adds elements from 5502(from last SA2) to elements from S2-2105397 |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 4.15.3.2.X	Handling AF requests when the UE is identified via UE addressing information

An AF may via the NEF request Event Exposure (as defined otherwise in other sub-clauses of clause 4.15.3) or parameter provisioning (as defined in clause 4.15.6) targetting an individual UE, identifying the target UE by providing UE addressing information.

In this case the 5GC first needs to retrieve a Permanent Identifier of the UE based on:

- the UE addressing information as provided by the AF: this may correspond to an UE IP address as allocated by 5GC,

- the corresponding DNN and/or S-NSSAI: this may have been provided by the AF or determined by the NEF using the identity of the AF;

Once this is done, the 5GC may carry out the action requested by the AF and may deliver back to the AF an AF specific UE Identifer (as defined in TS 23.501 [2]). This AF specific UE Identifer may later on be used by the AF to issue further requests about the same UE.

The AF may have its own means to maintain the AF specific UE Identifer through, e.g., an AF session. After the retrieval of an AF specific UE Identifer the AF shall not keep maintaining a mapping between the returned AF specific UE Identifer and the UE IP address as this mapping may change.

NEXT CHANGE (2)

### 4.15.6	External Parameter Provisioning

#### 4.15.6.X	Parameter Provisioning when the UE is identified via UE addressing information

Handling of Parameter Provisioning requests targeting an individual UE when the UE is identified via UE addressing information is described in clause 4.15.3.2.X. Once the procedure described in clause 4.15.3.2.X has taken place, the Parameter Provisioning as described in other sub-clauses of 4.15.6 may apply.

NEXT CHANGE (3)

##### 5.2.6.2.2	Nnef_EventExposure_Subscribe operation

Service operation name: Nnef_EventExposure_Subscribe

Description: the consumer subscribes to receive an event, or if the event is already defined in NEF, then the subscription is updated.

Inputs, Required: (Set of) Event ID(s) as specified in clause 4.15.3.1 or Npcf_PolicyAuthorization_Notify and Naf_EventExposure_Subscribe service operation, Target of Event Reporting (GPSI that may contain an AF specific UE Identifier as defined in clause 4.15.3.2.X, SUPI, External Group Identifier, Internal Group Identifier, UE addressing information (IP address)) Event Reporting Information defined in Table 4.15.1-1, Notification Target Address (+ Notification Correlation ID), MTC Provider Information.

Inputs, Optional: Event Filter, (set of) External Application Identifier(s), Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, list of group member UE(s) whose subscription to event notification(s) are removed from a group-based event notification subscription, DNN, S-NSSAI, Idle Status Indication request (if UE reachability or Availability after DDN failure reporting is requested).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if available (see clause 4.15.1), AF specific UE Identifer,see clause 4.15.3.2.X).

.

NEXT CHANGE (4)

#### 5.2.6.4	Nnef_ParameterProvision service

##### 5.2.6.4.1	General

This service is for allowing external party to provision of information which can be used for the UE in 5GS.

##### 5.2.6.4.2	Nnef_ParameterProvision_Update service operation

Service operation name: Nnef_ParameterProvision_Update

Description: The consumer updates the UE related information (e.g., Expected UE Behaviour, Network Configuration parameters, Location Privacy Indication parameters, ECS Address Configuration Information) or 5G VN Group related information (e.g. 5G VN group data, 5G VN membership management).

Inputs, Required: AF Identifier, Transaction Reference ID.

Inputs, Optional:

-	UE ID (GPSI that may contain an AF specific UE Identifier as defined in clause 4.15.3.2.X, UE addressing information), External Group ID

-	at least one of the Expected UE Behaviour parameters or at least one of the Network Configuration parameters or 5G VN related information or ECS Address Configuration Information, Validity Time or Location Privacy Indication parameters, MTC Provider Information.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available, AF specific UE Identifer see clause 4.15.3.2.X) in case of success.

.

##### 5.2.6.4.3	Nnef_ParameterProvision_Create service operation

Service operation name: Nnef_ParameterProvision_Create

Description: The consumer creates a 5G VN group.

Inputs, Required: AF Identifier, Transaction Reference ID.

Inputs, Optional:

-	UE ID (GPSI that may contain an AF specific UE Identifer as defined in clause 4.15.3.2.X, UE addressing information), External Group ID for 5G VN group creation,

-	5G VN group related information (e.g., 5G VN group data, 5G VN membership management), MTC Provider Information.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available, AF specific UE Identifer (see clause 4.15.3.2.X) in case of success.

##### 5.2.6.4.4	Nnef_ParameterProvision_Delete service operation

Service operation name: Nnef_ParameterProvision_Delete

Description: The consumer deletes a 5G VN group.

Inputs, Required: AF Identifier, Transaction Reference ID.

Inputs, Optional: External Group ID.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

##### 5.2.6.4.5	Nnef_ParameterProvision_Get service operation

Service operation name: Nnef_ParameterProvision_Get

Description: The consumer gets the UE related information (e.g. Expected UE Behaviour, Network Configuration parameters, ECS Address Configuration Information).

Inputs, Required: UE ID (GPSI that may contain an AF specific UE Identifer as defined in clause 4.15.3.2.X, UE addressing information), AF Identifier, requested information (e.g., Expected UE Behaviour, Network Configuration parameters, ECS Address Configuration Information).

Inputs, Optional: None.

Outputs, Required: Requested data, Operation execution result indication.

Outputs, Optional: AF specific UE Identifer (see clause 4.15.3.2.X) in case of success..

NEXT CHANGE

##### 5.2.13.2.4	Nbsf_Management_Discovery service operation

Service Operation name: Nbsf_Management discovery

Description: Discovers the PCF and PCF set selected for a PDU Session identified by the tuple (UE address(es), SUPI, GPSI, DNN, S-NSSAI), or discovers the PCF and PCF set selected for the UE identified by the tuple (SUPI, GPSI). This operation may also be used to determine the SUPI/GPSI from the tuple (UE address, DNN, S-NSSAI).

Inputs, Required: UE address (i.e. IP address or MAC address), [Required, for a PDU Session and for a UE], DNN [Conditional], S-NSSAI [Conditional], if the target PCF is for a PDU Session, MBS session ID as defined in TS 23.247 [78], [Required, for an MBS Session].

SUPI and/or GPSI, if the target PCF is for a UE.

NOTE:	For TSN scenarios the UE address contains the DS-TT port MAC address.

Inputs, Optional: If the target PCF is for a PDU Session, SUPI, GPSI.

Outputs, Required: PCF address(es), PCF instance ID [Conditional, if available] and PCF Set ID [Conditional, if available], level of Binding [Conditional, if available] (see clause 6.3.1.0 of TS 23.501 [2]).

Outputs, Optional: None.

NEXT CHANGE

### 4.15.z	explicit AF specific UE Identifer retrieval

This clause contains the detailed description and the procedures for the explicit AF specific UE Identifer retrieval when explicitly requested by the AF. The AF specific UE Identifer is defined in clause 4.15.3.2.X..

NOTE 1:	As defined in clause 4.15.3.2.X, the AF may implicitly request an AF specific UE Identifer by issuing a NEF request about an UE identified by UE addressing information (UE IP address)

NOTE 2:	After retrieving AF specific UE Identifer, the AF can invoke other NEF provided services (e.g. location monitoring) using this identifier.

Figure 4.15.z-1: AF specific UE Identifer retrieval

1.	AF requests to retrieve AF specific UE Identifer via the Nnef_UEId_Get service operation. The request message shall include UE address (IP address or MAC address) and AF Identifier, it may include AF Service Identifier, MTC Provider Information, DNN, S-NSSAI, Application port ID, IP domain. The MTC Provider Information identifies the MTC Service Provider and/or MTC Application.

NOTE 1:	The MTC Provider Information can be used by any type of Service Providers (MTC or non-MTC) or Corporate or External Parties for, e.g. to distinguish their different customers.

NOTE 2: 	The case where UE IP address provided by the AF to the NEF corresponds to an IP address that has been NATed (Network and Port Address Translation) is not supported in this release.

NOTE 3:	The NEF can validate the provided MTC Provider Information and override it to a NEF selected MTC Provider Information based on configuration. How the NEF determines the MTC Provider Information, if not present, is left to implementation (e.g., based on the requesting AF).

NOTE 4: 	The Application Port ID is as defined in Nnef_Trigger_Delivery.

2.	The NEF authorizes the AF request. If the authorisation is not granted, the NEF replies to the AF with a Result value indicating authorisation failure; otherwise the NEF proceeds with the following steps. The NEF determines corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or is determined by the NEF based on the requesting AF Service Identifier, MTC Provider Information.

3-4.	The NEF may use the Nbsf_Management_Discovery service operation with UE address and (DNN, S-NSSAI)  to retrieve the session binding information of the UE. If no SUPI is received in the session binding information from the BSF, the NEF replies to the AF with a Result value indicating that the UE ID is not available.

5.	The NEF interacts with UDM to retrieve the AF specific UE Identifer via the Nudm_SDM_Get service operation. The request message includes SUPI and at least one of Application PortMTC Provider Information or AF Service Identifier.

6.	The UDM responds to the NEF with AF specific UE Identifer.

7.	The NEF further responds to the AF with the AF specific UE Identiferinformation received from the UDM.

NEXT CHANGE

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_SliceStatus | Retrieve | Request/Response | AF |
| Nnef TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF |

NEXT CHANGE

#### 5.2.6.z	Nnef_UEId service

##### 5.2.6.z.1	General

See clause 4.15.z.

##### 5.2.6.z.2	Nnef_UEId_Get operation

Service operation name: Nnef_UEId_Get

Description: Get an AF specific UE Identifer.

Inputs, Required: UE address (i.e. IPv4/IPv6 address or MAC address), AF Identifier.

Inputs, Optional: AF Service Identifier, Application Port ID, MTC Provider Information

Outputs, Required: Result, AF specific UE Identifer if successful.

Outputs, Optional: None.

NEXT CHANGE

END OF CHANGES