3GPP TSG-SA WG2 Meeting #146e	S2-2108987

# Online, 15th - 22nd November, 2021

Source:	NEC,

Title:	DISC On AMF relocation in connected mode

Document for:	Discussion and Decision

Agenda Item:	9.1.3

# 1	Introduction

The aim of the discussion paper is to discuss the scenario related to AMF relocation in the connected mode.

# 2	Discussion

Currently AMF relocation is defined as below when the UE is in idle mode.

##### 4.2.2.2.3	Registration with AMF re-allocation

Following scenario is not handled in the specification.

1) A UE is registered to a slice S-NSSAI 1 at the initial AMF. PDU Session and DRB is established related to the S-NSSAI.

2) A UE is initiates registration to the S-NSSAI 1 and S-NSSAI 2.

3) The initial AMF can’t handle S-NSSAI 1 and S-NSSAI 2 together.

4) The initial AMF queries NSSF regarding the AMF handling S-NSSAI 1 and S-NSSAI 2.

5) The NSSF returns an address of a new AMF handling S-NSSAI 1 and S-NSSAI 2.

6) Currently the initial AMF can’t handover the UE context from the initial AMF to the new AMF.

Problem statement: AMF relocation in connected mode is not defined This is a FASMO issue as the UE can always step 2 in connected mode.

# Proposal

It is proposed to study the scenario and find a solution mentioned in the discussion paper. The key objective 12 is focusing on this scenario.